3GPP TSG-SA2 Meeting #165	S2-2409693

# Hyderabad, India, October 14 – 18, 2024          		       	       			(revision of S2-2408616)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0572 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | One Transmission Path used for One Positioning Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | User plane positioning is introduced in Rel-18 5G_eLCS_Ph3 to transfer the LPP messages and supplementary service messages via user plane. For the messages (LPP messages, supplementary services messages) belonging to one positioning procedure, there is no common understanding on how to transfer the messages via user plane and control plane. Some companies believe that for the messages belonging to one LCS session, either control plane or user plane should be used by UE and LMF at the same time. While some companies think it is flexible for LMF to choose control plane or user plane to transfer the messages above, e.g. some messages above are transferred via user plane and the others could be transferred via control plane. If some messages belonging to one LCS session could be transferred via user plane and others could be transferred via control plane, the LMF and UE need to be enhanced to select user plane or control plane before sending a message. So this makes LMF and UE more complicated and increases positioning delay. To reduce complicity and positioning delay, it is proposed that one transmission path (i.e. control plane or user plane) is used for one positioning procedure. The UE and the LMF is enhanced to support the principle. After the transmission path is selected by LMF, during the positioning procedure, if the selected transmission path fails, LMF determines to select another path to re-trigger the positioning procedure or to re-trigger the positiong procedure when the selected transmission path is recovered. For example, when user plane positioning is selected for a positioning procedure, during the positioning procedure, if the user plane positioning dedicated PDU Session is released. Due to different release cause, the LMF behaviour is different: - If the PDU Session is released due to handover or S-NSSAI based congestion control, the LMF selects control plane to re-trigger the positioning procedure. - If the PDU Session is released due to establish a new PDU Session with an identical PDU Session ID, the LMF re-triggers the user plane connection establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For a positioning procedure, one transmission path is used at a time. - During the positioning procedure, if the selected transmission path fails, LMF determines to select another path to re-trigger the positioning procedure or to re-trigger the positiong procedure when the selected transmission path is recovered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent behavior between LMF and UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.x	One Transmission Path used for one Positioning Procedure

The LMF is responsible to select control plane positioning or user plane positioning for a positioning procedure to transfer the supplementary services messages and LPP messages included in the procedure.

When the LMF selects control plane for a positioning procedure, the LMF sends the supplementary services messages and LPP messages related to the procedure over control plane to UE. If the UE receives the DL NAS TRANSPORT message including an LCS Corrleation ID, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the control plane. During the positioning procedure, if the control plane transmission fails due to control plane congestion. The LMF determines to select user plane to re-trigger the positioning procedure if the user plane connection exists or re-trigger the positioning procedure over the control plane when control plane congestion ceases.

When the user plane is selected by LMF for a positioning procedure, the LMF sends the supplementary services messages and LPP messages related to the procedure over user plane to UE. If the UE receives the user plane transport message including an LCS Corrleation ID from LMF, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the user plane. During the postioning procedure, if the user plane positioning dedicated PDU Session is released based on the triggers described in clause 4.3.4.2 in TS 23.502[19], the AMF notifies the PDU Session status (i.e. release) and the cause to LMF when the AMF receives the Nsmf_PDUSession_SMContextStatusNotify service operation including the PDU Session status(i.e. release) and the cause from SMF. Based on the notification, the LMF determines to select control plane to re-trigger the positioning procedure or to re-trigger the user plane connection establishment.

NOTE X:	As examples, if the PDU Session is released due to handover or S-NSSAI based congestion control as specified in clause 6.1.6.3.8 in TS 29.502[X], the LMF selects control plane to re-trigger the positioning procedure. If the PDU Session is released due to establish a new PDU Session with an identical PDU Session ID as specified in clause 6.1.6.3.8 in TS 29.502[X], the LMF re-triggers the user plane connection establishment.

NOTE Y:	The LMF is implicitly subscribed to be notified of PDU Session status by including notification endpoint for PDU Session status notification in the Nlmf_Location_UPNotify service operation in clause 6.18.1 or in the Nlmf_Location_UPConfig Response service operation in clause 6.18.2.

* * * End of Change * * *