3GPP SA WG2 Meeting #165	S2-2409717

# Hyderabad, India, 14 – 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1206 | rev | x | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the procedure of VFL Inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the previous meeting, a procedure for VFL inference was introduced. However, there are many ENs that still need to be addressed, and this CR is proposed to resolve some of them. The procedures for NWDAF and AF acting as VFL servers differ in two key aspects: receiving inference information requests from the AnLF and sending VFL inference requests to VFL clients. Thus, the 1st EN should be removed, and a new procedure for VFL inference when the AF is acting as the VFL server should be added. Additionally, the steps involving untrusted AF and NEF are unclear, so new services are introduced to clarify these processes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove some ENs; - Introduce a new procedure for VFL inference when the AF is acting as the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature of VFL Inference when the AF acts as VFL Server is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G, 6.2G.1, 6.2G.2, 6.2G.2.1, 6.2G.2.X (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2G	Vertical Federated Learning

6.2G.1	General

This clause specifies how NWDAF(s) and AF(s) can leverage Vertical Federated Learning technique to perform VFL inference.

6.2G.2	Procedures

6.2G.2.1	Inference procedure for vertical federated learning when NWDAF as VFL Server

Figure 6.2G.2.1-1: Inference procedure for vertical federated learning when NWDAF as VFL Server

0.	For an NWDAF as a VFL server, the analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF cannot generate the analytics output, the NWDAF containing AnLF sends a subscription request to VFL server NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs.

2.	Based on the information received in the step 0 or 1, NWDAF as VFL server decides to initiate the VFL inference procedure with the NWDAF(s)/AF(s) as VFL client(s). A VFL model ID is selected corresponding to the requested Analytics ID, and the VFL clients should be determined based on the selected VFL model ID.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

If the VFL server is the NWDAF and the VFL client is the NWDAF, VFL server sends a VFL Inference request/subscription to the VFL clients by invoking Nnwdaf_VFLInference_Request or Nnwdaf_VFLInference_Subscription which includes an Analytics ID, target of Analytics Reporting and the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

If the VFL server is the NWDAF and the VFL client is the trusted AF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients by invoking Naf_VFLInference_Request or Naf_VFLInference_Subscription which includes an Analytics ID, target of Analytics Reporting and the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

If the AF is untrusted, the VFL server NWDAF sends the Nnef_VFLInference_Request or Nnef_VFLInference_Subscription to the NEF with an Analytics ID, AF ID, target of Analytics Reporting and the VFL model ID, and NEF forwards the request to the untrusted AF.

Besides, the NEF translates the target of Analytics Reporting, e.g. SUPIs and Internal-Group-Id into GPSIs or External-Group-Id.

Editor's note:	Whether the VFL model correlation ID is needed or using the model ID is FFS.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether the VFL client needs to retrieval of stored local models is FFS.

5.	Each VFL Client sends the intermediate local inference results to the VFL server.

Editor's note:	Whether the VFL model correlation is required in step 5 is FFS.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to aggregates and generates the VFL inference results.

6.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL model correlation ID.

Editor's note:	Further details of the VFL server computing the VFL inference is FFS.

7.	If the NWDAF is the VFL server, the VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e. NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

* * * Next Change (ALL new text) * * *

#### 6.2G.2.X	Inference procedure for vertical federated learning when AF as VFL Server

Figure 6.2G.2.1-1: Inference procedure for vertical federated learning when AF as VFL Server

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting=e.g. UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe. If the NWDAF containing AnLF cannot generate the analytics output, but an AF as VFL server can do it based on VFL process, the following steps may be performed.

1a. If the VFL Server is a trusted AF, the NWDAF containing AnLF sends a subscription or request to VFL server AF using Naf_VFLAnalyticsInfo_Request or Naf_VFLAnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs.

1b.	If the VFL Server is an untrusted AF, the NWDAF containing AnLF sends a subscription or request to VFL server AF via NEF. The NWDAF containing AnLF sends a subscription or request to NEF by invoking Nnef_VFLAnalyticsInfo_Request or Nnef_VFLAnalyticsSubscription_Subscribe including Analytics ID, VFL server AF ID, Target of Analytics Reporting = e.g. UE IDs.The NEF translates the target of Analytics Reporting, e.g. UE IDs into external UE IDs, and maps the internal Analytics ID to external Analytics ID.

1c. The NEF sends a subscription or request to the VFL Server AF.

2.	Based on the information received in the step 0 or 1, AF as VFL server decides to initiate the VFL inference procedure with the NWDAF(s) as VFL client. A VFL model ID is selected corresponding to the requested Analytics ID, and the VFL clients should be determined based on the selected VFL model ID.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

If the VFL server is the trusted AF, VFL server sends a VFL Inference request/subscription to the NWDAF(s) as VFL client by invoking Nnwdaf_VFLInference_Request or Nnwdaf_VFLInference_Subscription which includes an Analytics ID, target of Analytics Reporting and the VFL model ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

If the VFL server is the untrusted AF, VFL server sends a VFL Inference request/subscription to the VFL clients via NEF including external Analytics ID, target of Analytics Reporting and temporary NWDAF(s) ID. The NEF translates the target of Analytics Reporting, e.g. external UE IDs into internal UE IDs, and maps the external Analytics ID to internal Analytics ID. The NEF sends the VFL Inference request/subscription to the VFL clients by invoking Nnwdaf_VFLInference_Request or Nnwdaf_VFLInference_Subscription which includes an internal Analytics ID, target of Analytics Reporting (i.e. internal UE IDs) and the VFL model ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

Editor's note:	Whether the VFL model correlation ID is needed or using the model ID is FFS.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL model ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether the VFL client needs to retrieval of stored local models is FFS.

5.	Each VFL Client sends the intermediate local inference results to the VFL server. If the VFL server is an untrusted AF, this step should be via NEF.

Editor's note:	Whether the VFL model correlation is required in step 5 is FFS.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to aggregates and generates the VFL inference results.

6.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL model ID.

Editor's note:	Further details of the VFL server computing the VFL inference is FFS.

7.	The AF as the VFL server sends response to the consumer (i.e. NWDAF containing AnLF) (via NEF) including the VFL inference results by invoking Naf_VFLAnalyticsInfo_Response or Naf_VFLAnalyticsSubscription_Notify. If the VFL server is an untrusted AF, this step should be via NEF.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

* * * End of Changes * * *