3GPP TSG-SA2 Meeting #165	S2-2409732

# 14 - 18 October 2024, Hyderabad, India

Source:	OPPO

Title:	Discussion on UE side data collection

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This contribution discusses the potential SA2 impact based on the RAN LS s2-2409600 and provide some proposals.

1. Background

In the RAN LS S2-2409600, the following agreements on UE side data collection are captured as bellow:

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

RAN#105 way forward and requirements are captured in the same LS as follows:

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

The standardized solutions for data collection (if standardized) need to meet the requirements above. TSG RAN kindly asks TSG SA to initiate the work and at least for SA2 to provide inputs on solutions 1b, 2, and 3 by RAN#106.

In this paper, we give our view and analysis of the options and their associated requirements captured in the RAN LS.

2. Discussion

Option 1a has no 3GPP impact. For option 1b, 2 and 3, it was clearly mentioned in the RAN LS that RAN2 could not reach consensus without input from SA groups on feasibility of MNO full controllability and full visibility (Note 5) for option 1b and couldn’t reach consensus on the feasibility of UP solution for option 2 and option 3 (Note 7).

Proposal 1: SA2 only need to focus on study and analyse the feasibility of MNO full controllability and full visibility and the feasibility of UP solution for option 2 and option 3.

Option 1b)

When preforming the UE data collection, the first termination entity is the server for data collection for UE-side model training. One of the potential solutions is to use DCAF as the server for data collection. However, currently, the DCAF can only collect the UE application data. Some enhancements are needed if using the DCAF framework to collect RAN measurement data. Also, currently, it is the provisioning AF, which belongs to the ASP, that provisions the DCAF specific data it can collect from UE applications. Since ASP cannot understand the RAN measurement data, if using the DCAF to collect the UE data, MNO needs to provision the DCAF what measurement configuration is needed for data collection and what types of data can be collected.

Regarding the feasibility of MNO full controllability and full visibility for option 1b, since the DCAF can be trust AF which is controlled by the network. Full controllability can be supported. However, for full visibility, the communication connection between the UE and the DCAF is based on the application layer connection. The data collected from UE will first go to DCAF. If the MNO wants to get the UE data, using the NWDAF as an example, there is an R5 interface between the DCAF and NWDAF. If the NWDAF wants to get the data from the DCAF, the current mechanism is that NWDAF will request the standardized data from the DCAF by invoking the DCAF service operation. In other word, only when the NWDAF needs standardized data, it will request the DCAF. However, for non-standardized data or the data the NWDAF does not need, the NWDAF could not get the data and even cannot understand the non-standardized data. Therefore, full visibility cannot be supported.

Observation 1: For option 1b, full controllability can be supported If the DCAF is a trusted AF. Full visibility cannot be supported if DCAF also collects non-standardized data or not all of the standardized data are needed by the MNO.

Observation 2: Using DCAF is one potential solution to support the option 1b but needs significant DCAF framework enhancement and also need MNO’s help to provision the DCAF on what measurement configuration is needed for data collection and what types of data can be collected.

Option 2

For option 2, the first termination entity is inside the CN. From SA2 perspective, regardless of CP or UP solution, the first impact is to determine which NF should be such first termination entity. It could be LMF, NWDAF or a new NF. If using LMF, the pros is that LMF has the existing LPP protocol which can be used to carry the UE data. But the LPP is designed only for positioning case which is not suitable to support the common UE data collection. If using NWDAF or new NF, the new protocol needs to be designed to support the UE data collection which will have big standard impact.

Considering the UE data size canbe quite large, it will be not suitable to use CP solution. We already have the LPP over UP solution and the similar method can be used for the NWDAF or new NF with the new protocol over UP. The NWDAF or new NF IP address can be sent to the UE via NAS signalling and UE will use specific PDU session to transmit the UE data to the NWDAF or new NF.

Observation 3: for option 2, the first termination entity needs to be determined and the new protocol used to carry the UE data need to be specified. Take the LPP over UP into consideration, the UP-based solution to support UE data collection is feasible.

Since the first termination entity is inside the CN, all the UE data will be sent to the CN NF first. The UE and the CN NF has the same level protocol to carry the data. CN can understand all the data received from UE and can determine which data can be exposed to the 3rd party. The MNO full controllability and full visibility can be supported.

Observation 4: For option 2, the MNO full controllability and full visibility can be supported.

Option 3

Option 3 is mainly in SA5 scope. Since OAM is controlled by the MNO, MNO can understand all the data received by OAM. The MNO full controllability and full visibility can be supported.

Observation 5: For option3, the MNO full controllability and full visibility can be supported.

Proposal 2: For option 2/3, full visibility is guaranteed so no need to have option B/C. For option 1b) full visibility can be supported for standardized data, but cannot be supported for non-standardized data and also a mix of standardized and non-standardized data. The table content can be updated as below:

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) Full controllability | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | Opt A) Partial visibility for partially standardized data contents. Opt B) No standardized visibility. | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

3. Summary of Proposals

Proposal 1: SA2 only need to focus on study and analysis the feasibility of MNO full controllability and full visibility and the feasibility of UP solution for option 2 and option 3.

Proposal 2: For option 2/3, full visibility is guaranteed so no need to have option B/C. For option 1b) full visibility can be supported for standardized data, but cannot be supported for non-standardized data and also a mix of standardized and non-standardized data. The table content can be updated as below:

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) Full controllability | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | Opt A) Partial visibility for partially standardized data contents. Opt B) No standardized visibility. | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |