3GPP TSG-SA2 Meeting #165	S2-2409734

# 14 – 18 October, 2024, Hyderabad, IN

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1209 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the general description for vertical federated learning to resolve ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | One Editor's note: Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS. is captured in the general description for the VFL. However, sample and feature alignment is needed in VFL to make sure the training dataset in the different entity has the same sample space and different feature space. Sample and feature information are tightly related to the available data set in each VFL entity. It wiil be a very dynamic information. Therefore, the sample and feature information need to be negotiated between the VFL server and VFL client. This CR add the sample and feature alignment functionality to resolve the EN. The VFL server is based on the intermediate training results reveived from VFL client(s) to compute intermediate training information. The current wording use the intermediate training results for the both case which is not clear. For the AF as the VFL server, the AF discovers the NWDAF as VFL clients is also added. The detail on how to perform the VFL client discovery can refer to the VFL discovery clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the sample and feature alignemnt functionality to resolve the EN. Change the intermediate training results to the intermediate training information which is used to update the VFL server local ML Model and the ML Models of VFL clients. Add the VFL discovery related description when the AF is the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature and functionality supported for the VFL is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning techinique without exchanging/sharing local data set, while maintaining some level of coordination amongst VFL participants, when training and inference are performed on local ML Models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AF.

For Vertical Federated Learning, there may be one NWDAF or one AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). When the AF acting as a VFL server, the VFL client(s) can only be the NWDAF(s). Vertical Federated Learning is available among NWDAFs within a single PLMN or between an AF and NWDAF(s) in a single PLMN.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure.

-	An AF acting as VFL server discovers and selects VFL client(s) (i.e. NWDAF(s)) to participate in a VFL procedure.

Editor's note:	Support for AF acting as VFL server selecting multiple NWDAFs is FFS.

-	VFL server sends a list of target samples (e.g. UE IDs) and optionally the required features in the feature profile to the VFL clients.

-	requests VFL clients to report the supported samples and optionally the supported features.

-	performs the sample and optionally feature alignment based on the supported samples and optionally the supported features of VFL clients.

-	requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL model correlation ID, and it requests to report intermediate training results.

-	aggregates intermediate training results from VFL client(s) and computes intermediate training information (e.g. gradient information, loss information) for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the intermediate training information towards VFL clients involved in the joint VFL training process.

-	It initiates the VFL inference process using VFL model correlation ID.

-	It aggregates local inference result from VFL clients and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer.

VFL client:

-	reports the supported samples and optionally the supported features.

-	locally trains ML Model with the available local data set, which includes the data that may not be allowed to be shared with other VFL clients due to e.g. data privacy, data security, data access rights.

-	computes the intermediate traning results for their local ML Models involved in the VFL training and provide reports with the intermediate traning results to the AF or NWDAF acting as VFL server.

-	performs local inference based on the local model and local data and provides local inference results to VFL server.

Editor's note:	Accuracy monitoring in VFL is FFS.

Editor's note:	For an NWDAF impacts of the split into AnLF and MTLF are FFS.

# * * * * End of changes * * * *