3GPP TSG-SA2 Meeting #165	S2-2409808

# Hyderabad, India, 14 - 18 October, 2024								(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0578 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of event filter for the event GNSS Assistance data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The event filter for AF exposure is used to specify the conditions to match for notifying the event. However, in the procedure of LMF collecting GNSS assistance data from AF, it is unclear whether to include the event filter to the NEF/AF. Without an event filter, the NEF may select AF without any criterion and AF may provide a huge amount of GNSS assistance data covering a huge area causing resource waste. With the event filter, the NEF can select an AF cover the target area and the AF can provide more adaptive GSS assistance data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the LMF optionally provides the event filter to the NEF and/or AF to help AF provide more adaptive GSS assistance data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF may provide a huge amount of GNSS assistance data covering a huge area causing useless data transfer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15.1, 6.15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

6.15.1	GNSS assistance data collection from untrusted AF via NEF

The following procedure is used by LMF to collect GNSS assistance information from AFs via the NEF.

Figure 6.15.1-1: GNSS assistance data collection from untrusted AF

1a-1c.	Reuse the steps 1a-1c in figure 6.2.2.3-1 of TS 23.288 [37] with following difference:

-	The AF should also provide its serving area to NEF. NEF should map the serving area into TA(s) if it is presented as geographical location and update the NEF profile in NRF including the serving area information.

1d-1e.	Reuse the steps 1d-1e in figure 6.2.2.3-1 of TS 23.288 [37] with following difference:

-	When LMF needs to discovery the available data from AFs and the appropriated NEF to collect GNSS assistance data, LMF additionally provides TAs in the Nnrf_NFDiscovery_Request_request service operation.

2-5.	Reuse the steps 2-5 in figure 6.2.2.3-1 of TS 23.288 [37] with following difference:

-	The LMF may provide the event filter (e.g., target area (TAs)) to the NEF. If the event filter is received, the NEF also includes the event filter in the Naf_EventExposure_Subscribe/Naf_EventExposure_Unsubscribe service operation to the AF.

-	Besides available GNSS assistance data, AF also provides associated serving area of the GNSS assistance data and optional precise global coordinates of the source of the GNSS assistance data, which are determined considering the event filter received in step 3. NEF may map the serving area into a TA or TA list.

6.	After LMF gets the GNSS assistance data, it can send the assistance data to UE via the procedures described in clauses 6.14 or 6.11.1 during a positioning session.

* * * Next Change * * *

6.15.2	GNSS assistance data collection from trusted AF

For trusted AFs, LMF collects GNSS assistance data through AFs directly. LMF discovers trusted AFs covering LMF serving area through NRF and subscribes to GNSS assistance data exposure from discovered trusted AFs.

Figure 6.15.2-1: GNSS assistance data collection from trusted AF

1a-1c.	AF invokes Nnrf_NFManagement_NFUpdate_request service operation to register its GNSS related information including Event IDs, and associated AF identification, Application ID(s) and its serving area (TAs). NRF stores the received AF update information and sends Nnrf_NFManagement_NFUpdate_response message to AF.

1d-1e.	When LMF needs to discovery the available GNSS assistance data from AFs, LMF provide TAs to NRF through Nnrf_NFDiscovery service to discover related AFs.

2.	LMF subscribes to or cancels subscription to GNSS assistance data in AF by invoking the Naf_EventExposure_Subscribe/Naf_EventExposure_Unsubscribe service operation. The service operation may include the event filter (e.g., target area (TAs)).

3.	AF notifies the LMF with the available GNSS assistance data by invoking Naf_EventExposure_Notify service operation. AF also provides associated serving area (TAs) of the GNSS assistance data and optional precise global coordinates of the source of the GNSS assistance data. The AF notifies the LMF with the data by invoking Nnef_EventExposure_Notify service operation.

4.	After LMF gets the GNSS assistance data, it can send the data to UE via the procedures described in clauses 6.14 or 6.11.1 during a positioning session.

* * * End of Change * * *