3GPP TSG-SA2 Meeting #165	S2-2409855

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1218 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Consumers of Nnwdaf, Ndccf, Nmfaf related services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As pointed out by CT3 in incoming LS S2-2409586, some Service Consumers of Nnwdaf, Ndccf, Nmfaf related services are not listed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing Service Consumers of Nnwdaf, Ndccf, Nmfaf related services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is inconsistent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2b.3, 7.1, 8.1, 9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 6.2B.3	Historical Data and Analytics Storage via Notifications

The procedure depicted in figure 6.2B.3-1 is used by consumers (NWDAF, DCCF) to store received notifications in the ADRF. The consumer requests the ADRF to initiate a subscription for data and/or analytics. Data and/or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF.

The consumer may include in the subscription request the DataSetTag attribute defined in Table 6.2B-1.

Figure 6.2B.3-1: Historical Data and Analytics Storage via Notifications

0a-c	NWDAF, DCCF or ADRF are configured with default operator storage policies as described in clause 5B.1.

1a-d.	Based on provisioning or based on reception of a DataManagement subscription request (e.g. see clause 6.2.6.3.2), the DCCF or the NWDAF determines that notifications are to be stored in an ADRF. The subscription request may contain Storage Handling information with a requested ADRF storage lifetime and a request to be notified before data are deleted from the ADRF.

2a-b.	The DCCF or the NWDAF determines the ADRF where data and/or analytics needs to be stored and requests that the ADRF subscribes to receive notifications. The determination may be made based on configuration or information supplied by the data consumer as described in clauses 6.1.4 and 6.2.6.3. The request to the ADRF specifies the data and/or analytics to which the ADRF will subscribe by invoking the Nadrf_DataManagement_StorageSubscriptionRequest service operation. The request may include the DataSetTag attribute which the data records are to be associated to when stored by ADRF. If the Storage Policy is not configured on the NWDAF or DCCF, the NWDAF or DCCF sends the Storage Handling Request to the ADRF. The NWDAF or DCCF may provide notification endpoint information to the ADRF for use by the ADRF to send notifications (implicit subscription) alerting the DCCF or NWDAF that data are about to be deleted (see step 12).

3.	[Optional] The ADRF may, based on implementation, determine whether the same data and/or analytics is already stored or being stored, based on the information sent in step 2 by the consumer. If the DataSetTag attribute is included for data and/or analytics already stored or being stored, then ADRF associates the data records with such DataSetTag.

3a-c.	[Optional] Based on Storage Handling information and Storage Policy, the ADRF, DCCF or NWDAF determines the Storage Approach (lifetime for storing data and whether consumer is notified prior to data deletion).

4.	[Optional] If the data and/or analytics is already stored and/or being stored in the ADRF, the ADRF sends Nadrf_DataManagement_StorageSubscriptionRequest Response message to the consumer indicating that data and/or analytics is stored. The ADRF includes the Storage Approach if determined in step 3c.

5a-b.	The DCCF or NWDAF sends a subscription response to the Data or Analytics Consumer. The response may contain the Storage Approach determined by the ADRF, NWDAF or DCCF.

6a-b.	ADRF subscribes to the DCCF or the NWDAF to receive notifications, providing its notification endpoint address and a notification correlation ID. In step 6b, the ADRF uses Nnwdaf_DataManagement_Subscribe (as depicted in Figure 6.2B.3-1) to obtain data from the NWDAF or Nnwdaf_AnalyticsSubscription_Subscribe to obtain analytics from the NWDAF (not depicted).

7.	The DCCF, the MFAF or the NWDAF sends Analytics or Data notifications containing the notification correlation ID provided by the ADRF to ADRF notification endpoint address. The Analytics or Data notifications shall contain timestamp. The ADRF stores the notifications.

Conditional on DCCF or NWDAF determining to stop storing data or analytics

8a-b.	The DCCF or the NWDAF determines to stop storing notifications in the ADRF.

9a-b.	The DCCF or the NWDAF requests that the ADRF unsubscribes to receive notifications.

10a-b.	The ADRF sends a request to the DCCF or the NWDAF to unsubscribe to data notifications.

The NWDAF may interact with the Data Source and the DCCF may interact with the Data Source and/or MFAF. Delivery of notifications from the DCCF/MFAF or NWDAF to the ADRF are subsequently halted. If Nnwdaf_AnalyticsSubscription_Subscribe was used in step 6b, then the Nnwdaf_AnalyticsSubscription_Unsubscribe is used in step 10b.

Conditional on ADRF Managing the Storage Approach

11.	The ADRF determines that the lifetime of the stored data or analytics has expired (according to the Storage Approach).

12.	If indicated by the Storage Approach, the ADRF sends a notification alerting the DCCF or NWDAF that data are about to be deleted.

NOTE:	This is an implicit subscription.

Conditional if the Storage Appoach is based on Default Operator Policies provisioned on the NWDAF or DCCF (see step 3a-c and clause 5B.1)

13.	The DCCF or NWDAF indicate in the response to the ADRF whether they will retrieve the Data or Analytics

14.	The DCCF or NWDAF may retrieve the Data or Analytics from the ADRF

Conditional if the Storage Appoach is based on a Storage Handling Request received from the Data or Analytics Consumer (see step 3a-c and clause 5B.1)

15a-b. The NWDAF or DCCF sends a notification to the Data or Analytics Consumer alerting it that the data or analytics are about to be deleted.

16a-b. The Data or Analytics Consumer indicates in the response to the NWDAF or DCCF whether it will retrieve the Data or Analytics

17. The DCCF or NWDAF indicate in the response to the ADRF whether the consumer will retrieve the Data or Analytics

18.	The Data or Analytics Consumer retrieves the stored data or analytics

Conditional on DCCF or NWDAF Managing the Storage Approach

19a-b.	The NWDAF or DCCF determines that the lifetime of the stored data has expired (according to the Storage Approach).

Conditional if the Storage Appoach is based on Default Operator Policies provisioned on the NWDAF or DCCF (see step 3a-c and clause 5B.1)

20.	The NWDAF or DCCF optionally retrieve the data or analytics from the ADRF

21.	The NWDAF or DCCF request that the data or analytics be deleted from the ADRF

Conditional if the Storage Appoach is based on a Storage Handling Request received from the Data or Analytics Consumer (see step 3a-c and clause 5B.1)

22a-b.	The NWDAF or DCCF sends a notification to the Data or Analytics Consumer alerting it that the data or analytics are about to be deleted.

23a-b.	The Data or Analytics Consumer indicates in the response to the NWDAF or DCCF whether it will retrieve the Data or Analytics

24.	The Data or Analytics Consumer retrieves the stored data or analytics

25.	The NWDAF or DCCF request that the data or analytics be deleted from the ADRF

26.	The ADRF deletes the data or analytics if:

-	the Storage Approach in the ADRF indicates alerting the consumer is not required prior to data or analytics deletion;

-	in Steps 13 or 17 the DCCF or NWDAF indicated data or analytics will not be retrieved prior to deletion;

-	data or analytics retrieval in steps 14 or 18 has completed or

-	A request to delete data or analytics was received in steps 21 or 25.

If the ADRF received a response from the NWDAF or DCCF in steps 14 or 17 indicating data or analytics will be retrieved but retrieval is not initiated before an adequate fixed time has elapsed, the ADRF may autonomously delete the data or analytics.

2nd change

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, ADRF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, ADRF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF, ADRF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, ADRF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |

3rd change

## 8.1	General

Table 8.1-1 shows the DCCF services and DCCF service operations.

Table 8.1-1: NF services provided by DCCF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndccf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF, OAM, CEF, LMF |
|  | Unsubscribe |  | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF, OAM, CEF, LMF |
|  | Notify |  | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF, MFAF, OAM, CEF, LMF |
|  | Fetch | Request / Response | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF, MFAF, OAM, CEF, LMF |
|  | Transfer | Request / Response | DCCF |
| Ndccf_ContextManagement | Register | Request / Response | NWDAF, ADRF |
|  | Update | Request / Response | NWDAF, ADRF |
|  | Deregister | Request / Response | NWDAF, ADRF |

4th change

## 9.1	General

Table 9.1-1 shows the MFAF services and MFAF service operations.

Table 9.1-1: NF services provided by MFAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmfaf_3daDataManagement | Configure | Request / Response | DCCF, NWDAF |
|  | Deconfigure | Request / Response | DCCF, NWDAF |
| Nmfaf_3caDataManagement | Notify | Subscribe / Notify | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF, OAM, CEF, LMF |
|  | Fetch | Request / Response | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF, OAM, CEF, LMF |

End of changes