SA WG2 Meeting #165	S2-2409868

Hyderabad, India, 14 October – 18 October, 2024

Source:	LG Electronics, NTT DOCOMO

Title:	Discussion on clarification regarding definition of 5G NR femto ownership

Document for:	Discussion

Agenda Item:	19.12.2

Work Item / Release:	5G_Femto / Rel-19

Abstract of the contribution: This paper discusses the LS from SA3 on Clarification regarding definition of 5G NR femto ownership.

Discussion

SA3 sent the LS on Clarification regarding definition of 5G NR femto ownership (S2-2409604/S3-243518) as below.

SA3 is studying the security aspects of 5G NR Femto as part of TR 33.745. In SA2 TR 23.700-45, following NOTE is observed. NOTE 1: Ownership of the 5G NR Femto (or CAG or both) concept and a mechanism will be defined in coordination with SA WG3. As per TS 33.320, Hosting party: The party hosting the H(e)NB and having a contract with the PLMN operator. SA3 considers that hosting party terminology should be inherited in 5G, and 5G NR Femto owner should be considered same as hosting party. Also, we recommend using hosting party as the terminology for normative work in 5G SA2 as well as SA3. SA3 requests SA2 to take above inputs and provide feedback if any.

To accommodate the SA3 observation and recommendation, two options/approaches can be considered as below.

Option#1

-	Defines "5G Femto Hosting Party" and uses it instead of "CAG owner".

-	For Option#1, S2-2409870 (TS 23.501 CR) and S2-2409871 (TS 23.502 CR) entitled "5G Femto Hosting Party" were submitted.

Option#2

-	Keeps using "CAG owner".

-	Defines "5G Femto Hosting Party" and clarifies that a 5G Femto Hosting Party can play the role of a CAG owner.

-	The motivation of Option#2 is as below:

•	CAG concept used by 5G Femto is also used by other entities such as MBSR (Mobile Base Station Relay) specified in Rel-18, MWAB (Mobile gNB with Wireless Access Backhauling) specified in Rel-19.
In addition, it is considered that 5G Femto Information provisioning (i.e. CAG information provisioning as specified in clauses 4.15.6.2 and 4.15.6.3h of TS 23.502) can be common and used by owner of equipment using CAG, i.e. not only by 5G Femto hosting party but also by other hosting party for equipment using CAG.

-	For Option#2, S2-2409872 (TS 23.501 CR) entitled "5G Femto Hosting Party acting as a CAG owner" was submitted.

Proposal

It is proposed to discuss and decide which option between Option#1 and Option#2 can be taken related to the LS from SA3.