3GPP TSG-SA2 Meeting #165	S2-2409934

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1173 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding HFL and VFL definitions, are required, and definitions have already been agreed in TR 23.700-84 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide Definitions for - Vertical Federated Learning - Horizontal Federated Learning - Vertical Federated Learning Client - Vertical Federated Learning Training Server - Label - Horizontal Federated Learning Server - Horizontal Federaed Learning Client |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear terminology |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4]. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Analytics Accuracy Information: Represent a performance measure of an analytics ID provided by an NWDAF containing AnLF, which is composed of the number of correct predictions of the analytics ID out of all predictions and the corresponding number of samples.

ML Model Accuracy Information: Represent a performance measure of a ML Model provided by an NWDAF containing MTLF, which is composed of the number of correct predictions by the ML Model out of all predictions and the corresponding number of samples.

Analytics Feedback Information: Indicates that the consumer NF has taken action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data.

Horizontal Federated Learning (HFL): A federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs).

Vertical Federated Learning (VFL): A federated learning technique without exchanging/sharing local data set and local ML models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs).

Label: A label corresponds to a ground truth for a property of interest that is to be learned in supervised machine learning

VFL Training Server: An NWDAF or AF participating in VFL training that discovers and selects VFL clients and coordinates the VFL training process. Only one VFL server may exist for each VFL process. VFL Server possesses the label (i.e. ground Truth) for the collaborative VFL training procedure.

VFL Client: An NWDAF or AF participating in VFL training with access to some required input data but without the required labels for a VFL training task. There can be multiple VFL clients in a VFL Training process.

HFL Server: A NWDAF that discover and selects HFL clients (NWDAFs) for the HFL procedure, requests the HFL clients to perfom local model training and generates a global model using updates from the HFL clients.

HFL Client:A NWDAF that trains local model tasked by HFL Server on local private dataset and reports the trained model information to HFL Server. HFL Client receives feedback from HFL Server as Global Model update.

# * * * * End of changes * * * *