SA WG2 Meeting #165 	S2-2409948

# 14 - 18 Oct 2024, Hyderabad, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5039 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UPF report via service based interface during UPF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are several issues in existing specification - In clause 4.15.4.5.6 the Relocation report indication is provided to UPF in the unsubscribe request, while in clause clause 4.15.4.5.7 the Relocation report indication is provided to UPF in the Subscribe request. It is proposed to provide the Relocation report indication in the Subscribe request. - According to clause 5.8.2.17 of TS 23.501, if the Analytics filter information includes the Area of Interest or DNAI, the consumer shall not use direct subscription to UPF event. The clause 4.15.4.5.7 needs to be corrected to align with 23.501. - The new parameters are missing in the service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - clarify that the SMF provide the Relocation report indication in the Subscribe request to the UPF - remove the parameters not used in clause 4.15.4.5.7 - Add new parameters in the SMF/UPF service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment within the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.6，4.15.4.5.7, 5.2.8.3.3, 5.2.26.2.2, 5.2.26.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

4.15.4.5.6	Information flow for subscription to UPF event exposure service via SMF during UPF relocation

Figure 4.15.4.5.6-1: Subscription to UPF event exposure service via SMF during UPF relocation

This information flow is used in the scenario of UPF relocation for the existing PDU Session, e.g. the source UPF is I-UPF or local PSA UPF which is relocated in SSC mode 1, or the source UPF is PSA UPF which is relocated in SSC mode 3.

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF data via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UE. The consumer may include a Relocation reporting indication, which indicates how to handle the data collected by the source UPF when UPF relocation occurs, e.g. discard the collected data or send the collected data to the consumer.

2.	The SMF subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation. The SMF may include the Relocation reporting indication received from the UPF event consumer.

3.	The source UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

4.	The SMF determines that the UPF needs to be relocated.

5.	The SMF unsubscribes to the source UPF by invoking Nupf_EventExposure_Unsubscribe service operation.

6.	The source UPF sends the collected data to the UPF event consumer if the relocation reporting indication from SMF indicates that the source UPF should send the collected data to the UPF event consumer. For example, for periodic reporting type of the UPF event exposure service when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change. Or for event triggered reporting with Reporting suggestion information, when the UPF changes, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet based on the relocation reporting indication.

Editor's note:	When the UPF changes, whether the UPF sends the data that has been collected but not sent to the consumer yet is based on the relocation reporting indication from consumer is FFS.

7-8.	To continue the subscription of the UPF event consumer, the SMF may select a target UPF, and re-subscribe to the target UPF based on the subscription from UPF event consumer in step 1. The target UPF will send the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

4.15.4.5.7	Information flow for subscription directly to UPF event exposure service during UPF relocation

Figure 4.15.4.5.7-1: Subscription directly to UPF event exposure service during UPF relocation

1.	(For the case when the UPF event consumer is NWDAF) The analytics consumer (e.g. AF/NEF) sends a request to the NWDAF for analytics, and indicates a single UE (e.g. UE IP address) in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, etc.

2.	If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the UPF event consumer triggers the UPF discovery to NRF by Nnrf_NFDiscovery_Request providing the UE IP address, and optional DNN, S-NSSAI, etc.

3.	The NRF provides Nnrf_NFDiscovery_Response that may refer to an UPF.

4.	The UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to the discovered UPF, and the Target of Event Reporting contains a single UE. The consumer may include a Relocation reporting indication, which indicates how to handle the data collected by the source UPF when UPF changes. The consumer may include an indication to receive subscription termination indication.

5.	The UPF invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

6.	When the PFCP session corresponding to the UE is released, e.g. due to UPF relocation, the UPF determines that the subscription to the UPF event has been terminated.

7.	If step 6 took place, the UPF sends the notification to the UPF event consumer by invoking Nupf_EventExposure_Notify service operation, which includes a subscription termination indication to indicate that the subscription to the UPF event has been terminated, and may also include a cause code indicating the reason for subscription termination, e.g. PFCP session release. The UPF may report to the consumer any data that has been collected but not sent to the consumer yet based on the Relocation reporting indication.

Editor's note:	When the UPF changes, whether the UPF sends the data that has been collected but not sent to the consumer yet is based on the relocation reporting indication from consumer is FFS.

* * * Next Change * * *

5.2.8.3.3	Nsmf_EventExposure_Subscribe service operation

Service operation name: Nsmf_EventExposure_Subscribe.

Description: This service operation is used by an NF to subscribe or modify a subscription for event notifications on a specified PDU Session or for all PDU Sessions of one UE, group of UE(s) or any UE.

Input, Required: NF ID, Target of Event Reporting as defined in clause 5.2.8.3.1, (set of) Event ID(s) defined in clause 5.2.8.3.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Input, Optional: Event Filter(s) associated with each Event ID; Event Filter(s) are defined in clause 5.2.8.3.1, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, DNAI, UPFId, UPF event exposure information (Type of measurement, granularity of measurement, reporting suggestion information, Relocation reporting indication, Subscription termination indication, etc. associated with the UPF Event IDs as described in clause 4.15.4.5).

NOTE:	The SMF is generally meant to determine the UPF to contact for a subscription related to UPF event exposure. UPF ID is only provided to indicate a UP Path as defined in Table 6.4.1-1 of TS 23.288 [50], i.e. when the NWDAF has received the target UPF as part of statistics of observed service experience on an UP path involving that UPF.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1).

Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by SMF or UPF with this subscription.

* * * Next Change * * *

5.2.26.2.2	Nupf_EventExposure_Notify service operation

Service operation name: Nupf_EventExposure_Notify

Description: This service operation reports the event and information to the consumer that has subscribed (implicitly or explicitly).

Input Required: Event ID, UE address (i.e. IP address or MAC address), Notification Correlation Information.

Input, Optional: UE ID, Event specific parameters as described in clause 5.2.26.2.1, time stamps for the measurements, Application Id(s), Packet Filter Set(s), Achieved sampling ratio, Subscription termination indication.

Output Required: Result Indication.

Output, Optional: None.

5.2.26.2.3	Nupf_EventExposure_Subscribe service operation

Service operation name: Nupf_EventExposure_Subscribe

Description: This service operation is used by an NF to subscribe or modify a subscription to UPF event exposure notifications e.g. for the purpose of UPF data collection on a specified PDU Session or for all PDU Sessions of one UE or any UE.

Input, Required: NF ID, Target of Event Reporting "any UE" or as defined in Table 4.15.4.5.1-1 when a specific UE is targeted), (set of) Event ID(s) defined in clause 5.2.26.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1 (TS 29.564 [93] defines the possible parameters).

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, either Application ID(s) or Traffic Filtering Information, Type of Measurement, Granularity of Measurement, Reporting suggestion information, Relocation reporting indication, UE (private) IP address, IP domain, IP address of the remote end, port number of the remote end.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1). Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by UPF with this subscription.

* * * End of Change * * *