3GPP TSG-WG SA2 Meeting #165	S2-2410005

14th – 18th October 2024, Hyderabad, India

Source:	Ericsson

Title:	Discussion on VFL

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract: In this paper some parameters supporting VFL is discussed

# 1. Introduction

### 1.1 Parameters needed to make VFL work

VFL makes it possible to not share models nor data sets when performing FL. Since models are not shared the features/input data is not revealed between participants. To not continue to rely on offline interoperability tests and the ML Model Interoperability indicator some interoperability information/parameters are needed to be specified. These parameters may be registered in NRF and/or set in preparation phase when training is starting.

Discussion item #1: It is not believed 3GPP shall restrict the VFL training method (neural networks, etc). This means this needs to be agreed before starting learning procedure. A participant may support multiple methods. Different methods may even be possible for one Analytics ID.

Observation #1: For interoperability purpose, the server needs to know which clients support certain training methods which therefore needs to be registered into NRF. To be able to select one of the supported methods for a particular VFL it should be hand-shaken between the participants in preparation phase.

Discussion item #2: There exists many different training methods, and new ones come very often. Too many to define in 3GPP. One common training method is neural networks (NN) where in its basic form constitutes of a server, which is in charge and collects intermediate results from clients. To manage this training method, additional interoperability information defining the cut layer between server and the clients are needed. This additional interoperability information shall not reveal the internals of the local models, nor the features.

Observation #2: For NNs, we can define additional interoperability information such as the dimensionality of the intermediate results including the number of samples, number of nodes in the cut layer for the intermediate matrix.
Maximum and minimum dimensionality per participant is valuable to have registered in NRF. To be able to select an appropriate dimensionality (less than maximum) for all participants for a particular VFL it should be hand-shaken between the participants in preparation phase.

Discussion item #3: Further, for NNs a simple approach is to let the server send gradients in Server intermediate matrixes to the clients, and clients send activations to server in Client intermediate matrixes. There are also other types of data that can be sent in intermediate matrixes (e.g. loss). Also, such intermediate matrixes can be calculated and sent at the same time in same message.

Observation #3: To indicate what is sent in the intermediate matrixes, an intermediate matrix indicator is needed. This indicator can easily be enhanced with any type of data sent in an intermediate matrix.

Discussion item #4: What is being defined in 3GPP is a fully automated VFL between participants, not involving any data scientist, nor control function. For all training types to be able to converge some inter-reporting between participants is needed to support the converge of the models.

Observation #4: Model convergence reports which are used to support the converge of a model during training is useful and needed in a real deployment. To have the possibility to perform more than basic VFL, without convergence support some of the reports shall be specified in 3GPP. The reports can easily be enhanced with vendor, application provider and operator specific reports. There can be many convergence reports, therefore only the support for such reports is registered into the NRF. To be able to know which reports that are supported for all participants for a particular VFL it should be hand-shaken between the participants in preparation phase.

Some references can be seen below for suggested reports:

-Expected number of iterations left.

-Convergence speed such as Fast/slow/almost done convergence. As an example, this may trigger the receiver to e.g., set the learning rate. Reference example - In “Increased Rates of convergence through learning rate adaptation” - [link] (Neural Networks, Elsevier, 1988) the authors show that the adjustment of learning rate can speed up convergence.

-Under or over fitting. This may trigger the receiver to e.g., expand or reduce the input data to be collected. Reference example - In “Revisiting Small Batch Training For Deep Neural Networks”, [link] the authors show that adjusting the batch-size, particularly employing mini-batches improves training performance.

-Not converging at all. This may trigger the receiver to e.g., change the ML Model.

-Stuck in a local critical point. May trigger the receiver to introduce random noise to its local model.

-Addition of random noise or similar in Intermediate results. This is sent to inform the receiver, that local monitoring of convergence may diverge from expected. Reference example - In “The Effects of Adding Noise During Backpropagation Training on a Generalization Performance”. [link] - “MIT Press, Neural Computation, Volume 8, Issue 3, April 1966” the authors show that the introduction of random noise

# 3. Conclusions and Proposals

Proposal #1: It is proposed to register a new parameter, named VFL Interoperability information containing the VFL training method (e.g., neural networks, etc) in NRF. It is also proposed to hand-shake on used method in preparation phase.

Proposal #2: It is proposed to define additional VFL interoperability information e.g. for NNs such as the dimensionality of the intermediate results including the number of samples, number of nodes in the cut layer for the intermediate matrix.

Proposal #2: It is proposed to define what is sent in the intermediate matrixes, namely an intermediate matrix indicator is needed.

Proposal #4: It is proposed to define model convergence reports which are used to support the converge of a model during training. Some of the reports shall also be specified.