SA WG2 Meeting #165	S2-2410016

October 14 – 18, 2024, Hyderabad, India

Source:	Ericsson

Title:	Handling of ATSSS capabilities when new MPQUIC options are added

Document for:	Discussion

Agenda Item:	19.13

Work Item / Release:	MASSS / Rel-19

Abstract of the contribution: This contribution discusses different open issue around how to introduce MPQUIC-E and MPQUIC-IP in rel-19.

# Introduction

This paper discusses different open issue around how to introduce MPQUIC-E and MPQUIC-IP in rel-19:

- 	UE and NW capability handling regarding what steering functionality is supported for a MA PDU Session. This includes both editorial and technical aspects.

- 	What steering functionalities are mandatory vs optional to support in UE/NW

- 	Handling of header compression for Ethernet PDU Sessions

# Existing (rel-18) capability negotiation (editorial aspects)

TS 23.501, clause 5.32.6.1 lists a set of UE capabilities indicating what steering functionalities the UE supports. There are in total 7 different UE capabilities listed (SM = steering mode), where the UE indicated which one (one of them) it supports:

1. 	ATSSS-LL with any SM

2. 	MPTCP with any SM + ATSSS-LL with Act-Stby only

3. 	MPTCP with any SM + ATSSS-LL with any SM

4. 	MPQUIC-UDP with any SM + ATSSS-LL with Act-Stby only

5. 	MPQUIC-UDP with any SM + ATSSS-LL with any SM

6. 	MPTCP with any SM + MPQUIC-UDP with any SM + ATSSS-LL with Act-Stby only

7. 	MPTCP with any SM + MPQUIC-UDP with any SM + ATSSS-LL with any SM

In NAS, this is implemented as per TS 24.193:

Table 6.1.6.2-1: ATSSS request PCO parameter container contents

| Supported ATSSS steering functionalities and steering modes (ATSSS-ST) (octet 1, bits 1, 2, 3 and 4) (see NOTE) |  |  |  |  |
| --- | --- | --- | --- | --- |
| This field indicates the 5GSM capability of ATSSS steering functionalities and steering modes. |  |  |  |  |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | ATSSS Low-Layer functionality with any steering mode allowed for ATSSS-LL supported |
| 0 | 0 | 1 | 0 | MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported |
| 0 | 0 | 1 | 1 | MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |
| 0 | 1 | 0 | 0 | MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported |
| 0 | 1 | 0 | 1 | MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |
| 0 | 1 | 1 | 0 | MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported |
| 0 | 1 | 1 | 1 | MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |
| All other values are reserved. |  |  |  |  |
| All other bits in octet 1 are spare and shall be coded as zero. |  |  |  |  |
| NOTE: If the ATSSS request PCO parameter is included in the PDN CONNECTIVITY REQUEST message with the request type information element set to "handover", the ATSSS-ST field is ignored. |  |  |  |  |

Observation 1: The UE indicates a combined capability, i.e. the UE does not indicate each supported steering functionality separately.

Furthermore, TS 23.501, clause 5.32.2 lists a set of NW capabilities/configuration that the SMF uses when it has received the UE ATSSS capabilities and determines what is supported in UL and DL for the MA PDU Session. This configuration on SMF determines what steering functionalities are supported/allowed for a DNN/S-NSSAI.  The following NW configurations are possible:

1. 	ATSSS-LL with any SM

2. 	MPTCP with any SM + ATSSS-LL with any SM, and able to measure RTT also if PMF cannot be used

3. 	MPTCP with any SM + ATSSS-LL with any SM except Smallest-Delay if PMF cannot be used

4. 	MPTCP with any SM + ATSSS-LL with Act-Stby only

5. 	MPTCP with any SM + ATSSS-LL with any SM

6. 	MPQUIC-UDP with any SM + ATSSS-LL with any SM, and able to measure RTT also if PMF cannot be used)

7. 	MPQUIC-UDP with any SM + ATSSS-LL with any SM except Smallest-Daley if PMF cannot be used

8. 	MPQUIC-UDP with any SM + ATSSS-LL with Act-Stby only

9. 	MPQUIC-UDP with any SM + ATSSS-LL with any SM

10.	MPTCP with any SM + MPQUIC-UDP with any SM + ATSSS-LL with any SM, and able to measure RTT also if PMF cannot be used

11.	MPTCP with any SM + MPQUIC-UDP with any SM + ATSSS-LL with any SM except Smallest-Delay if PMF cannot be used

12.	MPTCP with any SM + MPQUIC-UDP with any SM + ATSSS-LL with Act-Stby only

13.	MPTCP with any SM + MPQUIC-UDP with any SM + ATSSS-LL with any SM

Observation 2: As per TS 23.501, the network is configured with one out of 13 options for what steering functionalities are supported.

The SMF uses the UE capability (provided by the UE) and the NW configuration on SMF to determine the UL and DL capabilities for the MA PDU Session. The determined MA PDU Session capability is then provided to the PCF. The capabilities may be different in DL and UL.

23.501 uses a rather lengthy description, describing individual combinations for how MPCTP, MPQUIC-UDP and ATSSS-LL are supported in different combinations. This results in 13 different combinations in a bullet list in 23.501, clause 5.32.2 (bullets a – g, where bullets a,b,g also have sub-bullets). Text gets duplicated since e.g. the description for MPTCP ends up in all places where MPTCP is included in a combination. The combinations described are shown in Table 1 below, indicating which bullet in clause 5.32.2 refers to what combination.

Table 1. UE and network capability combinations described in TS 23.501

|  |  | NW configuration/capabilities |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1. ATSSS-LL (any SM) | 2. MPTCP + ATSSS-LL (RTT also if PMF cannot be used) | 3. MPTCP + ATSSS-LL (RTT only if PMF can be used) | 4. MPTCP + ATSSS-LL with Act-Stby only | 5. MPTCP + ATSS-LL (any SM) | 6. MPQUIC-UDP + ATSSS-LL (RTT also if PMF cannot be used) | 7. MPQUIC-UDP + ATSSS-LL (RTT only if PMF can be used) | 8. MPQUIC-UDP + ATSSS-LL with Act-Stby only | 9. MPQUIC-UDP + ATSS-LL (any SM) | 10. MPTCP + MPQUIC-UDP + ATSSS-LL (RTT also if PMF cannot be used) | 11. MPTCP + MPQUIC-UDP + ATSSS-LL (RTT only if PMF can be used) | 12. MPTCP + MPQUIC-UDP + ATSSS-LL with Act-Stby only | 13. MPTCP + MPQUIC-UDP + ATSS-LL (any SM) |
| UE capabilities | 1. ATSSS-LL (any SM) | d) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2. MPTCP + ATSSS-LL (Act-Stby only) |  | a.i) | a.ii) | a.iii) |  |  |  |  |  |  |  |  |  |
|  | 3. MPTCP + ATSSS-LL (any SM) |  |  |  |  | e) |  |  |  |  |  |  |  |  |
|  | 4. MPQUIC-UDP + ATSSS-LL (Act-Stby only) |  |  |  |  |  | b.i) | b.ii) | b.iii) |  |  |  |  |  |
|  | 5. MPQUIC-UDP + ATSSS-LL (any SM) |  |  |  |  |  |  |  |  | c) |  |  |  |  |
|  | 6. MPTCP + MPQUIC-UDP + ATSSS-LL (Act-Stby only) |  |  |  |  |  |  |  |  |  | g.i) | g.ii) | g.iii) |  |
|  | 7. MPTCP + MPQUIC-UDP + ATSSS-LL (any SM) |  |  |  |  |  |  |  |  |  |  |  |  | f) |

The SMF indicates to PCF which one of these 13 capabilities applies for the MA PDU Session. TS 29.512, clause 5.6.3.26, defines the alternative MA PDU Session capabilities that can be provided to the PCF (copied in Table 2 below).

Table 2: Enumeration AtsssCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ATSSS_LL | Indicates that the MA PDU Session supports the ATSSS-LL capability with any steering mode in the uplink and in the downlink. |  |
| MPTCP_ATSSS_LL | Indicates that the MA PDU Session supports both the MPTCP and ATSSS-LL capability with any steering mode in the uplink and in the downlink. |  |
| MPTCP_ATSSS_LL_WITH_ASMODE_UL | Indicates that the MA PDU Session supports the MPTCP capability with any steering mode in uplink and downlink, and ATSSS-LL capability with any steering mode in the downlink and Active-Standby mode in the uplink. |  |
| MPTCP_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL | Indicates that the MA PDU Session supports the MPTCP capability with any steering mode in uplink and downlink, and ATSSS-LL capability with any steering mode except Smallest Delay mode in the downlink and Active-Standby mode in the uplink. |  |
| MPTCP_ATSSS_LL_WITH_ASMODE_DLUL | Indicates that the MA PDU Session supports the MPTCP capability with any steering mode and ATSSS-LL capability with Active-Standby mode in uplink and downlink. |  |
| MPQUIC_ATSSS_LL | Indicates that the MA PDU Session supports the MPQUIC and ATSSS-LL capability with any steering mode in the uplink and in the downlink. | EnATSSS_v2 |
| MPQUIC_ATSSS_LL_WITH_ASMODE_UL | Indicates that the MA PDU Session supports the MPQUIC capability with any steering mode in uplink and downlink, and ATSSS-LL capability with any steering mode in the downlink and Active-Standby mode in the uplink. | EnATSSS_v2 |
| MPQUIC_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL | Indicates that the MA PDU Session supports the MPQUIC capability with any steering mode in uplink and downlink, and ATSSS-LL capability with any steering mode except Smallest Delay mode in the downlink and Active-Standby mode in the uplink. | EnATSSS_v2 |
| MPQUIC_ATSSS_LL_WITH_ASMODE_DLUL | Indicates that the MA PDU Session supports the MPQUIC capability with any steering mode and ATSSS-LL capability with Active-Standby mode in uplink and downlink. | EnATSSS_v2 |
| MPTCP_MPQUIC_ATSSS_LL | Indicates that the MA PDU Session supports the MPTCP, MPQUIC and ATSSS-LL capability with any steering mode in the uplink and in the downlink. | EnATSSS_v2 |
| MPTCP_MPQUIC_ATSSS_LL_WITH_ASMODE_UL | Indicates that the MA PDU Session supports the MPTCP and MPQUIC capability with any steering mode in uplink and downlink, and ATSSS-LL capability with any steering mode in the downlink and Active-Standby mode in the uplink. | EnATSSS_v2 |
| MPTCP_MPQUIC_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL | Indicates that the MA PDU Session supports the MPTCP and MPQUIC capability with any steering mode in uplink and downlink, and ATSSS-LL capability with any steering mode except Smallest Delay mode in the downlink and Active-Standby mode in the uplink. | EnATSSS_v2 |
| MPTCP_MPQUIC_ATSSS_LL_WITH_ASMODE_DLUL | Indicates that the MA PDU Session supports the MPTCP and MPQUIC capability with any steering mode and ATSSS-LL capability with Active-Standby mode in uplink and downlink. | EnATSSS_v2 |

Observation 3: The MA PDU Session capability provided to PCF is also a “combined” capability, indicating one option out of 13.

Adding two new capabilities (MPQUIC-E and MPQUIC-IP) will result in a significant increase in the number of possible combinations in the UE and in the NW. Instead of 7 UE capabilities we would have 17, and instead of 13 combinations of UE and NW capabilities we would have 23 combinations (see below). This may not be an issue for stage 3 as it is a matter of adding rows in a table but extending the verbose description of combinations in TS 23.501 would result in a long list with a lot of duplicated text that is hardly readable.

Proposal 1: The text in 23.501 for handling UE and NW capabilities for MA PDU Sessions should be reworked to avoid it becoming too long and too hard to parse. This is an editorial task.

# Rel-19 capability negotiation (technical aspects)

Below we discuss how to extend the rel-18 capabilities to include the new rel-19 steering functionalities MPQUIC-IP and MPQUIC-E.

##### IP PDU Sessions

In rel-18 there is no mandatory steering functionality for IP PDU Sessions, but a UE (and NW) that does not support ATSSS-LL with any SM (i.e. UE that only support MPTCP and/or MPQUIC-UDP) shall support ATSSS-LL with Active Standby. This is required to handle traffic that cannot be sent via MPQUIC-UDP/MPTCP. With the introduction of MPQUIC-IP this is in principle not needed, since MPQUIC-IP can carry any traffic. However, even for a UE/NW that support MPQUIC-IP there could still be a benefit to be able to send some traffic without MPQUIC, using ATSSS-LL with Active Standby.

Proposal 2: A UE/NW that supports MPQUIC-IP shall also support ATSSS-LL with Active-Standby, similar to what is required for a UE/NW supporting MPQUIC-UDP and/or MPTCP.

##### Ethernet PDU Sessions

The only steering functionality available in rel-18 for Ethernet PDU Sessions is ATSSS-LL. Due to this, it was regarded as mandatory in rel-18 for Ethernet MA PDU Sessions. With the introduction of MPQUIC-E, ATSSS-LL is no longer the only steering functionality available for Ethernet and it could be possible to make ATSSS-LL no longer mandatory. However, similar to MPQUIC-IP, even for a UE/NW that support MPQUIC-E there could still be a benefit to be able to send some traffic without MPQUIC, using ATSSS-LL with Active Standby.

Proposal 2: A UE/NW that supports MPQUIC-E shall also support ATSSS-LL with Active-Standby. Full ATSSS-LL support is not required.

It may be argued that making full ATSSS-LL support optional for rel-19 is not backwards compatible with rel-18 and that this can result in e.g. that a UE supporting rel-18 MA PDU Sessions for Ethernet can no longer be served by a rel-19 network if it only supports MPQUIC-E, or the other way around. However, it is always up to the operator what features it wants to support in its network, so the standard cannot mandate a network to support ATSSS-LL. Regarding the UE, it is of course nice if the UEs support as much functionality as possible, but there are not many rel-16 – 18 UEs available that support ATSSS for Ethernet. Mandating too much functionality causes complexity and creates a higher barrier for a wider support. Also, with the requirement that rel-19 UEs at least needs to support ATSSS-LL with Active-Standby, there is in fact a “least common denominator” also in that case, i.e. to use ATSSS-LL with Active-Standby.

This can also be applied to IP PDU Session types to handle “mismatch” cases. For example, if the UE supports MPQUIC-UDP and ATSSS-LL with Active-Standby (as per rel-18) and the network allows only MPQUIC-IP and ATSSS-LL with any SM, then the MA PDU Session can use ATSSS-LL with Active-Standby in UL and ATSSS-LL and potentially ay SM in DL.

Proposal 3: ATSSS-LL with Active-Standby is a common denominator for all steering functionalities that can be applied if there is a mismatch in capabilities.

# Handling of header compression

Three cases have been discussed for how to handle header compression for Ethernet PDU Sessions:

a) 	If MA PDU Session only uses ATSSS-LL, SMF indicates Ethernet PDU Session type to RAN. RAN can use EHC.

b) 	If MA PDU Session only uses MPQUIC-E, SMF indicates and IP based PDU Session type to RAN. RAN can use ROHC.

c) 	If MA PDU Session uses both ATSSS-LL and MPQUIC-E, SMF indicates Unstructured PDU Session type to RAN. No HC will be used.

The drawback with Alt c) is that header compression cannot be used, and that the MA PDU Session would be limited to a single QoS Flow. It is also not clear whether there is an important use case where both ATSSS-LL and MPQUIC-E need to be used on the same MA PDU Session. Therefore, it is suggested that only Alt a) and b) are supported in rel-19.

Proposal 4: It is assumed that the MA PDU Session either uses only MPQUIC-E or only ATSSS-LL. If both are supported by the UE and the network, the PCF ensures that the MA PDU Session capability is limited to either MPQUIC-E or ATSSS-LL, i.e. all SDFs in a MA PDU Sessions uses either MPQUIC-E or ATSSS-LL. The PCF indicates to the SMF during MA PDU Session establishment whether MPQUIC-E or ATSSS-LL is used for the MA PDU Session, which allows the SMF to provide the proper PDU Session Type to RAN.

# Proposal

The proposals are implemented in 23.501 CRs S2-2410017 (basic aspects of MPQUIC-IP and MPQUIC-E) and S2-2410018 (capability exchange between UE and network).