3GPP TSG-WG SA2 Meeting # 165	S2-2410030

# Hyderabad, IN, 14 - 18 October 2024										(revision of S2-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5697 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB UE Access Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce the UE access control. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce UE access control description in clause 5.49.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functions related to access control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.49.6	Control of UE access to MWAB

The control of UE access to MWAB cell is supported. If the MWAB-gNB is serving a PNI-NPN, the existing CAG mechanism described in clause 5.30.3 is used for managing UE access to MWAB cell with the following consideration:

-	When the MWAB is allowed to operate as an MWAB-gNB for a PLMN, the MWAB-gNB is configured with a CAG identifier which is unique within the scope of this PLMN, either during the communication with the serving PLMN OAM or based on (pre-)configuration mechanism. If the MWAB is (pre-)configured with the PLMN list in which the MWAB is allowed to operate as MWAB-gNB, the corresponding CAG Identifier per PLMN is also configured in the MWAB.

-	NG-RAN (i.e. MWAB-gNB) and 5GC support the UE access control based on the CAG identifier associated with the MWAB cell and the allowed CAG identifiers for the UE that supports CAG functionality.

NOTE 1: 	For a UE that does not support CAG functionality, NG-RAN and 5GC are allowed to use not only CAG mechanism but also other existing mechanism, e.g., forbidden Tracking Area, to manage its access to MWAB.

-	Time validity information may be provided together with the CAG Identifier(s) for the MWAB(s) that the UE access. The Allowed CAG list will be provided to UE and MWAB-gNB by the AMF for enforcement, to make sure that UE not accessing the MWAB cell outside of the time duration. For example, if the time when a certain CAG is allowed for a UE is up, the CAG for the UE is revoked from the network.

If the MWAB-gNB is serving an SNPN, the existing SNPN control mechanism described in clause 5.30.2.5 is used for managing the UE access to MWAB cell with the following consideration:

-	Dedicated SNPN ID(s) or GIN(s) is used if differentiation is desired to control UE access to the SNPN via MWAB-gNB or other fixed gNBs.

NOTE 2:	MWAB-UE's access to the network is controlled with mechanisms defined in clause 5.2 for PLMN and clauses 5.30.2 and 5.30.3 for SNPN/PNI-NPN, based on subscription data of the MWAB-UE.

# * * * * End of changes * * * *