3GPP TSG-WG SA2 Meeting # 165	S2-2410033

# Hyderabad, IN, 14 - 18 October 2024										(revision of S2-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5700 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB QoS realization and handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce QoS handling clarification for MWAB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce QoS handling clarification for MWAB in 5.49.1.X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear QoS handling with MWAB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.49.1.x	QoS realization for MWAB

When MWAB-gNB provides service to connected UEs, the traffic from and to the UEs is delivered via the BH PDU sessions of the MWAB-UE. The MWAB-UE considers the traffic from MWAB-gNB as application layer traffic and applies the related QoS handling as specified in clause 5.7, e.g., performing QoS flow mapping and marking.

The MWAB-gNB and the UPFs, in the MWAB broadcasted PLMN, set proper marking information (DSCP) for the N3 GTP-U traffic for the UEs served by the MWAB based on traffic mapping specified in TS 38.414 [195]. The MWAB-UE and BH UPFs also deliver the traffic in the BH PDU sessions based on the QoS handling as specified the clause 5.7 (i.e. mapping DSCP values to QoS flows of the BH PDU sessions).

The MWAB-UE may establish the BH PDU sessions accordingly based on the needs of the traffic of the MWAB-gNB. The BH 5GC may also modify the BH PDU sessions based on existing QoS mechanisms. Based on implementation based internal communication between MWAB-gNB and MWAB-UE, the MWAB-gNB may provide necessary information to MWAB-UE (e.g. information related to the QoS of PDU sessions of the UEs served by the MWAB-gNB or information from OAM) for the QoS handling described above.

# * * * * End of changes * * * *