3GPP TSG-WG2 Meeting #165	S2-2410043

# Hyderabad, India,  2024-10-14 -- 2024-10-18

Source:	Ericsson, Nokia, ETRI, Vodafone

Title:	UPEAS_Ph2: KI#3: N4 Rules

Document for:	Discussion

Agenda Item:	19.11.2

Work Item / Release:	UPEAS_Ph2 / Rel-19

Abstract: Provides arguments for which N4 rules to use for Handling of Payload Headers

# 1. Introduction

According to the UPEAS Phase 2 WID, for KI #3, the objective is to incorporate the conclusions in TR 23.700-63 including:

-	Enhancing N4 PFCP rules to instruct reporting of headers/tags handling upon AF request according to option 2 in TR 23.700-63. The impacts on N4 need to be determined.

During SA2 #164 it was not possible to agree the CR with impacts to TS 23.501 due to lack of time. Tdoc S2 –2409555 is the latest revision of the CR with the proposed updates to TS 23.501. The proposal is that UPF reports only using Nupf_Event Exposure (also when consumer is SMF). The Subscription to notification the event is conveyed by SMF to UPF over N4. No N4 reports are needed.

SA2 #164 left the N4 rules to provision the instructions from SMF to UPF for future discussion. This DP proposes a way forward.

# 2. Discussion

The instructions that SMF send to UPF on N4 for Handling of Payload Headers are based on the information in the AF request. As agreed, the API for influence on routing and service function chaining is extended for Handling of Payload Headers and the parameters that are shared with these other types of requests are: target UE identifiers and traffic descriptors, which are mandatory, and Spatial and Temporal validity conditions, which are optional.

The parameters in the AF request that are specific for Handling of Payload Headers are (from S2 –2409555):

1)  Header Handling Detection Reference:  Reference to a predefined configuration in UPF which defines the header handling detection information (e.g. protocol layer, type of encryption, message type etc).  This information is needed for manipulation and/or reporting of detected headers.

2)  Header Handling Information: The Header Information contains input information to identify the Header(s) for the Header Handling Manipulations and/or reporting.

3)  Header Handling Additional Data: This additional data complements the Header Handling Information.

4)  Header Handling Direction: Header Handling Direction indicates to which direction the Header handling Control information applies; uplink (UL) or downlink (DL).

5)  Header Handling Manipulation(s):  The manipulation(s) that AF may request on the target traffic. One or more of the following can be requested: Remove, Replace or Insert.

6)  Header Handling Condition: This defines how to apply the Header Manipulation. It may have one of the following values: every match related to the Handling of Headers Control Information or only for the first match

7)  Subscription information for header handling events when AF subscribes to receive reporting of header handling detection or of header handling manipulation from UPF. The AF may need to subscribe via NEF. SMF may also subscribe notifications

8)  Header handling pre-configuration reference: This refers to a pre-configuration in UPF for parameters 2-6.

TS 23.501 clause 5.8.5 describes the parameters for N4 session management. TS 29.244 specifies the stage 3 of the interface between the SMF and the UPF.

Observation 1: Handling of Payload Headers is an optional feature that allows exchange of information in-band in the user plane packets that are exchanged between the application (in the UE or in the application server) and UPF in the 5GS. The AF request does not include QoS enforcement control information, which is the scope of QER.

Observation 2: Header Enrichment (HE) has certain functional overlap with Handling of Payload Headers. In the baseline, HE controls insertion of HTTP headers in UL using IEs in FAR.

Observation 3: AF influence on routing and service function chaining use the same API that handling of Payload Headers. For those, Target UE identifiers determine the target PFCP sessions and the traffic descriptors are input to PDR (PDI). The rest of control information is provided by SMF to UPF using FAR.

Conclusion 1: FAR seems a good candidate for the instructions for Handling of Payload Headers.

Observation 4: SMF provisions different PDRs for UL and DL to UPF. 4) Header Handling Direction will be used by SMF to decide whether the instructions need to be included in UL or DL rules. It will not be as such an IE of the Rules.

An AF may request manipulation of headers (with or without reporting), reporting upon detecton of headers, or for both.

AF request for insertion, removal or replacement of headers will become instructions for how the packet should be processed by UPF. According to TS 29.244 clause 5.2.1, Each PDR shall contain a PDI, i.e. one or more match fields against which incoming packets are matched, and each PDR may be associated, among other, to one or more FARs, which contain instructions related to the processing of the packets.

Observation 5: When AF request is for traffic manipulation, AF input becomes SMF instructions to UPF for how the packet shall be processed, which is according to FAR scope. Parameters involved are 1), 5),6) 2),3) and 8).

Then, 1) Header Handling Detection Reference instructs UPF in relation to the protocol (and message) where the requested manipulations shall be performed. One may wonder if it should be part of PDR (Packet detection Rule) instead. Options include:

-  In PDR (PDI). As a consequence, UL and DL rules would need to be also provided for the traffic of the application that does not require the manipulation, multilpying the number of PDRs in the PFCP session.

-  In PDR but outside PDI. Following the example of Rel-18 XRM, where protocol description is in PDR and the QoS related instructions are in QER. But the XRM split intended to facilitate QER reusage by several applications and that is not foreseen for Handling of Payload Headers; AF request includes many parameters which are likely to be different for the different applications and FAR reusage is unlikely.

Observation 6: no benefits have been identified from the addional complexity of placing 1) in the PDR instead of with the rest of manipulation instructions in FAR, since reusage of FAR by several applications is not foreseen likely.

In addition, AF request for manipulation may also request reporting when the manipulation has been performed. In the baseline, N4 Session Reporting Rule (SRR) is used to send to UPF Subscripition to Nupf event exposure events. The subscription to new UP Handling of Payload Headers could be be included in SRR too. But for this event, cross-references between FAR and SRR to correlate the manipulation instructions and the corresponding subscription would have to be added increasing complexity for no real benefit.

Observation 7: no benefits have been identified from the additional complexity of placing 7) in SRR instead of with the rest of instructions in FAR.

Then, AF may send (only or in addition to the above) a request for reporting upon detection of headers. The AF request in this case is not for packet processing and other than FAR could be used. But the parameters provided by AF in this case are common to parameters used in AF manipulation requests. So, Should 3GPP actually define a new type of rule for the Handling of Payload Headers instructions (all use cases) that can also be associated to PDR? What would be the scope of the new Rule? Taking into account than other (similar) packet processing functionality is already part of FAR (e.g. HE) the scope and boundaries between these two rules would become blurry. In addition, having several rules which could impact the processing of packets complicates the resolution of possible interactions. A new rule is more likely to add confusion than structure and clarity and complicates implementations.

Observation 8: no benefits have been identified from the addional complexity of specifying a new type of rule that can also be associated to PDRs instead of using FAR for the Handling of Payload Headers instructions.

Conclusion 2: Handling of Payload Headers instructions in FAR is the simplest solution and according to FAR scope in many cases.

# 3. Proposal

Include in FAR the instructions that SMF sends to UPF to control Handling of Payload Headers