3GPP SA WG2 Meeting #165	S2-2410057

Hyderabad, IN, 14th -18th October 2024

Title:	[Draft] LS out on Application-layer FEC awareness at RAN

Response to:	-

Release:	Release 19

Work Item:	NR_XR_Ph3-Core / FS_XRM_Ph2

Source:	SA2

To:	RAN2

Cc:	SA4

Contact Person:

Name:	Lizhuo Zheng

E-mail Address:	zhenglz4 AT lenovo DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

After SA2#164 meeting in August 2024, conclusion for KI#1 on Application layer FEC has been agreed (TR 23.700-70 Clause 8.1.4). However, there is still one NOTE in the description that depends on RAN 2 further feedback, which is said as follows:

The principles concluded for AL-FEC awareness at RAN (as above) is subject to RAN-level enhancements of RLC Acknowledged Mode (RLC AM) for XR as per RP-240791 and further feasibility analyses and feedback from RAN WG2. RLC UM is not intended to be supported in this release due to concerns shared so far from RAN WG2 (see R2-2405781), pending further status update from RAN WG2.

The concern shared in the RAN2 LS reply (R2-2405781) is listed below:

RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer.

As can be seen, the concern shared in RAN2 LS has been avoided/addressed since the AL-FEC awareness at RAN will only be conducted under RLC AM mode. Under this mode, RAN can get the information about the successful delivery of the PDUs from UE and determine when to discard the remaining PDUs during congestion, based on the available content ratio, which is transmitted via control plane to the RAN from AF. What’s more, NG-RAN only discards the PDUs of the QoS flow during the congestion, which will not cause the confusion to the APP behaviour from SA4 perspective.

Based on the discussion above, SA2 believes that it is technically feasible to start research of the AL-FEC awareness at RAN from the core network perspective.

2. Actions:

To RAN2:

ACTION: 	SA2 would like to ask RAN2 to take the conclusion principles made in SA2 into consideration and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #166		18-22 November 2024	Orlando, FL, USA

TSG-SA2 Meeting #166 e-meeting		20-24 January 2025