3GPP TSG-SA2 Meeting #165	S2-2410072

# 14 – 18 October 2024, Hyderabad, India							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5711 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General description of relaying media related information over N6 using an encapsulation protocol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_ph2 |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR provides description of conveying metadata information over an N6 tunnel as per KI2 XRM study agreements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change proposes the following: - In a new clause add a general section to describe that the procedure is used to convey media related information (e.g. metadata) from the AS to the UPF via an N6 tunnel. An example when such method is needed is when the e2e conneciton is fully encrypted - Add three sub-clause to describe Connect-UDP/QUIC-Aware Proxy, Media Over QUIC and UDP-Options methods - Add description that the type of encpasulation protocol used in the N6 tunnel is provided within the ProtocolDescription. In additon the FQDN of the AS is included as a parameter outside the protocol description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI2 conclusions are not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.x (new), 5.37.x.1 (new), 5.37.x.2 (new), 5.37.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

### 5.37.x	Support of relaying media related information over N6

#### 5.37.x.1	General

This clause describes the procedure to support relaying of media related information (e.g. PDU set information) over N6. Examples where such such support is necessary is when the end-2-end connection between the UE and Application Server is fully encrypted thus UPF not being able to determine, for example, PDU set information when the XR media are conveyed over the 3GPP network.

There are three methods supported to provide media related information over N6.

- Using Proxying-UDP-in-HTTP RFC 9298 [170] and QUIC-Aware Proxy[y]

- Using Media over QUIC (MoQ) [z]

- Using UDP-Options [r]

Supported Media Related Information that are relayed over N6 are:

- PDU Set Information (as described in clause 5.37.5.2)

- Power Saving (as described in clause 5.37.8.3)

- Boost Indication as per clause xyz

- Data Burst size as per clause xyz

Editor’s Note: Other parameters are FFS

NOTE X: In this release Media over QUIC supports conveying PDU set information media related information.

#### 5.37.x.2	Relaying media related information over N6 using Proxying-UDP-in-HTTP and QUIC-Aware Proxy

In this method Media Related Information are included in HTTP datagrams [170] or packet transforms defined in [z].

Upon an AF session request with QoS the AF additionally provides the Protocol Description for media related information over N6 the AS address and the encapsulation protocol to enable a UDP tunnel connection between the UPF and AS using RFC9298 [170] or QUIC-Aware Proxy[y], the PCF generates PCC rules including the indication of the UDP tunnel establishment and AS address to the SMF. The SMF in turn provides corresponding N4 rules to the UPF.

Based on the N4 rules, the UPF supporting HTTP3/Client functionality establishes a UDP tunnel to the AS (supporting HTTP3/Proxy functionality) using the "connect-udp" HTTP upgrade token as defined in RFC9298 [170]. The UPF and the AS negotiate whether QUIC-Aware proxying tunnel can be setup as defined in [z].

NOTE X:	QUIC-Aware Proxying applies when the e2e media session between the UE and AS is based on QUIC protocol.

In the downlink AS forwards UDP payload packets over N6 in “tunnel-mode” as in RFC9298 [170] or in “forwarded-mode” as in [z].

In “tunnel mode”, media related information is encoded into the UDP payload packets in a HTTP Datagram by using a specific transport mode. The following transport modes are supported:

- N6 Tunnel Datagram mode 1: This transport mode is an extension of the mode defined in RFC 9298 [170]. It encapsulates UDP payload packets within HTTP Datagram frames and provides unreliable transport including media related information. The details of the datagram mode, including the potential use of a Context ID (see RFC 9298 [170]), are considered in stage-3 specifications.

Editor's note:	A reference to the applicable stage-3 specification needs to be added to the above paragraph, to point to the stage-3 details of this datagram mode.

Editor’s note:	Additional N6 Tunnel Datagram modes are FFS

In “forwarded mode”, media related information is included into the UDP payload packets via a new packet transform. The details of the packet transform are considered in stage-3 specifications. In “forwarded mode the UPF manages the Connection ID changes from the end-2-end QUIC connection. If for any reason, the UPF cannot reliably determine the Connection ID, subsequent packets are sent in “tunnel mode” until the end-to-end QUIC connection is closed.

The UPF extracts media related information from the UDP payload packets and performs actions for PDU set identification as per clause 5.37.5.2, Power Saving as per clause 5.37.8.3.

#### 5.37.x.3	Relaying media related information over N6 using Media over QUIC

In this method Media Related Information are encoded over Media over QUIC [z] parameters.

NOTE Z:	In this Release only PDU Set Information as per clause 5.37.5.2 are supported Media Related Information

Prerequisites of this method is that the UE establishes an MoQ session with an Application Server via a UPF supporting MoQ Relay Functionality defined in MoQ [z].

Editor’s note:	How the UE discovers a UPF supporting MoQ relay functionality is FFS.

Upon an AF session request with QoS the AF additionally provides the Protocol Description for MoQ [z] to enable identification of PDU Set Information and optionally the AS address, the PCF generates PCC rules including the Protocol Description and AS address to the SMF. The SMF in turn provides corresponding N4 rules to the UPF supporting MoQ relay functionality [z] to enable the UPF to identify PDU Set Information from MoQ traffic.

A UPF supporting MoQ Relay functionality [z] identifies PDUs of a PDU Set and PDU Set Information from MoQ Datagram messages defined in MoQ [z] and performs actions for PDU set identification as per clause 5.37.5.2.

#### 5.37.x.4	Relaying media related information over N6 using UDP-Options

FFS

************************************ SECOND CHANGE *****************************

#### 5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as a video frame or video slice. The PDU Set based QoS handling by the NG-RAN is determined by PDU Set QoS Parameters in the QoS profile of the QoS Flow (specified in clause 5.7.7) and PDU Set Information provided by the PSA UPF via N3/N9 interface as described in clause 5.37.5.2. The PDU Set based Handling can be applied for GBR and non-GBR QoS Flows. The AF should provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS Parameters as described in clause 5.7.7

-	Protocol Description: Indicates the transport protocol used by the service data flow and information, e.g. the following:

-	RTP [185] or SRTP [186];

-	RTP or SRTP with RTP Header Extensions, including:

-	RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179];

-	Other RTP Header Extensions as defined RFC 8285 [189];

-	RTP or SRTP without RTP Header Extensions, but together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179], and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with other RTP Header Extensions following RFC 8285 [189], and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]).

-	Encapsulation protocol used for transporting the application layer payload and conveying additional information (e.g. PDU-Set Marking) (i.e., Connect-UDP as per RFC 9298 [170], QUIC-Aware Proxy [z], UDP-Options [x], Media over QUIC [x])

-	The format for conveying PDU set marking for each encapsulation protocol is described in clause 5.37.x.

NOTE 1:	With the Protocol Description options combining SRTP together with RTP Payload Format the UPF can still obtain some of the PDU Set information from the RTP Header (refer to Annex A of TS 26.522 [179]).

When RTP Header Extensions for PDU Set Marking (as defined in TS 26.522 [179] or other RTP header extensions as defined in RFC 8285 [189] is included, the differentiation between different RTP Header Extension Types should be supported.

When RTP Payload Format is included, the differentiation between different RTP Payload Formats should be supported.

When encapsulation protocol is included as part of the Protocol Description the FQDN of the AS is also included

NOTE 2:	Multiplexing of different transport protocols and different media traffic for differentiated PDU Set based handling is not supported in the current Release.

The Protocol Description can be UL only, DL only or UL and DL. The Protocol Description for UL and DL traffic may be different.

AF provided PDU Set QoS Parameters and UL and/or DL Protocol Description may be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45] and the DL Protocol Description may be used for identifying the PDU Set Information and PDU Set Information marking by the PSA UPF.

When the SMF receives the PCC rule, the SMF performs binding of the PCC rule to one QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45]. At least one of the following shall be included in the PCC rule to enable PDU Set based handling: 1) a PSIHI and/or 2) both PSDB and PSER. Based on the PCC rule, the SMF adds the PDU Set QoS Parameters to the QoS Profile of the QoS Flow as described in clause 6.2.2.4 of TS 23.503 [45]. Alternatively, the SMF may be configured to support PDU Set based Handling without receiving PCC rules from a PCF.

For the downlink direction, the PSA UPF identifies PDUs that belong to PDU Sets and marks them accordingly as described in clause 5.37.5.2. If the PSA UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the PSA UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2.

NOTE 3:	If the PSA UPF receives a PDU that does not belong to a PDU Set, then it is assumed that the UPF determines the PDU Set Importance value based on pre-configuration.

For the uplink direction, the UE may identify PDU Sets, and how this is done is left up to UE implementation. The SMF may send the UL Protocol Description associated with the QoS rule to UE.

NOTE 4:	Using the Protocol Description or not is left to UE implementation. The use of Protocol Description does not impact QoS Flow Mapping in the UE.

In this Release, the PDU Set based handling is supported in 5GS for UE registered in 3GPP access for single access PDU Session with IP PDU Session Type.

************************************ END OF CHANGES ****************************************