3GPP TSG-SA WG2#165	S2-2410076

Hyderabad, India, 14-18 October 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated, Google

Title:	Motivation for support of emergency SMS over NAS

Document for:	Discussion

Agenda Item:	30.2

Work Item / Release:	NA/Rel-19

Abstract of the contribution: Describes the rationale to support emergency SMS over NAS as TEI19.

# 1.	Background

Service requirements for emergency SMS were defined in clause 10.11 of  3GPP TS 22.101 in Release 18. These service requirements apply only to SMS over IP (using IMS).

SA2 could not agree on a TEI19 WI at SA2#161 since there were two different technical proposals (one from Qualcomm and one from other companies) and SA2 could not find a way forward. No study is allowed according to SA2 rules to compare these two alternatives.

From the perspective of the authors, the design for SMS over IP (using IMS) would involve new UE impacts and therefore has to be able to support more advanced needs. Deployment of new phones will take several years from now. The same time can be used for upgrade of legacy PSAPs. Support of legacy emergency SMS can remain in PSAPs but usage will decline as older phones are replaced by new phones.

# 2.	Proposed way forward

For SMS over NAS there is a “template” for support of SMS to emergency number (112, 911) for roaming users that is used for transporting AML using SMS

The solution is documented in clause 6.4.2 and Annex D.2 of GSMA NG.119: https://www.gsma.com/newsroom/wp-content/uploads//NG.119-V2.1-2.pdf

Figure: Sending AML for roamers (extracted from NG.119)

The solution is supported in several existing (aka legacy) UEs deployed on the field and deployed in several EU countries.

It is based on the principle that the UEs (supporting AML for roamers), SMS client in HLOS will replace the current SMSC address with the visited SMSC address (CC+112/911). Destination number is still unchanged (Transaction Processing TP-Destination-Address=112/911 or local emergency numbers).

The MME (when SMS in MME is used) or SMSF (in 5GS) or MSC (when using SGs)  in VPLMN will be responsible to route the SMS to the visited SMSC. In the context of the SA2 TEI19 we propose the support for emergency SMS in MME and 5GS using SMSF. When/if the same solution can be used for CS (MSC), the possible impacts are out of scope of SA2.

The solution is a low hanging fruit can be documented in 3GPP SA2 specs as TEI19:

- Add description in TS 23.167, TS 23.221 and TS 23.501 for UE SMS domain selection

- Add general description in TS 23.501 (for 5GS), TS 23.272 (for SMS in MME)

- Add procedures/signalling flows in TS 23.272 (for SMS in MME) and TS 23.502 (for 5GS)

# 3. Proposal

It is proposed to agree the TEI19 WI in S2-2410073 and endorse the draft CRs as baseline for the work in S2-2410085.