SA WG2 Meeting #165	S2-2410118

14 - 18 October, 2024, Hyderabad, India

# Title:	On RAN LS on AIML data collection

# Source:	Nokia, NTT DOCOMO, MediaTek Inc., Verizon, InterDigital, BT, AT&T, T-Mobile USA, DISH Network

# Document for:	Endorsement

Agenda Item:	19.15.2

1	Introduction

SA2 received LS from RAN on AIML data collection in S2-2409600/RP-242389. In this LS, RAN informs that RAN2 has discussed the topic of UE side data collection for UE-side model training and analyzed and identified different options as documented in TR 38.843 (see also annex in this contribution).

These options are summarized as follows, considering changes agreed during RAN2#127 (in R2-2407807):

- Option 1a.		UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

- Option 1b.		UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

- Option 2.		UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

- Option 3.		UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

These options are all under consideration in RAN, and RAN is asking for inputs from SA2, SA3 and SA5 by RAN#106 (December 2024) to allow RAN to make a decision on how to progress regarding UE data collection for UE-side model training functionality.

SA2 is asked to provide feedback on options 1b, 2 and 3. RAN LS mentions the following aspects in particular:

“RAN2 could not reach consensus without input from SA groups on feasibility of MNO full controllability and full visibility (Note 5) for option 1b and couldn’t reach consensus on the feasibility of UP solution for option 2 and option 3 (Note 7).”

Also, the RAN LS provides requirements that solutions would need to meet:

RAN#105 way forward and requirements

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

Option 1a is transparent to 3GPP. Option 1a is not explored further in this contribution.

In option 3, OAM would manage the data collection procedures and interact with the gNB to configure UEs to collect data and transmit the collected data to the gNB, which would then forward the data to OAM. OAM would then transfer the training data to the server for data collection for UE-side model training/OTT server. RAN WGs considered two control plane (CP) and user plane (UP) tunnelling options for this purpose. The CP option, where the data is transmitted in the RRC layer (as it is mentioned by RAN WGs), could be realized using any of the existing data collection methods identified by RAN WGs and SA5. Similarly, our understanding is that a UP approach is feasible for option 3 but would require coordination with RAN WGs and/or SA5 depending on the termination endpoint of the UP tunnel. It is our understanding that this option 3 falls under SA5 purview. Should SA5 or RAN WGs analysis conclude that some SA2 involvement is needed, then SA2 could investigate this option 3.

From this incoming RAN LS, it is then our understanding that SA2 should prioritize reviewing option 1b and option 2 (plus some analyses on option 3 subject to SA5 feedback) in light of RAN questions and requirements for data collection for UE sided model training for standardized solution, i.e. on the feasibility of UP solution for option 2, and on level of controllability of the standardized data collection transfer process without the need of SLA for this purpose and on level of visibility for standardized data for options 1b and 2 and option 3.

SA2 is not expected to take decision on which option to pursue further, rather decision would need to be taken by RAN, as indicated in the LS.

2	Discussion

On UP solution for option 2

For option 2, RAN2 is considering a CP tunnel approach, provided that the data volume remains within the NAS signalling capacity. It is our understanding that a CP approach can easily be developed by SA2, such as relying on NAS protocol between the UE and AMF and SBI between the AMF and first termination entity.

However, RAN2 could not conclude whether a UP tunnel approach could be feasible.

It is our understanding that a UP solution could be developed by SA2 for option 2, should RAN decide that this is the way to go for UE data collection for UE side model training. The UP solution would address the step of collecting data from UE device towards an entity in the CN (i.e. first termination entity), while the exposure from CN to OTT server would be a different step.

Already, for AI based positioning use case, it is our understanding that one solution can already leverage LMF initiated User Plane Connection, which is already specified in TS 23.273 e.g. in clause 6.18. So LMF could be the central entity in control of the data collection procedures via CP procedures and receiving the collected data via UP after having provided its user plane information (i.e. IP address or FQDN) to the UE via a DL NAS TRANSPORT message of the AMF.

It is our understanding that relying only on a UP solution or only on a CP solution may not fit all use cases, because while some data collection might require larger data volumes, some data collection may be small enough that they can be provided by CP. It should also be understood that CP solution and a UP solution are not mutually exclusive and may actually co-exist. Configuration for data collection will be sent over the CP from an entity in the Core Network to RAN, while data collected by the UE can be sent over UP. Note that data can also be sent over CP for small amount of data, so a solution with CP for control of data collection process and collection of small amount of data, and UP for collection of large amount of data would also work.

The first termination entity in the Core Network (i.e. a new NF or enhanced existing NF), as per option 2, would need to be decided by SA2, and may depend on the use cases from RAN. Different flavours are possible for UP approach, depending on the termination point of data, but also depending on how control messages are exchanged (either via CP or UP). SA2 could look into detailed solutions if and when option 2 is selected by RAN.

Conclusion 1: A UP approach is feasible and can be developed by SA2 should RAN conclude on option 2. The CP and UP approaches are not mutually exclusive. The two approaches can coexist for the same use case, with configuration of data collection via CP, and data transfer either via CP or UP depending on the volume of data to be collected.

Note SA2 also does not see any issue with developing a UP solution for option 3 in coordination with SA5 should RAN decide that this is the way to go for UE data collection for UE side model training.

On controllability and visibility from MNO with option 1b

As indicated in RAN LS, the MNO needs to have full control of the standardized data collection transfer process and be able to manage data transfer to the server for UE-side data collection, “without the need of SLA for this purpose”. This includes initiating, terminating, and fully managing data transfer. The MNO also needs full visibility for standardized data.

Network awareness and control over the data collection process is basically required for efficiency and to ensure that enablement of UE data collection does not eliminate the gains that AI/ML features have as the potential to offer. In other words, it is our understanding that the Network is in the best position to determine data collection configurations which would match with future configurations for AI/ML-enabled features.

With option 1b, the first termination entity may be inside the MNO or outside the MNO domain, and it could be that the entity receiving the collected data does not belong to the MNO, which prevents any degree of controllability on the data collection process and any degree of visibility on the collected data unless an SLA exists between MNO and the owner of the OTT server.

An SLA between the MNO and the owner of the OTT server could provide a limited degree of control and visibility (e.g. via using APIs, Event IDs and Event Filters Supported by Data Collection AF). However, even with an SLA in place, option 1b  would not allow dynamic controllability on the data collection process, e.g. when to transmit the data, and adaptability to changing network conditions or changing data collection configurations.

A network may support multiple AI/ML related features and functions simultaneously. In case of solution 1b, the network cannot harmonize these simultaneously ongoing activities as they are triggered and controlled by the external OTT server, and the configuration is Application specific (i.e., per Application ID). The lack of harmonization may result in network performance issues or conflicting configurations. Moreover, in the case of end-to-end encryption or data pe-processing at Data Collection AF outside MNO domain, the MNO has no visibility or limited visibility on the data that is transmitted from the UE to the server.

Conclusion 2: Option 1b does not allow full controllability on the data collection process nor full visibility on the collected data.

On controllability and visibility from MNO with option 2 (and option 3)

On the contrary, option 2, which has a first termination entity in the CN, allows visibility on the collected data at least on the CN side. Controllability can also be achieved e.g. via CP signalling between the entity in the CN and the UE (via RAN) or between the entity in the CN and the RAN. The data collection path between the UE and the first termination entity in the CN can be via the user plane or via the control plane.

The exposure of the collected data from the first termination entity can be achieved via NEF APIs available to the OTT server. The authorization of the NEF API invocations would be based on SLA. However the SLA described here would only be an SLA used for exposing the collected data, not for the step of collecting the data.

It should be noted that, with option 3, the data path is UE  gNB  OAM. Given that gNB and OAM are in MNO domain, full visibility of standardized data is also achieved.

Conclusion 3: Option 2 and option 3 allow full controllability on the data collection process and full visibility on the collected data.

3	Proposals

Based on the following conclusions:

Conclusion 1: A UP approach is feasible and can be developed by SA2 should RAN conclude on option 2. The CP and UP approaches are not mutually exclusive. The two approaches can coexist for the same use case, with configuration of data collection via CP, and data transfer either via CP or UP depending on the volume of data to be collected.

Note SA2 also does not see any issue with developing a UP solution for option 3 in coordination with SA5 should RAN decide that this is the way to go for UE data collection for UE side model training.

Conclusion 2: Option 1b does not allow full controllability on the data collection process nor full visibility on the collected data.

Conclusion 3: Option 2 and option 3 allow full controllability on the data collection process and full visibility on the collected data.

It is proposed to answer to RAN LS indicating that:

- SA2 sees no issue with developing a UP solution for option 2 and in coordination with SA5 for option 3 should RAN decide that this is the way to go for UE data collection for UE side model training.

- SA2 understanding is that option 2 and option 3 allow full controllability on data collection process and full visibility on the collected data, and this without the need for any related SLA. On the contrary, SA2 believes that option 1b cannot provide MNO with full control on data collection process nor visibility on the collected data.

Annex

##### 7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc. The result of this analysis can be found below in Table 7.2.1.3.2-1.

It is worth noting that different options for the data content visibility were discussed. The different levels of data content visibility are captured in the Note 3 in the Table 7.2.1.3.2-1.

Note:	RAN2 discussed that, except for the case of standardized data content, the data content visibility and any level of controllability could be achieved via SLA (Service Level Agreement). However, SLA is out of RAN2 scope.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | FFS: level of controllability (Note 5) (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure, FFS other procedures | Via RRC procedure |
| Possible Options for data content visibility in MNO (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. UE proprietary information can only be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3 |  |  |  |  |

Related to privacy, it has been stressed in RAN2 the importance that any potential mechanism to collect UE side data for model training purposes (including the options 1a, 1b, 2, 3 listed above) must comply with privacy protection regulations, requirements, laws and/or policies.