3GPP TSG-SA2 Meeting #165	S2-2410120

# Hyderabad, India, 14 – 18th  October 2024	  			       		          (revision of S2-240)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5723 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage of UDP-Connect for Handling of end-to-end encrypted XR flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA plenary approved conclusions in 23.700-70 clause 8.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a high level description of The UDP-Connect mechanism defined to support XRM metadata detection by UPF when XR traffic is encrypted between UE and AS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | conclusions in 23.700-70 clause 8.2 are not supported by normative work |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR should be implemented after CR 5722 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE ALL text is new

#### 5.37.X.2 Usage of Proxying UDP in HTTP in order to handle end-to-end encrypted XR flows

##### 5.37.X.2.1  Baseline usage of Proxying UDP in HTTP to deliver XRM metadata to the UPF

Proxying UDP in HTTP per RFC 9298 [170] may be used to support secured delivery to the PSA UPF of XRM metadata from an AS sending XRM data towards a UE. It is referred as usage of UDP-Connect in this clause. When Baseline usage of UDP-Connect to deliver XRM metadata to the UPF applies, XRM metadata are included in HTTP datagrams.

The mechanisms described in this clause apply when the AS is collocated with the UDP Proxy defined in RFC 9298 [170].

NOTE 1:	This does not preclude other potential deployments where there is cascading of UDP proxies but these potential deployments are not addressed in this specification.

A UDP tunnel using Proxying UDP in HTTP per RFC 9298 and aiming at carrying DL XRM metadata.may be established between the UPF and the AS as follows:

-	the AF may provide via the NEF an indication of support of UDP-Connect to deliver XRM metadata for encrypted traffic together with the corresponding AS address and the corresponding QoS requirements. This is done using the AF session with QoS procedure (as defined in clause 4.15.6.6 of TS 23.502 [3]), and meant to support the establishment of a UDP tunnel with the connect-UDP upgrade token between the UPF and the AS.

-	The PCF generates a PCC rule including corresponding information.

-	The SMF provides N4 rules to the UPF including the corresponding information.

-	In the uplink direction, PDR and FAR rules are used to detect (PDR) XR flow subject to Usage of UDP-Connect and (FAR) to associate these flows with UDP-connect based proxying.

-	In the downlink direction, N4 rules are used to detect (PDR) XR flows and to associate them with proper QoS (QER) possibly including PDU set related marking (as defined in clause 5.37.5), expedited DL data (as defined in clause 5.37.y).

-	The UPF supports the HTTP/3 Client functionality, which establishes a UDP tunnel to the AS (where HTTP/3 Proxy operates) with the connect-UDP upgrade token at the latest when the UPF detects the start of a new UE flow towards the AS supporting UDP-connect based proxying for XRM metadata delivery.

NOTE 2:	When many UEs served by the same UPF have an encrypted XRM connection towards the same XRM AS, the UPF can determine to use a common single UDP-Connect tunnel with that AS. The UPF needs nevertheless to ensure it can De-multiplex DL traffic from the AS to be able to route this DL traffic towards the PDU Session (GTP-u tunnel) of the correct UE. For this purpose, the UPF can for example associate each UE’s PDU Session with a corresponding QUIC stream.

-	the UPF shall use context id =0 for uplink HTTP datagram payloads.  The UPF determines a non-zero context ID for downlink HTTP datagram payload that during the tunnel creation, the UPF shall register with AS and associate with the format to be used for Downlink HTTP datagram: when requesting an HTTP CONNECT for connect-udp, the UPF indicates the HTTP datagram payload format and the XRM metadata delivery format that it supports. The Proxy in the AS answers with the HTTP datagram payload format and the XRM metadata delivery format to be associated with the non-zero context ID for downlink HTTP datagram.

Downlink HTTP datagram payload contains a context ID, XRM metadata length, XRM metadata, and XRM data. The HTTP datagram payload format indicates how this information is encoded. The XRM metadata delivery format indicates how the XRM metadata are encoded

NOTE 3:   The HTTP datagram payload as well as the XRM metadata delivery format will be further defined by CT groups.

NOTE 4: If there is no more traffic using an established UDP-tunnel, the UPF can use implementation mechanisms such as idle timer to close a UDP-Connect stream or a UDP-Connect association.

When UDP tunnel using UDP-Connect has been established PDU(s) are handled as follows:

-	Downlink PDU(s) received by UPF on N6 interface contain encrypted E2E XRM media along with XRM metadata. XRM metadata is encrypted between UPF and AS

-	SMF instructs UPF on what packet should be captured using PDR rules and what action to be made on the detected packeted using Forwarded Action Rule.

-	For DL traffic, UPF identifies the XRM metadata from the HTTP datagrams, and uses these XRM metadata to ensure relevant Qos is delivered to the PDU; This may include marking the PDU Set Information into GTP-U header as described in clause 5.37.5.2. When UPF receives DL packets over the UDP tunnel from AS, the UPF shall send over the right PDU session towards UE. The UPF shall remove the context ID, XRM metadata length, and XRM metadata in the packet sent in the GTP-u tunnel towards the UE.

-	When UE sends UL packets carrying XRM data that is encrypted between UE and AS, the UPF sends those packets over the the UDP tunnel.

XRM metadata may carry:

-	PDU set Information as defined in clause 5.37.5.2 ,

-	End of data burst as defined in clause 5.37.8.3,

-	information defined in in clause 5.37.y such as Source (SSRC) and Payload Type (PT) and used to detect multiplexed flows.

-   	Burst size as defined in clause 5.37.z;

-	Data boosting information triggered by the AS as defined in clause 5.37.w;

##### 5.37.X.2.2  Usage of Proxying UDP in HTTP coupled with QUIC-Aware-proxying

Editor’s Note: the whole content of this clause is FFS and depends on IETF feedback

When Proxying UDP in HTTP is used for XRM traffic using QUIC, double encryption of XRM traffic (once via QUIC between UE and AS and another time in Proxying UDP in HTTP related encapsulation / tunnel between UPF and AS) D ouble encryption of XRM traffic may be avoided using QUIC aware proxying with HTTP defined in [xx2]. Clause 5.37.X.2.2 defines the usage of Proxying UDP in HTTP coupled with QUIC-Aware-proxying

By default, UPF and AS use tunnelling mode to exchange XRM traffic. The UPF and AS may use forwarded mode as defined in [xx3] when the XR flow is based on QUIC, they support this feature,  and they have negotiated Virtual ConnectionID using RFC 9297 [172] as defined in  QUIC aware proxying with HTTP [xx2].

During the QUIC connection establishment between the UE and the AS, the UPF may learn the QUIC Connection IDs that are used in the flow from UE to AS and those used in the flow from AS to UE. Based on the learnt Connection ID(s), the UPF may negotiate with AS per [xx3] to get Virtual ConnectionIDs (CID(s)). These virtual CIDs may be used by UPFand AS to support the forwarded mode.

When forwarded mode is active and possible for UL direction, UPF sends UL QUIC packet over UPF-AS UDP 4-tuples with the Virtual ConnectionIDs negotiated via QUIC aware proxying with HTTP [xx3]; such UL traffic contains only the UDP payload received from the UE.

When forwarded mode is active and possible for DL direction, XRM metadata is included by the AS into the forwarded UDP payload packets via a packet transform defined by 3GPP. The UPF uses a 3GPP defined packet transformer [clause 5.3 of xx3] to retrieve the XRM metadata and the content of the original end-to-end QUIC packet.

During the negotiation between UPF and AS to activate the forwarded mode, UPF and AS need to agree on the 3GPP packet transformer to apply in the DL direction.

NOTE 1: Since the connect-UDP client (in the UPF) is separate from the end-to-end QUIC client (in the UE) it cannot coordinate as tightly with the client QUIC stack as is assumed by the “QUIC-Aware Proxying Using HTTP” draft [xx2]. The connect-UDP client in the UPF can extract CIDs from e2e QUIC Initial packets and uses these values in REGISTER_CLIENT_CID and REGISTER_TARGET_CID capsules defined in [xx2]. Prior to forwarding QUIC packets with short header, the connect-UDP client (UL) and the proxy (DL) check whether the destination CID is identical to an already registered CIDs, if not, the packets are sent in tunneled-mode.

NOTE 2: The 3GPP packet transform is defined in stage-3 specifications.

NEXT CHANGE (2)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES