3GPP TSG-WG SA2 Meeting #165	S2-2410125

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1476 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional descriptions of UE-satellite-UE communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of features to support UE-satellite-UE communications for satellite communications based on the conclusions for KI#3 in TR 23.700-29. - activation of UE-SAT-UE: To support UE-SAT-UE communication, the P-CSCF subscribes to PCF services to obtain the satellite ID and access details related to the UE. Standard call setup procedures are followed, and the use of AGWs for transport traversal is optional. The satellite ID is included in SIP messages between P-CSCFs to prepare for communication. Activation is triggered by the originating P-CSCF, which verifies connectivity, selects an AGW, and updates the terminating P-CSCF with the new transport address and satellite ID. Once both sides confirm connectivity, UE-SAT-UE is activated, and the P-CSCFs notify the PCF of the status. - mobility support of UE-SAT-UE communication: To ensure service continuity when a UE's serving gNB satellite changes during UE-SAT-UE communication (due to UE or satellite movement), the P-CSCF on the satellite change side must be notified of the new satellite ID, access information, and any user plane fallback indication. If no fallback is indicated and satellite connectivity is confirmed, the P-CSCF selects an AGW on the new satellite, reserves a transport address, and updates the other P-CSCF via SIP RE-INVITE. Both sides confirm connectivity, and the session continues. If a fallback indication is received or satellite connectivity can't be maintained, the P-CSCF selects an AGW on the ground and updates the transport address to switch the communication back to ground-based handling. - AGW reallocation: AGW re-allocation in UE-SAT-UE communication involves selecting a new AGW for NAT traversal during call setup or an active session, while terminating the old AGW. The P-CSCF reserves new transport addresses, updates the SDP offer, and configures the media flow directions (T1 for incoming and T2 for outgoing). Once both sides confirm the new addresses, the old AGW is released. During call setup, SIP UPDATE/200 OK is used to exchange updated transport addresses, while during an active session, SIP RE-INVITE/200 OK is used if the serving satellite changes, requiring re-negotiation of transport addresses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 3 new clauses are added. - activation of UE-SAT-UE - mobility support of UE-SAT-UE communication - AGW reallocation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-SAT-UE communication cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AY.3(new), AY.4(new), AY.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AY.3 Activation of UE-satellite-UE communication

To support UE-SAT-UE communication, the P-CSCF should subscribe to PCF services to obtain the satellite ID and access information related to the UE. The satellite ID identifies the satellite that hosts at least one gNB.

Prior to activation, standard call setup procedures (refer to clause 5.6.2 for originating, and clause 5.7.2 for terminating) are followed. The use of AGWs (on the ground) for transport address traversal is optional (see clause G.4.3) and depends on the operator's deployment. The serving satellite ID of the AGW should be included in the SIP messages exchanged between the P-CSCF of the originating and terminating networks to prepare for UE-SAT-UE communication.

The activation of UE-SAT-UE communication is determined separately at the originating side and the terminating side, but is triggered at the originating side:

- At the originating side: The P-CSCF verifies the connectivity status of the serving satellite(s) between the originating and terminating UEs. Once connectivity is confirmed, the P-CSCF activates UE-SAT-UE communication by selecting an AGW on a satellite serving the UE and reserving a new transport address with the chosen AGW. The P-CSCF then updates the terminating side's P-CSCF with the new transport address and satellite ID via a SIP UPDATE message.

- At the terminating side: If the satellite ID received matches the one obtained during the standard call setup, and the updated transport address is for the same UE, the P-CSCF checks that connectivity between the serving satellite(s) and both UEs. Once the connectivity is ensured, it activates UE-SAT-UE communication by selecting an AGW that serves the UE and reserving a new transport address with this AGW. The P-CSCF updates the originating side's P-CSCF with the new transport address via a SIP 200 OK (UPDATE) message.

Once the UE-SAT-UE is determined to be activated, the P-CSCFs at both the originating side and the terminating side should notify the PCF about the status of UE-SAT-UE communication, i.e. by including a U-S-U indication and the new transport address in the notification back to the PCF. Based on this indication, the 5GC will insert a UL CL/L-PSA into the satellite that is serving UE and update the UL CL filters.

# * * * * Third change * * * *

## AY.4 Mobility support of UE-satellite-UE communication

Editor’s Note:	the following mechanism assumes gNB, UL CL/L-PSA, IMS-AGW onboarding the same satellite, how to adjust it to apply to other deployments is FFS.

To support service continuity when one of the UE’s serving gNB satellite in the UE-SAT-UE communication changes (e.g. UE’s movement, or satellite’s movement), the P-CSCF in the satellite change network side needs to be notified of the new satellite ID and access information related to the UE, and in addition a user plane fallback indication if applicable.

The continuation of the UE-SAT-UE communication decision is triggered at the satellite change network side.

-	At the satellite change network side: If there is no user plane fallback indication received (i.e. the user plane is still kept on a satellite), and the connectivity status of the serving satellite(s) between the originating and terminating UEs is confirmed, the P-CSCF will decide to continue UE-SAT-UE communication by selecting an AGW on the target satellite that is now serving the UE and reserving a new transport address with the chosen AGW. The P-CSCF then triggers to update the new transport address and the target satellite ID to the other side's P-CSCF via a SIP RE-INVITE message.

-	At the other network side: When the P-CSCF receives a RE-INVITE message that contains a different satellite ID and a different transport address, it means that a serving satellite changes and the UE-SAT-UE continues at the satellite change network side. The P-CSCF verifies the connectivity status of the serving satellite(s) between the originating and terminating UEs. Once connectivity is confirmed, the P-CSCF decides to continue UE-SAT-UE communication by updating the new transport address with the AGW that is serving the UE. The P-CSCF then updates the new transport address and the satellite ID to the satellite change networking side's P-CSCF via SIP 200 OK (RE-INVITE) message.

-	Once the UE-SAT-UE is determined to be activated, the P-CSCF at the satellite change network side should notify the PCF about the status of UE-SAT-UE communication, i.e. by including a U-S-U indication and the new updated transport address in the notification back to the PCF. Based on this indication and the transport address, the 5GC will change UL CL/L-PSA the satellite that is serving UE and update the UL CL filters.

The termination of UE-SAT-UE communication decision can be triggered at 5GC, satellite change side or the other network side. If

- At satellite change networking side: If a user plane fallback indication is received (i.e. the user plane is fallback to the ground) from 5GC, or no satellite ID included in a SIP 200 0K (RE-INVITE) message, the P-CSCF will terminate the UE-SAT-UE communication by selecting an AGW on the ground and reserving a new transport address with the chosen AGW. The P-CSCF then triggers to update the other side's P-CSCF with the new transport address via a SIP RE-INVITE message.

- At the other side: if there is no satellite ID included in the SIP RE-INVITE message (i.e. implies the UE-SAT-UE terminates at the satellite change network side), or the connectivity status of the serving satellite(s) between the originating and terminating UEs cannot be maintained, the P-CSCF will terminate UE-SAT-UE communication by selecting a AGW on the ground and updating the new transport address with the AGW. The P-CSCF then updates the satellite change networking side's P-CSCF with the new transport address via a SIP 200 (RE-INVITE) message.

# * * * * Fourth change * * * *

## AY.5 AGW reallocation

AGW re-allocation means during call setup, or a call session, a new AGW will be selected for NAT traversal and the old AGW will be terminated (if there is one). It is only applied to UE-SAT-UE communication.

Once a new AGW is selected, the P-CSCF will reserve a new transport address (outgoing) with the new AGW, and modify the SDP offer with the new transport address. After receiving an SDP answer with the transport address of the other side, the P-CSCF will configure the outgoing termination T2, reserve a new transport address (incoming), and configure the incoming termination T1. The P-CSCF will also modify the SDP answer with the newly reserved transport address.

NOTE 1: 	outgoing termination T2 and incoming termination T1 describe the direction of call media flow as outlined in clause 6.2.1 in TS 23.334.

Once the incoming termination T1 and outgoing termination T2 are updated, the session with any previous AGW will be terminated, and the resources will be released

In case of activation of UE-SAT-UE communication, both the originating side and the terminating side need to select new AGWs on the satellite. As it is during the call setup period, address negotiation is undergoing, there is no need to trigger the address re-negotiation. The SIP UPDATE/200 OK (UPDATE) can be used between P-CSCF on the originating side and P-CSCF on the terminating side to exchange the updated transport addresses reserved with the newly selected AGWs.

In case when one of the UE’s serving gNB-onboarded satellite in the UE-SAT-UE communication changes, a new AGW on the satellite needs to be selected if UE-SAT-UE is to be continued, new AGWs on the ground need to be selected on both the originating side and the terminating side if UE-SAT-UE is to be terminated. As it is during the call session, transport address re-negotiation is needed. The SIP RE-INVITE/200 OK (RE-INVITE) can be used to exchange the updated transport addresses reserved with the newly selected AGW(s).

# * * * * End of change * * * *