3GPP TSG-SA2 Meeting #165	S2-2410128

# 14 – 18 October, 2024, Hyderabad, IN											revision of S2-2407755

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1133 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of VFL model training with successive VFL client information sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusions for VFL training documented in clause 8.2 of TR 23.700-84, including: P#2.4.5: VFL clients may also provide intermediate results (e.g. gradient information, loss information) to other VFL clients as instructed by the VFL server. During VFL process, both VFL model training and inference, the VFL server and clients will coordinate to perform local computation, e.g. forward computation and backwards propagation for VFL model training. Coordination between VFL clients to share intermediate results are supported by multiple VFL algorithms, e.g. split VFL. Rather than only deploying the VFL server to aggregate all the intermediate results from a huge number of VFL clients, the intermediate results shared between VFL clients can reduce the computational load of the server and also improve the VFL computation efficiency. This CR specifies the procedures for VFL model training and inference with successive VFL client information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce procedure for VFL model training and inference with successive VFL client information sharing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL cannot be fully supported by 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.2.1, New 6.2G.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes  * * *

#### 6.2G.2.1	General inference procedure for vertical federated learning

Figure 6.2G.2.1-1: General inference procedure for vertical federated learning

Editor's note	Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable.

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	For an NWDAF as a VFL server, the analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF cannot generate the analytics output, the NWDAF containing AnLF sends a subscription request to VFL server NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs.

Editor's note:	Which service used between the consumer and the VFL server is FFS.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients.

Editor's note:	How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

If the VFL server is the NWDAF and the VFL client is the NWDAF, VFL server sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

If the VFL server is the NWDAF and the VFL client is the AF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

Editor's note:	Whether the VFL model correlation ID is needed or using the model ID is FFS.

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

Editor's note:	Additional parameters in the request are FFS, e.g. UE ID(s) which will need to be sent to VFL clients.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

4a.	Optional, the VFL Client may share intermediate local inference with another VFL Client, if configured by the VFL server.

4b.	Optional, the same step 3 and step 4. Based on the received VFL inference request or subscription, the VFL client perform VFL inference to generate the intermediate local inference results. The VFL client may perform data collection, if its local data is not sufficient.

4c. Optional, the same as 4a.

Editor's note:	Whether the VFL client needs to retrieval of stored local models is FFS.

5.	Each VFL Client sends the intermediate local inference results to the VFL server.

Editor's note:	Whether the VFL model correlation is required in step 5 is FFS.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to aggregates and generates the VFL inference results.

6.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL model correlation ID.

Editor's note:	Further details of the VFL server computiong the VFL inference is FFS.

7.	If the NWDAF is the VFL server, the VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

* * *Next Change (new text) * * *

#### 6.2G.2.x	General model training procedure for vertical federated learning with successive VFL client information sharing

The procedure in Figure 6.2x.2-1 to support VFL model training with AF or NWDAF acting as the VFL server and successive VFL client information sharing is described step by step below.

Figure 6.2x.2-1: Procedure for VFL training with successive VFL client information sharing

0.	VFL server (i.e. AF or NWDAF) and VFL clients (i.e. NWDAF) register to NRF. The registration may include their NF profiles, Analytics ID(s), Address information of NWDAF, Service Area, VFL capability type information (i.e. VFL server and VFL client type), VFL client coordination capability, VFL computational capability (i.e. acting as VFL server and/or VFL client ), and Time interval supporting VFL. The latter parameter can be the same as Time interval supporting FL described in clause 5.2.

The VFL server and clients are discovered via NRF by invoking the Nnrf_NFDiscovery_Request service operation. During the discovery, the VFL server may include the requirements on the VFL clients and server in the discovery request, e.g. the location of the VFL clients and server, the (minimum) available capacity of the VFL clients and server, VFL client coordination capability.

NOTE 1:	The initial selection of VFL clients by the VFL server may happen in step 0. The selection of VFL clients by the VFL server may also be finalized in step 2.

NOTE 2:	The details of sample and/or feature alignment are out of scope of this procedure.

1.	The VFL server determines to initiate the VFL model training based on its internal logic and sends a VFL preparation request to the VFL client NWDAF(s) (via NEF if the VFL server AF is untrusted AF).

If the VFL server is trusted AF or NWDAF, the VFL server invokes Nnwdaf_MLModelTrainingInfo_Request or Nnwdaf_MLModelTraining_Subscribe service operation towards the VFL client NWDAF(s).

For untrusted VFL server AF, the AF invokes new service operation Nnef_MLModelTrainingInfo_Request or Nnef_MLModelTraining_Subscribe request towards the NEF. Then the NEF forwards the model preparation request to the corresponding VFL client NWDAF(s) by invoking Nnwdaf_MLModelTrainingInfo_Request or Nnwdaf_MLModelTraining_Subscribe service operation.

Editor's note:	The details of the new NEF service operations, including Nnef_MLModelTrainingInfo_Request, Nnef_MLModelTraining_Subscribe, Nnef_MLModelTraining_Notify, Nnef_MLModelTraining_Unsubscribe, etc., are FFS.

The VFL server may include ML Preparation Flag to check if the VFL clients can meet the ML model training requirements in the VFL preparation request. The VFL server may also include Analytics ID and VFL process ID, the target samples (e.g. UE ID, application ID, etc.), optional target features, ML Model Interoperability information, Available data requirement, Availability time requirement, required NWDAF capacity for the VFL process etc.

The VFL clients may respond to the VFL server by indicating whether they will join the VFL operation and may include the reason in the response message if it cannot join the VFL operation, e.g. due to no sufficient  capacity to perform VFL model training, not able to coordinate with other VFL client NWDAF(s), etc.

2.	The VFL server may performed VFL client selection or refinement based on the responses received by from the VFL clients.

3.	The VFL requests the VFL clients to start the VFL Model training process where intermediate training results are shared and coordinated by the VFL server, facilitating a collaborative approach to model refinement across the VFL clients.

The VFL server may trigger the VFL training by invoking a ML Model Training request towards VFL client #1 (NWDAF) (via NEF if the AF is untrusted).

3a.	For untrusted VFL server AF, the AF invokes new service operation Nnef_MLModelTraining_Subscribe request or Nnef_MLModelTrainingInfo_Request towards the NEF for VFL training request. Then the NEF forwards the received VEL training request to the selected VFL client NWDAF(s) by invoking Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service operation.

3b.	If the VFL server is trusted AF or NWDAF, the VFL server invokes Nnwdaf_MLModelTraining_Subscribe  request or Nnwdaf_MLModelTrainingInfo_Request towards the selected VFL client NWDAF(s).

The VFL server may indicate the Analytics ID and a VFL process ID associated to the VFL training, target samples (e.g. UE ID, application ID, etc.), optional target features, ML Model Interoperability information, Available data requirement, Availability time requirement, the VFL clients that participate the VFL process, the order of VFL clients for performing local model computation to the VFL clients.

Editor's note:	The complete list of parameters to support the successive VFL client information sharing is FFS.

4a.	VFL client #1 (NWDAF) triggers the local model computation to calculate the intermediate results. The VFL client may collect data for model training if the local data is not sufficient.

4b.	VFL client#1 notifies the intermediate results to the next VFL client #2 based on the order configured by the VFL server , by invoking the Nnwdaf_MLModelTrainingInfo_Request or Nnwdaf_MLModelTraining_Subscribe service operation.

VFL client#1 may indicate the remaining VFL clients that participate the VFL training process, the order of the remaining VFL clients for performing local model computation and other information as detailed in Step 3 to VFL client #2 (NWDAF).

Alternatively, the full list of the VFL clients that participate the VFL training process and the order of the VFL clients to perform VFL training might be indicated to each client by the server, e.g. in step 1 or in parallel with step 3, And. In this case, each VFL client can work out the next VFL client to interact with based on the VFL client list, the order, and the ID of the previous VFL client.

4c – 4e. Repeat Step 4a and 4b until the VFL training procedure propagates to the last VFL client based on the order configured by the VFL server.

5.	The VFL client #N NWDAF notifies the intermediate results to the VFL server.

5a.	If the VFL client is untrusted AF, the VFL client #N NWDAF notifies the intermediate results to the VFL server via NEF by invoking new service operation Nnef_MLModelTrainingInfo_Request response or Nnef_MLModelTraining_Notify based on the service operation used in Step 3. The NEF forwards the intermediate results to the VFL server by invoking Nnwdaf_MLModelTrainingInfo_Request response or Nnwdaf_MLModelTraining_Notify service operation based on the service operation received from VFL client.

5b.	If the VFL client is trusted AF or NWDAF, the VFL client #N invokes Nnwdaf_MLModelTrainingInfo_Request response or Nnwdaf_MLModelTraining_Notify based on the service operation used in Step 3.

6.	The VFL server performs further VFL computation and aggregation for local model (e.g. calculates the loss using the labels) by using the intermediate results received from VFL client #N. The VFL server updates its local model.

7.	In the subsequent iteration, the VFL server triggers the backwards propagation VFL computation for model update and refinement in each successive VFL clients, in the reversed order of VFL forward computation in step 4a-4e. VFL server sends VFL model refinement request to the last client in the previous iteration, e.g. VFL client #N.

7a.	 If the VFL client is untrusted AF, the VFL server invokes new service operation Nnef_MLModelTraining_Notify towards NEF. Then the NEF forwards the request to the VFL client by invoking Nnwdaf_MLModelTraining_Notify.

7b.	If the VFL client is trusted AF or NWDAF, the VFL server invokes Nnwdaf_MLModelTraining_Notify service operation toward VFL client #N.

The VFL server may include the intermediate results (e.g. the loss which will be used by the VFL client to compute gradient for model refinement and update), and optionally the VFL clients that participate in the VFL training process and the order of VFL clients for performing local computation for model refinement to the VFL client #N.

NOTE 3:	the order of VFL clients to perform model refinement in step 8a-8e might be reserved order of forward computation in step 4a-4e. The VFL clients that participate in the forward computation and backward propagation are the same. In this case, the VFL clients may derive the order of the backward propagation based on the ID of the previous VFL client.

8a.	VFL client #N performs VFL local computation to refine the local model based on the intermediate results provided by the VFL server.

8b.	VFL client #N deliveries the model refinement results to the subsequent VFL client based on the order configured by the VFL server or derived by itself by invoking Nnwdaf_MLModelTraining_Notify.

8c. – 8e. Repeat Step 8a and 8b until the VFL local computation for model refinement procedure propagates to the VFL client which is the first VFL client to perform the forward computation in step 4a, e.g. VFL client #1.

9.	VFL client #1 notifies the model refinement results to the VFL server.

9a.	 If the VFL client is untrusted AF, the VFL client #1 NWDAF notifies the intermediate results to the VFL server via NEF by invoking new service operation Nnef_MLModelTraining_Notify. The NEF forwards the model refinement response to the VFL server by invoking Nnwdaf_MLModelTraining_Notify service operation.

9b.	If the VFL client is trusted AF or NWDAF, VFL client #1 invokes Nnwdaf_MLModelTraining_Notify.

10.	The VFL server performs further VFL computation and model to update local model by using the intermediate results received from VFL client #1.

Steps 3-10 will be repeated until the VFL server determines to terminate the VFL model training.

11.	The VFL server determines to terminate the VFL model training process based on its internal logic, e.g. if the model is converged.

If the VFL server is untrusted AF, the VFL server invokes new service operation Nnef_MLModelTraining_Unsubscribe service operation towards NEF, and then the NEF forwards the request to the corresponding VFL client(s) by invoking Nnwdaf_MLModelTraining_Unsubscribe service operation.

If the VFL server is trusted AF or NWDAF , the VFL server sends VFL model training termination request by invoking Nnwdaf_MLModelTraining_Unsubscribe service operation toward NWDAF.

Step 11 may also happen after Step 6. In this case, Step 7-10 will be skipped.

Editor's note:	The VFL inference procedure with successive VFL client information sharing is FFS.

* * *End of Changes * * *