3GPP TSG-SA WG2#165	S2-2410137

Hyderabad, India, 14-18 October 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	Discussion solution options on AIML data collection

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	NR_AIML_Air / Rel-19

Abstract of the contribution: This contribution discusses the feasibility of different options listed in RAN table to support UE data collection for UE-side model training.

# 1.	Discussion

## 1.1	RAN status

The LS RP-242389 corresponds to the UE side data collection discussion status sent from RAN plenary to SA2 and other SA WGs. Four different options and related analysis are captured in Table 7.2.1.3.2-1 in TR 38.843 and copied in the LS from RAN#105. RAN2 could not reach consensus without input from SA groups for several aspects and several FFS’ are remained in the table.

RAN#105 further discussed the above-mentioned table and agreed some way forward and requirements and all the standard solutions for data collection need to meet the requirements.

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

## 1.2	SA2 analysis for different options

### 1.2.1	Option 1b

For option 1b, the first termination entity is a data collection server for UE-side model training. The data transfer path between the UE and the data collection server via  a UP path. The description of option 1b in Table 7.2.1.3.2-1 is very close to the Data collection from the UE Application in clause 6.2.8 in TS 23.288. SA2 defined this procedure in Rel-17 to support data collection from the UE application to the Data Collection Application Function (DCAF) which is a data collection server terminated in the 5GC  via UP path. The DCAF can further send the collected UE data to NWDAF or AF.

The Data collection from the UE Application solution is already used for several analytics that defined in TS 23.288. For example:

Table 6.4.2-1 in TS 23.288: input data for Observed Service Experience related network data analytics

| QoE metrics | UE (via AF) | QoE metrics observed at the UE(s). QoE metrics and measurement as described in TS 26.114 [27], TS 26.247 [28], TS 26.118 [29], TS 26.346 [30], TS 26.512 [31] or ASP specific QoE metrics in TS 26.512 [31], as agreed in the SLA with the MNO, may be used. |
| --- | --- | --- |

In the example above, the QoE metrics can be a standard QoE metrics and a measurement for some specific services, such as Multimedia Telephony Services for IMS (MTSI), streaming service, XR service, MBMS that are defined in SA4. It is also supported to transfer some Application Service Provider (ASP) specific QoE metrices that not defined in 3GPP standard by using a container.

In addition, it is supported to collect some speed and location destination related information from the UE application as defined in table 6.5.2-4 for NF load analytics and proximity related input data as defined in Table 6.19.2-2 for Relative Proximity Analytics.

However, if we want to reuse the current UE data collection procedure defined in clause 6.2.8 in TS 23.288 to match option 1b defined by RAN2, there are several aspects that need further enhancements to fulfil the requirements agreed in RAN#105:

- The data collected is secured and data integrity and confidentiality for that data is ensured.

>> Already supported in clause 6.2.8 in TS 23.288 and similar mechanisms can be reused for option 1b.

- User data  privacy, anonymity and user consent is respected

>> Already supported in clause 6.2.8 in TS 23.288 and similar mechanisms can be reused for option 1b.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer.

>>	The current SA2 supported procedure was designed to collect some application layer data from UE side: in order to avoid the risk of user privacy violation from UE side, the allowed collected data is restricted by the SLA between the Service provider and the operator , the service provider configures the allowed collected data in both DCAF and UE application client, DCAF and UE application client will further check if the requested UE application layer data is allowed or not when the data collection request is received.

Although the allowed collected data is controlled based on SLA, the initiation and termination of data transfer are fully controlled by the operators for the existing SA2 defined procedure in TS 23.288.

On the contrary, the currently discussed collected input data for UE side model training in NR_AIML_Air are irrelevant to the UE application layer data for an application provider: all the collected data that used for UE side model training are radio interface layer measurement parameters or radio configurations that defined in RAN1 / RAN2, the privacy issue about UE data collection for UE side model training in NG_AIML_Air can be achieved based on user consent check and there is no need for the SLA configuration about the allowed collected data. The corresponding procedures mentioned in clause 6.2.8 in TS 23.288 can be easily skipped, and some further enhancement is required to fulfil the requirement of MNO full controllability.

- MNO has full visibility for standardized data.

Data collection from the UE Application in clause 6.2.8 in TS 23.288 was designed as a general solution to support data collection for all the applications, it is not only applicable for the 3GPP defined services (e.g., MBMS, MTSI, XR,. etc), but it also applicable for any 3rd party provided services. Therefore, it can be supported to transfer the 3GPP defined parameters as defined Table 6.4.2-1 in TS 23.288, and also provide a mechanism to transfer some other non-3GPP-standardized parameters. Whether the non-3GPP-standardized parameters are transferred or not purely depends on the request from the data consumer (e.g., NWDAF).

Considering the three use cases defined in NR_AIML_Air, if all the input data used for the UE side model training were defined in the standard specs based on RAN1/RAN2’s conclusion, there would be no need to support the feature to transfer the non-3GPP-standardized parameters and the corresponding container IEs could be saved in the UP signalling.

One example of option 1b based on Data collection from the UE Application defined in clause 6.2.8 in TS 23.288 is described as below.

Figure 1.2.1-1 UE data collection for option 1b

The DCAF is controlled by operator to support the UE data collection for UE-side model training.

- UE establishes a user plane connection to DCAF. The DCAF address can be provided to UE via different way. For example, UE is local configured about the DCAF address for each operator, PCF may provide the DCAF address in URSP rule, or 5GC may provide the DCAF address to UE via some other control plane procedure.

- Data Consumer (UE-side ML model training server), which can be deployed either in or out of operator’s domain. If the Data Consumer is out of operator’s domain, the UE Data Collection request is sent to NEF and then to DCAF.

The feature information (i.e., beam forming, CSI) is included in the UE data collection request message from  Data Consumer  to DCAF.

DCAF is configured a mapping list about the standardized collected UE parameters for each feature. The mapping list is standardized based on RAN1/RAN2’s conclusion and configured by operator. DCAF requests the UE data collection with the parameters for the requested feature based on the configured mapping list.

DCAF can also perform user consent check with UDM to check if UE data collection is allowed or not.

- The data collection request and response between DCAF and UE are performed via user plane connection. The protocol between DCAF and UE can be also standardized in 3GPP.

This is just one example to show how to support the MNO full controllability and visibility for option 1b, we are open to discuss some other solutions to support RAN’s requirement, the more detail should be further discussed in SA2.

Observation 1: It is possible to reuse the Data collection from the UE Application defined in clause 6.2.8 in TS 23.288 with further enhancements to support option 1b.

Observation 2: It is feasible to support the MNO full controllability and full visibility for option 1b. SA2 need further study on how to tailor and enhance the existing solution to support 1b based on RAN’s requirement.

### 1.2.2	Option 2

For option 2, the first termination entity is inside the CN and the data transfer path is UE->CN-> UE data collection server.  Both CP and UP solutions are kept for this solution with several FFS’, especially for the UP solution in the table.

Since it is not mentioned which CN node is the termination entity in the table, the overall framework for this option is not clear and it is further unclear whether a single framework or multiple frameworks should be defined to support the three use cases discussed in NR_AIML_Air.

One potential solution is to consider the LMF for AI/ML based positioning as the first termination entity in the CN. In this case, the UE reports the location information to the LMF and the LMF further sends the UE data to an LCS client or AF for UE side model training. The following 5GC-MT-LR procedures are already supported in TS 23.273:

- Clause 6.3, Deferred 5GC-MT-LR Procedure for Periodic, Triggered and UE Available Location events.

With this procedure, the LMF sends the location request via LPP message container to the AMF, the AMF sends the LPP container to the UE via NAS message. The reported location information is also included in the LPP container sent to the AMF via NAS message and the AMF will further forward the LPP container to LMF.

- Clause 6.16.1, Reporting of Location Events to an LCS Client or AF via user plane.

With this procedure, the LCS Client or the AF may request a user plane reporting in the LCS service request. If the UE supports the user plane positioning capabilities, the LMF may include the request for user plane reporting, the user plane address and the security information in the LPP container sent to AMF, the AMF then sends the LPP container to the UE via NAS message. The UE establishes a secure user plane connection to the LCS Client or the AF using the user plane address and the security information that received from the LMF. The UE data reporting will be performed via the user plane connection from UE to LCS Client / AF.

- Clause 6.18, Procedures of User Plane Connection between UE and LMF.

With this procedure, the LMF sends its user plane information to the UE via NAS message. The UE establishes a secure user plane connection with the LMF and LPP messages are then transferred between the UE and the LMF for UE based positioning via the established user plane connection.

Based on these existing procedures supported in TS 23. 273, both CP and UP solutions are feasible for positioning related UE data collection for option 2 assuming the first termination entity inside the CN is the LMF. There are, however, some conflicts with the analysis for option 2 as described in table 7.2.1.3.2-1.

CP/UP tunnel: as reported in the analysis above, both CP/UP tunnel are feasible. Both the security of data transfer and user data privacy are supported in LCS for both CP/UP solutions and  similar mechanism to the one described in TS 23.237 can be reused for option 2.

Protocol layer for data transfer: it is indicated that NAS layer is used for the CP tunnel while the protocol layer for UP tunnel is FFS. However, based on the procedure in TS 23.273, the LPP protocol will be used for both CP and UP tunnel.

Network controllability: based on TS 23.273, the MNO can fully control the data collection procedures for both CP and UP solution.

Visibility: based on TS 23.273, the MNO has full visibility for both CP and UP solution.

However, the LMF can only be used for AIML based positioning, use case. There is no other network entity in 5GC and existing procedures to support UE data collection as described in option 2, for either CP or UP solution for the other two use cases of beamforming and Channel Status Information (CSI). SA2 needs to study which entity is the first termination entity in 5GC for option 2 and define the UE data collection procedures for either CP or UP procedure.

Observation 3: If the first termination entity is the LMF, both CP and UP solutions are feasible to support UE data collection for AI/ML based positioning. The existing security and user data privacy procedure in TS 23.273 can be reused for option 2. The MNO controllability and visibility for both CP and UP solutions are feasible. The LPP protocol is used for data collection for both CP and UP solutions. SA2 needs to study the enhancement to support AI/ML based positioning feature based on the existing procedure defined in TS 23.273.

Observation 4: For CSI and beamforming use cases, there is no existing network entity in the 5GC to support the UE data collection termination and there is no existing procedure to support UE data collection via either CP or UP connection. SA2 needs to study which network entity is the first termination entity in 5GC and how to support the UE data collection procedure via either CP or UP solution for option 2 to fulfil RAN’s requirements.

### 1.2.3	Option 3

For option 3, the first termination entity is inside the OAM domain and the data transfer path is UE->gNB->OAM-> UE data collection server. Both CP and UP solutions  in the table include several FFS.

There is no existing user plane solution for data collection from UE->gNB->OAM, and we do not think it is feasible to define a UP solution between UE and OAM for data collection in Rel-19. Consequently the UP solution should be removed from the table for option 3.

The MDT/SON CP solution will likely need to be enhanced to support UE data collection for option 3. The detailed technical analysis should be discussed in SA5 and not in SA2.

Observation 5: It is proposed to remove the UP solution from option 3 and that the detailed technical analysis is to be discussed in SA5.

# 2.	Proposal

It is proposed to agree the above observations and the associated reply LS as proposed in S2-2410138.