SA WG2 Meeting #165 	S2-2410204

# 14 - 18 October 2024, Hyderabad, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0595 |  | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support AI/ML model performance monitoring by LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.18.1, there is still one EN that the performance monitoring procedure are FFS, this paper proposes to resolve the EN.. Editor's note: The Performance monitoring procedures are FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Model performance monitoring by LMF side for LMF-based AI/ML Positioning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIML_CN feature is not completed in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 5.18.X; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

5.18.X	Model performance monitoring by LMF side for LMF-based AI/ML Positioning.

Figure 5.18.X shows a procedure for AI/ML Model performance monitoring by LMF side for LMF-based AI/ML Positioning. LMF needs to select one or more PRU(s)/UE(s) to collect the known location of the PRU(s)/UE(s), and calculated location of the PRU(s)/UE(s) by using LMF-based AI/ML Positioning. LMF evaluates the Model performance by using the known location and the calculated location of the PRU(s)/UE(s) to obtain the result. The result may trigger LMF to change the positioning method, e.g., from LMF-based Positioning to the legacy positioning, or retrain the AI/ML Model.

Figure 5.18.X procedure for AI/ML Model performance monitoring by LMF side for LMF-based AI/ML Positioning

Procedure:

- LMF is triggered to perform the AI/ML Model performance monitoring for LMF-based AI/ML Positioning.

- LMF may determine the PRUs/UEs to support the AI/ML Model performance monitoring. These PRUs/UEs may be pre-configured in LMF, or LMF requests AMF to retrieve the PRUs/UEs.

The UE/PRU has subscription data to indicate in UDM whether it can support the Model performance monitoring for LMF-based AI/ML Positioning. During the registration, AMF retrieves this information from UDM.

- Based on the PRUs/UEs provided by AMF or configured in LMF, LMF triggers to initiate the AI based Positioning procedure for each PRU/UE to collect the data for AI/ML Model performance monitoring.

- a, LMF collects measurements data from UE/gNB to calculate the PRU/UE location via LMF-based AI/ML Positioning.

b, LMF also collects the known location of PRU/UE via control plane or user plane for each PRU/UE.

- LMF evaluates the AI/ML Model performance by using the calculated location and the known location of each UE/PRU to obtain the result.

- Based on the result in step 5, LMF may trigger to retrain the AI/ML Model, or change the positioning method (e.g., from AI based positioning to non-AI based positioning).

* * * End of Changes * * *