SA WG2 Meeting #165 	S2-2410215

Hyderabad, India, 14th – 18th October 2024

Source:	Ericsson

Title:	KI#1 Resolution of Editor´s Notes on Data collection

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN/Rel-19

Abstract of the contribution: This contribution proposes resolution for some of the Editor´s Note in TS 23.273.

# 1. Background

This paper discusses possible solutions to resolve some of the Editor´s Note related to LMF-based AI/ML Positioning in 23.273 and proposes a way forward that is later documented in CR 0597 to 23.273:

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The procedure for data collection from NG-RAN was studied in the TR 38.843 that concludes the following: “The necessity, feasibility and potential enhancements to facilitate the support of AI/ML for positioning accuracy enhancements with NR RAT-dependent positioning methods were studied and the outcome are outlined in clause 7.” And the in clause 7 says: “For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Proposal 1: A prerequisite procedure of Inference of UE Positioning using LMF-based AI/ML Positioning requires data collection from NG-RAN that is performed using Network Assisted Positioning Procedure as in clause 6.11.2 of TS 23.273. Note that clause 6.11.2 refers to refers to NRPPa protocol. This is already studied by RAN WGs.

Proposal 2: LMF-based AI/ML Positioning is one of the positioning methods (pending RAN2 decision) that LMF may use, those positioning methods are defined by RAN2, as such 23.273 just needs high level description and a reference to RAN2 TSs, and possibly a LS to RAN2 to ask them to inform SA2 when the LMF-based AI/ML Positioning is specified.

Editor’s note:	The procedure of LMF collects input data from NG-RAN for ML model training is FFS.

The procedure for data collection from NG-RAN was not part of the conclusions in TR 38.843. On the other hand, the conclusions in the TR 23.700-84 did not mention the procedure for data collection from NG-RAN, but the solutions listed proposed control plane solutions where the LMF provides a N2 container to NG-RAN to request channel measurements. Given that there is a procedure to provide a N2 container to request UE location to NG-RAN, i.e. Network Assisted Positioning Procedure, it is proposed to reuse it for sending a N2 container to NG-RAN to request channel measurements.

Proposal 3: The data collection for LMF-based AI/ML Positioning requires from NG-RAN is performed using Network Assisted Positioning Procedure as in clause 6.11.2 of TS 23.273.

Editor's note:	Further details on user consent for data collection for model training will be aligned with SA WG3.

Last SA2 meeting there was a discussion on whether the LCS Privacy Profile should be extended to include user consent for data collection for model training or whether to reuse the existing user consent subscription data in UDM without extensions. The following analysis explains our proposal:

The LCS Privacy Profile is used for authorization of the LCS client or the AF to access the UE location service, and the consumer of the LCS Privacy data is the GMLC, that receives requests from the LCS Client or the AF. On the other hand, the user consent for data collection for ML Model Training is to support the AIML assisted positioning method thus it is rather an agreement between the UE subscriber and the Operator no connection with the LC Client exists.

In addition, the LCS Client/AF can set specific LCS QoS requirement in the location request. As long as the LCS client or the AF is authorized to access the UE location, how the operator network will fulfill the LCS QoS requirement, e.g. choose suitable positioning methods, is transparent to the LCS client or the AF. As such the LMF checks user consent in UDM, and at the same time as part of the UE positioning requests the authorizaiton is performed using the LCS privacy profile (unchanged). The user consent is not specific per consumer or per LCS client, but it is on a per user basis.

Proposal 4: The LMF checks user consent for data collection for training using existing user consent functionality in 23.288. The authorization of the LCS Clients is still performed as part of the UE Posiotiong but is not part of the AIML model training.

Editor's note:	Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS.

Whether the AMF providing a SUPI as UE identifier to LMF violates any user privacy or not is probably not a discussion for AI/ML but is ongoing as part of 5G_eLCS as such we can wait to see what the outcome is. However, for PRU UEs, the procedures in 23.273 shows that LMF can receive a SUPI from AMF.

Alternatively, for non PRU UEs, the LMF could send a request to initiate NI-LR to the AMF, and then AMF asks LMF to obtain UE location, this is a possible solution for the AMF to send an LCS Correlation ID, as such the LMF has the LCS Correlation ID that needs to trigger a Network Initiated Positing procedure. This also works, but it generates extra and unnecessary signalling as the LMF asks AMF to trigger NI-LR, then AMF asks (the same or a different) LMF to perform UE Positioning, and then when the LMF receives the data, it sends it to AMF then to LMF again.

Proposal 5: The AMF provides a SUPI to the LMF, for both PRU and non-PRU UEs, the LMF generates a LCS Correlation Identifier to be included in a Network Assisted Positioning Request.

Other aspects:

The procedure for data collection from NG-RAN is defined in a new clause, there are common parts with the data collection from the UE, however RAN1 has indicated in LS that case 2b is second priority in RAN WID RP-240774, as such it is proposed to analyze possible common functionality when case 2b (Data Collection from the UE) is also covered by RAN WID.

# 2. Proposal

It is proposed to adopt the following proposals in the attached CR 0597, S2-2410214.

Proposal 1: LMF-based AI/ML Positioning is one of the positioning methods that LMF may use, those positioning methods are defined by RAN2, as such inference in TS 23.273 just needs high level description and a reference to RAN2 TSs, and possibly a LS to RAN2 to ask them to inform SA2 when LMF-based AI/ML Positioning inference is specified.

Proposal 2: The data collection, for training the LMF-based AI/ML Positioning, from NG-RAN is performed using Network Assisted Positioning Procedure as in clause 6.11.2 of TS 23.273.

Proposal 3: The LMF checks user consent for data collection for training using existing user consent functionality in 23.288. The authorization of the LCS Clients is still performed as part of the Positioning but is not part of the AIML model training.

Proposal 4: To start the data collection for training a ML Model to position a UE, the AMF provides a SUPI to the LMF, for both PRU and non-PRU UEs, the LMF generates a LCS Correlation Identifier to be included in a Network Assisted Positioning Request.

Proposal 5: Maintain the procedure of data collection from the UE with an Editor´s Note to indicate that if RAN2 works in case 2b in this Release, then TS 23.273 should be updated to update the procedure.