3GPP TSG-WG SA2 Meeting #165	S2-2410235

Hyderabad, Oct 14 – 18, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	A new solution: AIoT data aggregation control

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: a new solution to control AIoT data aggregation before forwarding to the consumer AF

# 1. Introduction/Discussion

This contribution is prepared to propose a new solution for Ambient IoT.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.0.0.

FIRST CHANGE (All text is new)

## 6.x	Solution #x: AIoT data aggregation control

### 6.x.1	Description

This solution address aspects of key issue #3 on Support of Ambient IoT Services.

This solution can be applied for both Topology 1 and 2.

In the 3GPP AIoT system, a consumer AF can request the 5GC to communicate with a list of target AIoT devices within the same transaction. In such cases, it would be highly inefficient for the 5GC to provide communication results one by one for each AIoT device, especially considering the large number of devices involved in typical AIoT use cases like inventory services. Therefore, several solutions (i.e., solutions #6, #8, #10, #11, #17, #18, #22, #30, #31, and #33) proposed that the AIoT reader (UE reader or gNB reader), the gNB, and/or the 5GC may aggregate the AIoT data from multiple devices before forwarding it to the consumer AF. However, none of these solutions explicitly explain the criteria for performing the aggregation, how these criteria should be determined (based on which requirements), and how they should be configured for the entity responsible for data aggregation.

Therefore, this solution outlines the considerations for deciding the aggregation rules and details how the responsible entities can get provisioned with the aggregation policy and perform the aggregation

### 6.x.2	Procedures

Figure 6.x.2-1: Information Flow for AIoT data aggregation

- The UE indicates its aggregation capability during the registration procedure.

- The AF initiates an AIoT request or subscription. This request/subscription could be for an inventory service using the AIoT devices. The request includes parameters such as the target AIoT devices, expected locations of the target AIoT devices and specific service requirements which include the followings:

- Report per AIoT device type or category: the AF may want to receive aggregated data from devices of the same type (e.g., temperature sensors, logistics tags) at once to simplify processing and analysis.

- Geographical location: the AF may want to receive aggregated data from devices that are located within the same geographical area to improve relevance.

- Time-based aggregation: the AF may want to receive aggregated data within a specific time window to provide time-synchronized information.

- Priority or criticality: the AF may want to receive aggregated data from high-priority or critical devices separately from less critical data to ensure timely processing.

- The NEF/AIoTF selects a UE reader based on location information based on the selection mechanisms captured in solution#11. This UE reader will be responsible for interacting with the target AIoT devices.

- The NEF/AIoTF decides on the aggregation rule. This decision is based on operator policy and the specific service requirements provided by the AF as well as the UE aggregation capability indicated in step 0. These rules will govern how data collected from multiple AIoT devices will be aggregated before being sent back to the AF.

The aggregation rule can include the followings:

- UE level aggregation

  - Per time window

  - Per device type

  - Per priority/criticality level

  - Per location

  - Any combination of above

- gNB level aggregation (optional)

  - Per time window

  - Per UE reader

  - Per cell

  - Per location

  - Per priority/criticality level

  - Any combination of above

- The NEF/AIoTF sends an AIoT request to the gNB over the AMF. This request includes the AIoT session ID, the target AIoT devices, and the aggregation rule that was decided in Step 3.

- The gNB may apply the aggregation policy as directed by the NEF/AIoTF. This step ensures that the gNB correctly handles data from AIoT devices according to the policy.

- The gNB forwards the AIoT request, including the AIoT session ID, target AIoT devices, and aggregation policy, to the UE reader. The gNB instructs the selected UE reader to apply the aggregation policy.

- The UE reader applies the aggregation policy as directed by the NEF/AIoTF. This step ensures that the UE reader correctly handles data from AIoT devices according to the policy.

- The UE reader begins communication with the AIoT devices, collecting data as per the AIoT request.

- The UE reader aggregates the data collected from the AIoT devices according to the applied aggregation policy.

- The UE reader sends the aggregated data back to the gNB. The response includes the AIoT session ID, the aggregated data, AIoT device IDs, and a flag indicating the completion of data reporting. If data reporting is complete, the flag is set to be a specific value like “true” or “false” otherwise. In some cases, a portion of the AIoT data may be aggregated and reported first, with the remaining data being reported in subsequent messages. The flag is used to indicate whether the UE has finished reporting all AIoT data within the same AIoT session.

- If multiple UE readers are involved, the gNB aggregates data from all the UE readers to compile a comprehensive dataset according to the aggregation rule that was decided in Step 3.

- The gNB sends the AIoT response back to the NEF/AIoTF, including the AIoT session ID, the list of target AIoT devices, the aggregated data, and a flag indicating completion.

- The NEF/AIoTF may perform further aggregation at the session level if needed, combining data from various sources into a final format.

- The NEF/AIoTF sends the final AIoT response to the AF, including the target AIoT devices, the fully aggregated data, and the end flag, indicating that the data processing is complete. If not completed, further AIoT responses can be provided until the session is completed.

### 6.x.3	Impacts on services, entities and interfaces

Entity in 5GC, responsible for AIoT service handling (e.g., AIoTF) :

- Supports determining and providing the aggregation rule considering the AF requirements and the local policy

- Supports aggregating the AIoT data at the session level.

AIoT UE reader:

- Supports performing the UE level aggregation according to the aggregation rule provided by the network.

- Supports indicating its aggregation capability during the registration procedure.

RAN node:

- Supports performing the gNB level aggregation according to the aggregation rule provided by the network.

END OF CHANGES