3GPP TSG-WG SA2 Meeting #165	S2-2410237

Hyderabad, Oct 14 – 18, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	Conclusions for KI#1

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#1 are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#1.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.0.0.

FIRST CHANGE

## 8.1	Interim Conclusion on Key Issue #1

### 8.1.1	Architecture aspects

The clause concludes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3;

The following principles are agreed for the architecture to commonly support both topologies:

-	A new core network function is introduced to support Ambient IoT.

-	The AIoT reader and the new core network function always communicate for AIoT signaling (e.g., inventory request) through AMF.

The following principles are agreed for the architecture to specifically support topology 1:

-	The BS reader communicates with the new core network function via AMF over N2 interface with NGAP.

The following principles are agreed for the architecture to specifically support topology 2:

-	The UE reader communicates with the new core network function via gNB and AMF.

-	RRC based protocol stack is supported at least for the DL.

Editor's note:	Additional conclusions are FFS.

### 8.1.2	Security aspects

The clause concludes the following aspects:

-	Authentication and authorization for the Ambient IoT Device;

-	Validation of the Ambient IoT Device identifier;

-	Whether and how to secure device operations and services for an Ambient IoT Device or a group of Ambient IoT Devices;

The following principles are agreed for the security:

-	The AIoT device does not support a UICC.

-	There is no UDM/UDR/AUSF involvement for AIoT device authentication and authorization.

-	The AIoT device is assumed to be authenticated and authorized over the top mechanism.

-	The AIoT device ID validation is supported during an AIoT service procedure.

The following principles are agreed for the security to specifically support topology 1:

-

The following principles are agreed for the security to specifically support topology 2:

-	AMF is responsible for authorizing the UE as an AIoT reader based on UE subscription information.

Editor's note:	Before making the final conclusions, SA2 should consult with SA3 to evaluate all the security related aspects.

Editor's note:	Additional conclusions are FFS.

END OF CHANGES