3GPP TSG-SA WG2 Meeting #164	S2-2410253

# Maastricht, NL, August 19-23, 2024											(revision of S2-2409421)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1428 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-Satellite-UE communication in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [NTT DOCOMO, CATT, Samsung, China Telecom], Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3_ARCH work item and the conclusion of FS_5GSAT_Ph3_ARCH captured in clause 8.3 of TR 23.700-29, this CR adds description on Support of UE-Satellite-UE communication in IMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Support of UE-Satellite-UE communication in IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

Annex AX (normative):
Support of UE-Satellite-UE communication in IMS

# AX.1	General

This annex describes IMS architecture enhancements to support UE-Satellite-UE communication in IMS, i.e., optimized media routing via IMS user plane on-board satellite.

In this Release of the specification, this feature is supported only for IMS voice/video service and for UEs belonging to the same PLMN and in the non-roaming scenario.

The term and the 5GS architecture for UE-Satellite-UE communication is defined in TS 23.501 [93]. The optimized media routing described in this annex refers to a routing of media between UEs under the coverage of the same or of different serving satellites, using this IP-CAN and with the media not transiting through any network elements on the ground. The ground fallback routing refers to the case with the media transiting through the ground segment.

The P-CSCF is responsible for determining whether to activate UE-satellite-UE communication, that is, whether it is possible to apply media routing that only relies on IMS user plane on-board satellite. P-CSCFs serving the MO UE and the MT UE shall negotiate with each other to determine whether to activate UE-satellite-UE communication.

# AX.2	Architecture and functional entities

## AX.2.1	Architecture

AX.2.1-1 Reference Architecture of the IP Multimedia Core Network Subsystem (IMS-AGW onboard)

The above figure depicts a reference architecture for UE-Satellite-UE communication.

-	For the IMS related PDU Session, UE(s) get an IP address from a PSA UPF located on the ground. Thus, their IP address is not changed when they change satellite.

-	UE(s) register onto IMS with this IP address. UE(s) keep their IP address constant and don't need to reregister when there is change of serving satellite.

-	The architecture deployment assumes, ISL(s) can be set-up within the same satellite constellation or across different constellations depending on satellite operator's deployments (SLA). The set of ISL(s) build up an IP network which is outside of the scope of 3gpp.

-	The routing between AGW-NTs across satellite via ISL is assumed to be based on IP routing and the AGWs in satellite needs to have non-conflicting IP address during resource allocation. It is up to the deployment to manage IP routing across ISL links and is out of scope of 3gpp.

## AX.2.2	Functional entities

### AX.2.2.1		P-CSCF

The P-CSCF used for this feature is enhanced to support the following functionalities:

-	the P-CSCF determines the activation of optimized media routing and interacts with 5GS, as described in Annex AX.3.

-	the P-CSCF interacts with IMS AGW, as described in Annex AX.4.

# AX.3	Optimized media routing activation

## AX.3.1	At call setup

The P-CSCF receives from the PCF the identifier of the satellite serving the UE and receives from the P-CSCF serving the remote UE the identifier of the satellite serving the remote UE. The P-CSCF shall use this information to determine whether or not activate optimized media routing.

Editor's note: 	Whether and how P-CSCF determines activation of optimized media routing based on satellite identifiers is FFS.

Editor's note: 	Whether and how to take UE subscription aspect into account for this activation is FFS.

If P-CSCF determines the activation of optimized media routing, the P-CSCF shall instruct PCF to authorize the necessary resources as per clause 5.6.2 and clause 5.7.2, so that the PCF proceeds to establish a path for optimized media routing.

Figure AX.3-1: Procedure for UE-Satellite-UE communication

1	The UE obtains an IP address from the EPS/5GS in the visited network, according to the IP Connectivity Access Network procedures specified by TS 23.401 [70] /TS 23.502 [94].

2	The SMF obtains default PCC rules, and associates it with this IP-CAN. The PCF provides these rules according to TS 23.203 [54] / TS 23.503 [95].

3	Using the IP address obtained in step 1, the UE performs IMS registration. This SIP message is IP-routed by the 5GS via PSA-UPF on the ground to the P-CSCF. When P-CSCF receives the REGISTER message, it interacts with PCF to subscribe to signalling bearer state changes and satellite access information.

4	Using the IP address obtained in step 1 in the SDP, the UE initiates a SIP session. The INVITE request is routed from the 5GS, to the P-CSCF.

5	If the PCSCF determines the UE is under satellite access, it shall include additional Satellite access information such as satellite ID and access network information. This determination is based on the P-CSCF subscription towards PCF. The PCF can subsequently fetch UE’s presence in satellite from SMF.

6.	INVITE proceeds from P-CSCF to S-CSCF and onwards.

7.	Offer is received from the far end by the P-CSCF. If an IMS-AGW on-board satellite is allocated for the far end UE, the P-CSCF will receive the additional satellite access information such as satellite ID and access network information.

8.	If the P-CSCF determine the possibility of UE-Sat-UE communication based on satellite access information received from far end, it will proceed with IMS-AGW resource allocation on-board satellite.

9.	If the P-CSCF decides to route media via satellite, it then allocates resources in IMS-AGW on board the satellite and alters the offered SDP answer accordingly.

10.	The P-CSCF provides the session information to the PCF.

11-12. Based on the IP address included in the session information, the PCF provides the PCC rules to the SMF. Based on the indication, the SMF shall insert the I-UPF on-board satellite, configure the I-UPF’s N6 to point to IMS-AGW on-board the satellite and configure N9 in PSA-UPF and I-UPF.

13.	P-CSCF forwards the Offer Response message to the originating endpoint.

14.	UE confirms receipt of the Offer Response and sends the Response Confirmation to P-CSCF. P-CSCF forwards this message to S-CSCF. The P-CSCF will include modified SDP as per resource allocation in step 8. The terminating end point responds to the originating end with an acknowledgement.

15.	When the destination party answers, the terminating endpoint sends a SIP 200-OK final response along the signalling path to the originating end to S-CSCF/P-CSCF.

15a.	The IMS-AGW resource may be updated if the SDP has changed after receiving the modified SDP in step 14.

16.	P-CSCF passes the 200-OK response back to UE. P-CSCF indicates that the media flows authorized for this session should now be enabled.

17.	Media exchanged between the UE and the far end is now routed between the I-UPF on-board serving satellite via the IMS AGW on-board the serving satellite and IMS-AGW on-board the satellite serving the far end UE.

## AX.3.2	At change of satellite

The P-CSCF shall determine whether optimized media routing continues to be possible between a satellite that is about to serve the local UE and the satellite serving the remote UE based on the identifiers of those satellites.

The UE upon detection of change of satellite, shall send a SIP re-invite. When P-CSCF receives the re-invite, P-CSCF shall check the satellite ID information from PCF based on subscribe/notify method. If the P-CSCF determines that optimized media routing can continue at change of satellite, the P-CSCF shall send a message to the PCF as defined in TS 23.502 [94] to establish a path on the target satellite for optimized media routing. The P-CSCF shall inform the far end about the new resource allocation in new target satellite (IMS-AGW).

If the P-CSCF determines that optimized media routing is no longer possible (e.g. due to a change of satellite, or no ISL between new target satellite and terminating satellite), the P-CSCF shall execute the ground fallback procedure to reroute the media via the ground.

Editor's note:	When and how P-CSCF determines that optimized media routing can continue at change of satellite is FFS.

Editor's note:	It is FFS what the ground fall back procedure is and how P-CSCF invoke this procedure.

## ---End of the Change---