3GPP TSG-WG SA2 Meeting #165	S2-2410284

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2138 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of N3GPP device behind a 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add the support of Non-3GPP device connecting behind a 5G-RG as concluded in the TR 23.700-32 KI#4. This CR adds the support for traffic differentiation, the identifiers and the description of difference between UIA scenairos and NAUN3 device. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The support of identifiable Non-3GPP device connecting behind the UE are defined in TS 23.501, TS 23.502 and TS 23.503. This CR propose to add the difference to support the identifiable Non-3GPP device connecting behind a 5G-RG. The following new specifications are added: - definition of support of QoS differentiation for identifiable Non-3GPP device in clause 4.5.3 - how to identify the Identifiable non-3GPP Devices Connecting behind a 5G-RG - Definition of the support of individual Identifiable non-3GPP devices behind a 5G-RG. - 4. Definition of the usage of device identifier in the URSP rule. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Complete the UIA specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

c

# * * * First change * * * *

### 4.5.3	Differentiated QoS for devices behind 5G-RG

When the 5G-RG does not support the identification of non-3GPP Device or does not apply QoS differentiation based on Non-3GPP identity, during PDU session establishment and PDU session modification, if the SMF provides to the 5G-RG QoS flow descriptions, the SMF may additionally signal Non-3GPP QoS Assistance Information (N3QAI) for each QoS flow to the 5G-RG. Based on the N3QAI together with QoS rule information, the 5G-RG may reserve resources in the non-3GPP network behind the 5G-RG (e.g. home LAN network). N3QAI consists of the following QoS information: QoS characteristics, GFBR/MFBR, Maximum Packet Loss Rate, ARP and Periodicity (if available at the SMF).

NOTE 1:	How 5G-RG uses the Non-3GPP QoS Assistance Information to enforce QoS in the non-3GPP network is outside the scope of 3GPP.

NOTE 2:	Transferring information like Periodicity to the 5G-RG is not meant to support TSC/TSN like flows but to support consumer real time applications like XR (extended Reality, etc.).

When the 5G-RG supports the QoS differentiation for Non-3GPP Devices, the QoS differention defined in TS  23.501 [nn] clause 5.x.y applies with 5G-RG taking the role of the UE.

# * * * * Second change * * * *

### 4.7.x	Non-3GPP Device Identifier

The Non-3GPP Device Identifier defined in TS 23.501 to identify a Non-3GPP device connecting behind a UE for which traffic differentiation is supported apply also to the Non-3GPP device connecting behind a 5G-RGwhich are nto considered as NAUN3 device .

# * * * * Third change * * * *

## 4.10e	Non-3GPP devices accessing the 5GC from behind a 5G-RG

TS 23.501 [nn] clause 5.x applies to Non-3GPP device connected behind a 5G-RG with the 5G-RG taking the role of the UE.

Unlike AUN3 devices, the Non-3GPP devices behind a 5G-RG is not authenticated by 5GC but is instead associated to the the subscription of the 5G-RG and uses the 5G-RG subscription to access the 5GC. The 5G-RG needs not support the NAS context for each Non-3GPP device behind a 5G-RG. Unlike NAUN3 device the Non-3GPP device behind a 5G-RG is uniquely identified by 5GC and associated to the 5G-RG subscription, therefore the 5GC may provide policy specific for each Non-3GPP device connected behind the same 5G-RG even if they are connected to the same LAN interface of the 5G-RG.

Figure 4.10d-1 shows an example of the overall architecture.

Figure 4.10e-1: Example scenario for Non-3GPP devices accessing 5GC from behind the same 5G-RG subscription

# * * * * Fourth change * * * *

### 9.5.2	UE access selection and PDU Session selection related policy information

#### 9.5.2.1	5G-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6. for 5G-RG.

Only URSP policies are applicable for a 5G -RG.

NOTE:	For example, ANDSP policies, ProSe Layer-3 UE-to-Network Relay Offload Policy are not applicable to 5G-RG,

If the PCF provides a URSP policy to the 5G-RG, the PCF should only provide the following URSP policy information:

-	Rule Precedence.

-	Traffic descriptor/Application descriptors, IP descriptors, Domain descriptors, Non-IP descriptors, DNN, Connection Capabilities.

-	Route Selection Descriptor Precedence.

-	Route selection components/ SSC Mode Selection, Network Slice Selection, DNN Selection, PDU Session Type Selection.

-	Route Selection Validation Criteria/ Time Window.

The 5G-RG shall ignore any of the parameters and policies not listed above, if received from the 5GC.

The 5G-RG shall use the URSP policy as specified in TS 23.503 [4], for example for the association of application and PDU session, slices, etc.

The URSP indicates for the application of Auto-Configuration Server (ACS) which PDU session type, NSSAI and/or DNN is to be used. The 5G-RG establishes the connectivity to the management entity (e.g. ACS) via user plane connection on a PDU session according to the URSP.

UE Policy procedures defined in clause 6.1.2.2 of TS 23.503 [4] are applicable as follows:

-	Roaming is not applicable to W-5GAN access in this release of specification.

-	USRP rule enforcement reporting is not defined for 5G-RG.

In the case of an Authenticable Non-3GPP (AUN3) device behind a 5G-RG only URSP policies are applicable. Each AUN3 device is managed in 5G-RG as a separate device with its own NAS and states, therefore from point of view of 5GC, the AUN3 device is seen as a single device and the URSP received in the NAS dedicated to an AUN3 device shall apply to this AUN3 device only the 5G-RG shall apply URSP received within NAS for AUN3 device acting on behalf of AUN3 device. Therefore to support the case when AUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF for the SUPI associated with the AUN3 device.

UE Route Selection Policy information targeting an AUN3 device (i.e. sent to a 5G-RG in the NAS connection corresponding to an AUN3 device) follows the structure defined in clause 6.2.2 of TS 23.503 [4] with the following difference:

-	As an AUN3 can have only one PDU Session, its URSP shall contain a match all TD.

USRP rule enforcement reporting is not defined for AUN3 devices.

In order to support the case when NAUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF on 5G-RG.

URSP rules for NAUN3 devices connected to 5G-RG follow the structure defined in clause 6.6.2 of TS 23.503 [4] and may contain any combination of the following traffic descriptors:

-	IP Descriptors: For IP traffic from NAUN3 devices connected to 5G-RG, IP descriptors are matched against header information contained in IP packets sent by NAUN3 devices; IP descriptors are only applicable for traffic from NAUN3 devices if network address translation (NAT) is performed for that traffic.

-	Non-IP descriptors: For Ethernet traffic from NAUN3 devices connected to 5G-RG, Non-IP descriptors are matched against header information contained in Ethernet frames sent by NAUN3 devices.

-	Connectivity Group ID: For traffic from a NAUN3 device connected to 5G-RG, Connectivity Group ID in the URSP rule is matched against the Connectivity Group ID that the NAUN3 device is associated with (see clause 4.10b).

NOTE : the Connectivity Group ID does not apply to the case of Non-3GPP devices accessing the 5GC behind a 5G-RG for which the traffic is identified as defined in TS 23.501 clause 5.x.

URSP rules for Non-3GPP devices accessing the 5GC behind a 5G-RG for which the traffic is identified as defined in TS 23.501 clause 5.x follow the structure defined in clause 6.6.2 of TS 23.503 [4]

# * * * * End of changes * * * *