3GPP TSG-SA WG2 Meeting #165	S2-2410309

October 14 - 18, 2024, Hyderabad, Inda	(was S2-230xxxx)

Source:	Nokia

Title:	Discussion paper UE capabilities for ATSSS with introduction of MPQUIC-E and MPQUIC-IP

Document for:	Discussion and Agreement

Agenda Item:	19.13.2

Work Item / Release:	MASSS / Rel-19

Abstract of the contribution: This paper discusses how to maintain backward compatibility while introducing new MPQUIC based steering functionalities

# 1.	Discussion

## 1.1	MA PDU Session of type Ethernet

ATSSS-LL is currently specified as mandatory funtionalitiy in the UE for MA PDU Session of type Ethernet

Clause 4.2.10 of TS 23.501 states as below:

"The ATSSS-LL functionality is mandatory in the UE for MA PDU Session of type Ethernet."

Clause 6.1.4.1.2 of TS 24.193 states as below:

"In an ATSSS capable UE, the following ATSSS-LL requirements apply:

a)	for an MA PDU session of Ethernet PDU session type, the ATSSS-LL functionality with any steering mode (i.e., any steering mode allowed for ATSSS-LL functionality) is mandatory; and"

Making a mandory feature as optional in Rel-19 will break backward compatibility. Following issues may occur with the backward incompatible change:

- If the network is at Rel-19 and only supports MPQUIC-E for Ethernet MA PDU Sessions, then any Rel-18 based UE cannot access MA PDU Session of type Ethernet. So, to provide services to a Rel-18 UE, the network shall always support ATSSS-LL for MA PDU Session of type Ethernet.

- If a Rel-19 based UE only supports MPQUIC-E, then it cannot access MA PDU Session in a network that is still running on Rel-18. So, a Rel-19 UE shall always support ATSSS-LL in order to use MA PDU Session of type Ethernet in a network running on Rel-18.

Based on the above observation, it can be concluded that ATSSS-LL shall remain as mandatory functionality for both UE and the network to support MA PDU Session of type Ethernet. Which means when the UE provides ATSSS capabilities, for an Ethernet PDU Session only 2 combination of steering functionalities possible:

i)	The UE supports only ATSSS-LL (with any steering mode) or

ii)	The UE supports ATSSS-LL (with any steering mode) and MPQUIC-E (with any steering mode)

Proposal: Keep ATSSS-LL as mandatory functionality and add MPQUIC-E as an additional optional ATSSS functionality for MA PDU Session of type Ethernet.

## 1.2	MA PDU Session of type IP

For MAP PDU Session of type IP, the situation is a bit more complicated. Since MPTCP and MPQUIC-UDP cannot be used to steer, switch, split all type of IP traffic (e.g. non TCP or non UDP traffic), it was agreed that the MPTCP functionality is always implemented bundled with ATSSS-LL in the UE; i.e. a UE that support MPTCP or MPQUIC-UDP shall also support at the minimum ATSSS-LL in Active-Stanby mode. This is also clear from the encoding of the ATSSS capability information in stage 3 specification. As per TS 23.193, the UE ATSSS capability information is encoded as below:

====================================================================================

6.1.6.2	ATSSS request PCO parameter

The purpose of the ATSSS request PCO parameter is to provide UE parameters for MA PDU session management.

The ATSSS request PCO parameter container contents are coded as shown in figure 6.1.6.2-1 and table 6.1.6.2-1.

The ATSSS request PCO parameter container contents may be one or more octets long. If the ATSSS request PCO parameter container contents is longer than one octet, octets other than the first octet shall be ignored.

| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | ATSSS-ST |  |  |  | octet 1 |

Figure 6.1.6.2-1: ATSSS request PCO parameter container contents

Table 6.1.6.2-1: ATSSS request PCO parameter container contents

| Supported ATSSS steering functionalities and steering modes (ATSSS-ST) (octet 1, bits 1, 2, 3 and 4) (see NOTE) |  |  |  |  |
| --- | --- | --- | --- | --- |
| This field indicates the 5GSM capability of ATSSS steering functionalities and steering modes. |  |  |  |  |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | ATSSS Low-Layer functionality with any steering mode allowed for ATSSS-LL supported |
| 0 | 0 | 1 | 0 | MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported |
| 0 | 0 | 1 | 1 | MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |
| 0 | 1 | 0 | 0 | MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported |
| 0 | 1 | 0 | 1 | MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |
| 0 | 1 | 1 | 0 | MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported |
| 0 | 1 | 1 | 1 | MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |
| All other values are reserved. |  |  |  |  |
| All other bits in octet 1 are spare and shall be coded as zero. |  |  |  |  |
| NOTE: If the ATSSS request PCO parameter is included in the PDN CONNECTIVITY REQUEST message with the request type information element set to "handover", the ATSSS-ST field is ignored. |  |  |  |  |

====================================================================================

From the above, it is clear that a Rel-18 UE and the network always supports ATSSS-LL based functionality, at least in Active-Standby mode. Now below situations are possible in Rel-18:

-	If the UE only supports "MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode" (i.e. bit 2 is set in the PCO parameter) and the network supports only ATSSS-LL, then the MA PDU Session can be still served with ATSSS-LL in Active-Stanby mode.

-	If the UE only supports "MPQUIC-UDP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode"(i.e. bit 3 is set in the PCO parameter) and the network supports only ATSSS-LL, then the MA PDU Session can be still served with ATSSS-LL in Active-Stanby mode.

-	If the UE only supports "MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode" and "MPQUIC-UDP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode" (i.e. bits 2 and 3 are set in the PCO parameter) and the network supports only ATSSS-LL, then the MA PDU Session can be still served with ATSSS-LL in Active-Stanby mode.

Therefore, in Rel-18 there is always a possibility to serve a MA PDU Session request with only ATSSS-LL functionality (at least in Active-Stanby mode).

Now with the introduction of MPQUIC-IP, the proposal to introduce MPQUIC-IP as a standalone functionality without any dependence of ATSSS-LL (unlike MPTCP or MPQUIC-UDP) due to the fact that MPQUIC-IP can be used for steering, switching and splitting of any IP traffic. However, below issues are foreseen while introducing MPQUIC-IP as a standalone function:

-	If the Rel-19 UE only supports "MPQUIC-IP functionality with any steering mode" and the network supports only ATSSS-LL, then the MA PDU Session request from the UE cannot be served.

-	If a Rel-19 based network supports only "MPQUIC-IP functionality with any steering mode", then it cannot serve a Rel-18 UE

So, with the above proposal, there are practical scenarios where the MA PDU Session request from the UE cannot be served by the network due to capability mismatch. Whereas this was not the case upto Rel-18. The minimum a UE and the network would always support is ATSSS-LL Active-Standby mode of operation and a MA PDU Session request can be always served if both UE and network supports ATSSS feature.

The above identified problem can be easily solved if we also introduce MPQUIC-IP functionality bundled with ATSSS-LL as was done for MPTCP and MPQUIC-UDP in the previous releases.

Based on the above observation, it is proposed that MPQUIC-IP functionality shall be introduced bundled with ATSSS-LL (at least Active-Standby mode of operation) to maintain backward compatibily and avoid issues identified. So it is propsed to introduce MPQUIC-IP functionality as below:

-	"MPQUIC-IP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode"

Proposal: Introduce MPQUIC-IP steering functionality bundled with ATSSS-LL as "MPQUIC-IP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode".

## 1.3	Conclusion

Based on the discussions above it is proposed to proceed with normative work with the below principles:

-	Keep ATSSS-LL as mandatory functionality and add MPQUIC-E as an additional optional ATSSS functionality for MA PDU Session of type Ethernet.

-	Introduce MPQUIC-IP steering functionality bundled with ATSSS-LL i.e. "MPQUIC-IP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode"