3GPP TSG-WG SA2 Meeting #165	S2-2410319

Hyderabad, India, Oct 14th – 18th, 2024

Source:	NTT DOCOMO

Title:	Discussion on interaction between IMS subsystem and PCF for enabling UE-SAT-UE communication

Document for:	Discussion

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3_ARCH / Rel-19

Abstract: Discussion on KI#3 about interaction between IMS subsystem and PCF for enabling UE-SAT-UE communication.

# 1. Introduction

This discussion paper analyses various options for interaction between IMS subsystem and the CN NFs for enabling UE-SAT-UE communication and for detecting change of serving satellite in case of mobility and proposes a way forward for which procedures and which APIs to use to enable UE-SAT-UE communication.

# 2. Discussion

The procedures on how the P-CSCF receives the satellite ID to determine the activation of UE-Sat-UE call is open. Furthermore, in case of mobility if the serving satellite changes, the procedures for the detection of change of satellite by the IMS subsystem has also been left for the normative phase. There could be two potential solutions/options as described below.

Option#1: this solution involves using the “Request of access network information” service in TS 29.514. Here, the IMS AF requests the PCF to report the access network information at the creation of the AF session. The PCF provides a Policy Control Request Trigger for the event “ANI_REPORT” to the SMF. The SMF initiates the SM Policy Association Modification procedure with PCF to report the satellite information such as satellite ID. Similarly, upon change of satellite due to mobility, the PCF gets notified about the new target satellite information. PCF reports the access network information to the AF.

Observation 1: in option#1, the event type “ANI_REPORT” currently supports “ONE_TIME” only notification method i.e., events are reported once the event is met and are not reported again unless the IMS AF refreshes the subscription. This could be a concern in case of the mobility scenario where the event will not be reported based on occurrence of change of satellite. Therefore the “ANI_REPORT” event should be enhanced to support reporting whenever the change of the access network information is detected by the SMF/PCF.

Observation 2: in option#1, the event type “ANI_REPORT” does not include notifications of early and/or late DNAI change. This could be a concern in case of the mobility scenario where an early notification of a potential change to ULCL/L-PSA on the target satellite and a late notification of establishment of ULCL/L-PSA on the target satellite could optimize the procedure.

Observation 3: in option#1, it requires the involvement of PCF during the notification procedure. For example, in the case of mobility, the SMF upon detection of change of satellite (based on gNB and satellite ID change), would need to communicate with the PCF notifying the change of satellite information. Then, the PCF could notify the P-CSCF of change of satellite information. The concern with this is more API invocation between IMS AF and the PCF which in turn would lead to more signaling and hence could increase the delay.

Option#2: this solution involves introducing a new event “SAT_UP_PATH_CH”, that is similar to “UP_PATH_CH” but for satellites. The P-CSCF uses the Npcf_PolicyAuthorization service to subscribe to the event “SAT_UP_PATH_CH” for satellites to learn the satellite information such as satellite ID. Similarly, upon change of satellite due to mobility, the P-CSCF gets notified about the new target satellite information. The SMF may notify the P-CSCF directly using the Nsmf_EventExposure_Notify service operation.

NOTE:	In this option, the event could be termed “SAT_UP_PATH_CH” to distinguish from the “UP_PATH_CH” event. The terminology could be decided during the normative phase. For clarity, the term “SAT_UP_PATH_CH” would be used here on to indicate option#2.

Observation 1: in option#2, the event type “SAT_UP_PATH_CH” could be an “EVENT_DETECTION” notification method i.e., event is reported whenever the event is met, and the subscription is alive. Hence, in case of mobility the occurrence of change of satellite could be reported whenever this event occurs.

Observation 2: in option#2, the event type “SAT_UP_PATH_CH” includes notifications of early and/or late DNAI change. Thus, in case of mobility a late notification of establishment of ULCL/L-PSA on the target satellite could optimize the procedure and further reduce delays.

Observation 3: in option#2, for notification upon occurrence of event (for example change of satellite), there is no PCF involvement. The P-CSCF could directly receive the notifications from the SMF as defined in clause 4.2.2.2 of 3GPP TS 29.508. The advantage of such a direct notification as opposed to option#1 is the reduced API invocation between IMS AF and the PCF, reduced signaling, and hence reduced delay.

The key differences between option#1 and option#2 are highlighted in the table below:

Table 1: Differences between Option#1 and Option#2

| Option#1 | Option#2 |
| --- | --- |
| Currently “ONE_TIME” only notification method. Needs to be enhanced for “EVENT_DETECTION” method | “EVENT_DETECTION” notification method possible. |
| Absence of early and/or late notifications for reporting. | Early and/or late notifications possible for event reporting. |
| PCF involvement as intermediary between SMF and IMS AF for notification. | No PCF involvement, direct notification from SMF to IMS AF. |

# 3. Conclusion and proposal

Based on the above observations, we have the following proposal:

Proposal: For the interaction between IMS subsystem and PCF for enabling UE-SAT-UE communication, the “SAT_UP_PATH_CHG” event using Npcf_PolicyAuthorization services should be used.