SA WG2 Meeting #165	S2-2410340

Hyderabad, India, October 14 – 18, 2024                  	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1487 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing MRF Usage for Data Channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are some description for data channel still uses MRF for media handling; the paper is proposed to remove MRF usage for data channel. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove MRF usage for data channel. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MRF usage for data channel needs to be cleaned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.1.1, AC.2.1, AC.10.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AA.1.1	Architectural Support

Figure AA.1.1-1 shows the architecture to support SBA interactions between IMS entities.

Figure AA.1.1-1: System Architecture to support SBA in IMS

NOTE 1:	The DCSF supports NF registration and discovery, but they do not provide services in this release of the specification.

NOTE 2:	Nimsas services are defined in the context of IMS Data Channel, see clause AA.2.4.

Figure AA.1.1-2 shows the architecture using the reference point representation.

Figure AA.1.1-2: System Architecture to support SBA in IMS in reference point representation

NOTE 3:	In this Release of the specification the SBI capable IMS AS supporting data channel services is collocated with the TAS.

NEXT CHANGE

AC.2.1	Architecture

Figure AC.2.1-1 shows the data channel architecture when using the service-based DC media function.

Figure AC.2.1-1: Architecture of IMS supporting DC

NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this Release.

NOTE 5:	Enhancements to the N33 reference point for support of data channel services are not specified in this Release.

NEXT CHANGE

AC.10.2.1	Originating Standalone Bootstrap DC Setup using PSI

The standalone IMS data channel session setup with only bootstrap DC is triggered by a UE or the user of a UE for downloading application list and/or applications. After standalone bootstrap DC is established, the UE downloads the application list via the bootstrap DC, and may display the list of application information to the user for selection, downloading, and/or initiating standalone application DC. The UE terminates the standalone IMS data channel session when the session timer expired.

Figure AC.10.2.1-1depicts a signalling flow diagram for establishing a standalone bootstrap data channel using PSI.

AC.10.2.1-1: Originating Standalone Bootstrap DC Setup using PSI

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The Request URI of the SIP INVITE message is a specific target URI, such as a specific PSI indicating standalone bootstrap DC. The initial SDP contains offers only for the bootstrap data channel establishment request with bootstrap DC stream ID.

NOTE 1:	The PSI is pre-configured in the UE#1. How and when the PSI is pre-configured is left to implementation.

NOTE 2:	UE#1 can perform step 1 at any time before initiating a standalone application data channel, such as, immediately after completing the SIP registration procedure.

2-4.	Steps 2-4 of clause AC.7.1 apply.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered local bootstrap media, DCSF, based on the value of the Called ID (i.e. the specific PSI) and the content of the initial SDP, determines that UE#1 requests to establish a standalone bootstrap DC and reserves only originating side MDC1 media information, which are used to receive UE request for application downloading from MF.

6.	DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) operation based on its policies instructing the IMS AS how to set up bootstrap data channel with MF for originating side and to terminate the session. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

7.	Step 7 of clause AC.7.1 applies.

8.	IMS AS invokes Nmf_MRM_Create (List of Media Termination Descriptors) service operation to instruct MF to allocate required data channel media resources for originating side only. IMS AS request creation of only one Media Termination which representing the local bootstrap media to be terminated.

9-10.	Steps 9-10 of clause AC.7.1 apply.

11.	IMS AS terminates the IMS session based on the specific Request URI and sends 18X response with the SDP answer through S-CSCF and P-CSCF to UE#1.

12-13.	The session with bootstrap data channel is established.

14.	The bootstrap data channels have been established between originating MF and UE#1. The UE#1 sends application request messages to MF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 based on their data channel capabilities and their choices through MF.

15-16.	If the established bootstrap DC is not used for a period of time (expiry of the session timer), UE#1 sends SIP BYE to release the bootstrap DC to save power.

END OF CHANGES