SA WG2 Meeting #165	S2-2410343

Hyderabad, India, October 14 – 18, 2024	(revision of S2-24xxxxx)

Source:	ZTE

Title:	Interim Conclusion on AIoT Device ID

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: Update interim conclusion on AIoT Device Identity and identification management.

1. Discussion

The paper is proposed to update interim conclusion on AIoT Device Identity and identification management:

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier  and Identification Management:

An ambient IoT Device Identifier can be assigned by an operator or by a third party.

Editor's note:	How it is used is FFS.

2. Proposal

It is proposed to capture the following changes into TR 23.700-13.

* * * * Start of Changes * * * *

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier  and Identification Management:

An ambient IoT Device Identifier can be assigned by an operator or by a third party.

A permanent Device ID will be stored in device and used during AIoT service procedures.

Device ID can be a part of subscription-like information of the device.

The subscription-like information is either stored in UDM when ID is assigned by operator or in an AAA server when ID is assigned by third party.

The mask information (partial of Device ID) can be used to represent a group of devices in service operations.

Readers (including RAN Readers and UE Readers) can understand Device ID and mask information for service operations.

* * * * End of Changes * * * *