3GPP TSG-WG SA2 Meeting #165	S2-2410348

# Hyderabad, India, 14th Oct – 18th Oct, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3800 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction to Split MME architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Sateliot, NEC, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions agreed in the TR 23.700-29 for KI#2 this CR proposes to introduce S&F features. This CR proposes the Annex to describe the S&F architectures using split MME option. |  |  |  |  |  |  |  |  |
| Summary of change: |  | S&F feature is implemented based on the conclusions reached for KI#2 in TR 23.700-29. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | S&F features is not completely described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex Y(new), Y.0(new), Y.1.1(new), Y.1.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

Annex Y (informative): 
Example Models of Store and Forward Satellite operation

## Y.0	Introduction

This annex provides guidance on deployment options to support store and forward satellite operation.

## Y.1	Model A: Split CN architecture

### Y.1.1	Model A-1: Split MME architecture

In this architecture option:

1)	eNB is onboard the satellite.

1)	MME is split into two functions:

a)	MME-onboard:	the MME part which is onboard the satellite. MME-onboard is in charge of (1) handling the S1 interface with the onboard eNB and (2) terminating the NAS protocol signalling from/to UEs via the onboard eNB.

b)	MME-ground:	the MME part which is on the ground network. MME-ground is in charge of handling the rest of interfaces towards other CN functions (e.g. S6a towards HSS, SGd towards SMS-GMSC/IWMSC /SMS Router, T6a towards SCEF, T6ai towards IWF-SCEF, S11 towards SGW). One MME-ground instance can be associated with one or multiple MME-onboard instances (see Figure Y.1.1-1).

Figure Y.1.1-1: "Split-MME" architecture for supporting S&F satellite operation for SMS and CP CIoT services

The split-MME architecture has below principles:

1)	How MME-onboard interacts with MME-ground is outside the scope of 3GPP in this release of specification.

2)	The MME-ground together with the set of MME-onboard instances deployed in the set of satellites behaves jointly as a single MME entity and the UE context is synchronized between them. Each MME-onboard instance is associated with a different Satellite ID identifier.

3)	When UE initiates a MO procedure that needs an interaction with a core network node on the ground, the MME-onboard stores the MO procedure transaction if the feeder link is not available and synchronizes with the MME-ground when the feeder link becomes available. The MME-ground executes the procedure with the ground network nodes and syncs back the UE context with the MME-onboard when the feeder link becomes available. The transfer of MT data from MME-onboard to UE may fail due to unsynchronized UE context at MME-onboard, in such a case the MME-onboard may attempt to deliver the MT data again after synchronizing the UE context with MME-ground.

4)	The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MME determine the satellite which will serve the UE next and the MT data is sent to the respective MME-onboard and stored in the MME-onboard of the satellite(s) when the feeder link is available (and service link is unavailable), and transferred to the UE when service link becomes available.

5)	To support UE location verification on satellite, the E-SMLC is deployed on satellite to perform the verification of UE location functionality.

6)	For the monitoring event which allows the SCS/AS to be notified of Store and Forward Satellite operation(see TS 23.682 [74] clause 5.6.1.X and 5.6.3.X) the SCS/AS communicates with HSS and MME-ground to configure and/or delete Monitoring Event.

### Y.1.2	Model A-2: Split-MME/SMS-GMSC architecture

Figure Y.1.2-1: "Split-MME/SMS-GMSC" architecture for supporting S&F satellite operation for SMS

In addition to "Split-MME" architecture option described in clause Y.1.1 to support immediate delivery report(i.e. RP-ACK) to the UE the SMS-GMSC can be deployed onboard the satellite collocated with MME-onboard. For MO SMS, if the feeder link is not available upon reception of the MO SMS the MME/SMS-GMSC onboard can store the MO-SMS and can immediately send the delivery report(i.e. RP-ACK) to the UE. The SMS-GMSC onboard and SMS-GMSC ground synchronizes the context in the same mechanism as described for MME in clause Y.1.1.

*************************** End of Changes ***************************