3GPP TSG-SA WG2 Meeting #165	S2-2410368

# Hyderabad, India, October 14 - 18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5091 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support local services in 5G Femto |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As RAN3 WG’s recommendation, for local services in 5G femto cell we can reuse LADN mechanisms. Accordingly how the LADN information(e.g. LADN DNN, LADN service area) can be provisioned to 5GS should be clarified further. Additionally to restrict the local services to the Femto cells only(i.e. not TA granularity), we may reuse NW slice having S-NSSAI location availabitity information(i.e. NW slice service area set by the Femto cells only). And, UCU is re-used for provisioning such LADN information and S-NSSAI location availability information to UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NEF can determined the LADN information associated with the 5G Femto information provisioned from AF or 5G Femto information may contain its LADN DNNs for local services. - the associated LADN information can be updated into UDM/UDR in accordance with other 5G Femto information provisioning. - UCU is used to provision UE with the LADN information and S-NSSAI location availability information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local services in 5G Femto cell are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3h |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

4.15.6.3h	5G Femto CAG provisioning information parameters

The 5G Femto information provisioning parameters are the parameters sent from an AF by invoking the Nnef_ParameterProvision Service as described in clause 4.15.6.2. The AF request can be driven by the 5G Femto CAG owner or an authorized administrator. The 5G Femto information provisioning includes the parameters listed in Table 4.15.6.3h-1. According to the received 5G Femto information provisioning parameters at NEF, the NEF updates the UDM with the provided information. The UDM updates the allowed CAG list and DNN(s) of the given UE(s) and notifies AMF via Nudm_SDM_Notification Notify message as defined in clause 4.15.6.2. Then, AMF initiates UE Configuration Update procedure to the UE(s) to update the allowed CAG list of the UE(s) along with the provided validity timer and LADN information (i.e. LADN DNN and LADN Service Area Information per DNN and S-NSSAI) and S-NSSAI location availability information as described in clause 4.2.4.2.

NOTE:	The existing CAG concept defined for PNI-NPN in 5.30.3.3 of TS 23.501 [2] is re-used for 5G Femto information provisioning.

Table 4.15.6.3h-1: Description of 5G Femto information provisioning parameters

| 5G Femto information provisioning Parameters | Description |
| --- | --- |
| UE ID(s) | Indicates the unique identifier for the user to be updated with provided CAG information. It can be GPSI or MSISDN. |
| CAG ID(s) | Indicates the CAG ID(s) to be included to the existing UE Allowed CAG list. |
| Validity time | Indicates the expiry time if the CAG information is only valid temporarily. If no expiry time is present, the provided CAG information will be valid until UE leaves the CAG cells or the next update is performed. |
| DNN(s) | Indicates the LADN DNN(s) for local services in the Femto cell(s), which can be determed by NEF based on the CAG ID(s) if not provisioned from AF. NOTE: whether and what to use the associated LADN Service Area Information can be determined by NEF; In order to restrict the local services at the Femto cell(s)(i.e. cell granularity), the LADN Service Area Information is determined per DNN and S-NSSAI, and its S-NSSAI location availability information is configured by AMF or OAM configuration. |

********** End of Changes **********