3GPP TSG-WG SA2 Meeting #165	S2-2410384

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1493 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS-AGW relocation for UE-Sat-UE communication at change of satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on 5GSAT_Ph3_ARCH WID work task, UE-satellite-UE communication via IMS-AGW deployed on satellite should be supported. When satellite changes, the IMS-AGW on satellite may be relocated to a target satellite or on the ground. This can be triggerd by a re-INVITE message. Two options are considered for the case where the UE is tasked to send the re-INVITE: - Option# A: Network sends an indication via IMS layer to trigger UE sending a re-INVITE message. This option has UE impacts. - Option# B: the UE sends the re-INVITE based on indication from modem layer to IMS layer in the UE during or after handover. However the UE does not have visibility of the user plane events in 5GC, and therefore it does not know when to send the re-INVITE message (the right time is after SMF removing onboard UPF to use ground PSA UPF or after SMF decides to select a target UPF on the target satellite). This option also has UE impacts. To avoid UE impacts two different network-based mechanisms are described in this document to trigger the re-negotiation: - Option#X: the IMS-AS, based on trigger from the P-CSCF acts as back to back user agent and sends a re-INVITE message. - Option#Y: the P-CSCF acts as back to back user agent and directly sends a re-INVITE message. Which way or whether both ways should be adopted can be decided during the meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an IMS-AGW relocation procedure for UE-Sat-UE communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompleted specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Implementation notes: The referenced TS23.501 clause 5.4.x.2.3 is added by TS23.501 CR5770 and this CR should be implemented after TS23.228 CR1492. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AX.3.2	At change of satellite

Based on the call setup procedure described in clause AX.3.1, both originating and terminating P-CSCF should subscribe to the SMF for the Notification of User Plane Management Events associated with UE-Satellite-UE communication media traffic, with Early notifications type based on clause 5.6.7.1 in TS 23.501 [93] and clause 4.3.6 in TS 23.502 [94].

Based on the description in clause 5.4.x.2.3 of TS 23.501 [93], at change of satellite, the SMF serving the originating or terminating UE may report a target satellite identifier and a candidate DNAI associated with UPF onboard the target satellite (if the target satellite is connected with source satellite) or associated with UPF on ground network (if the target satellite is not connected with source satellite). The procedure in this clause describes how media re-negotiation and IMS-AGW relocation are performed after receiving the report, in which the P-CSCF A or B can be any side of the originating or terminating P-CSCF. The P-CSCF performs different actions based on the candidate DNAI reported from SMF:

-	If the SMF reports a target satellite identifier and a candidate DNAI associated the UPF onboard the target satellite, the P-CSCF shall determine whether UE-satellite-UE communication can continue, e.g., by checking whether the target satellite is connected with the peer side satellite (if they are not the same).

-	If the SMF reports a candidate DNAI associated with the PSA UPF on ground network, the P-CSCF shall perform media re-negotiation as normal after selecting a ground IMS-AGW.

Two different mechanisms are described to trigger the re-negotiation

-	clause AX.3.2.1 describes how IMS-AS sends a re-INVITE message to UE based on the indication from P-CSCF and the re-negotiation procedure;

-	clause AX.3.2.2 describes how the P-CSCF or S-CSCF directly sends a re-INVITE message to UE and the re-negotiation procedure.

Editor’s Note: whether both mechanisms are specified or one mechanism is removed is FFS.

### AX.3.2.1 IMS-AS triggers UE to send a re-INVTE

#### AX.3.2.1.1 In case of ground DNAI reported from 5GC

Figure AX.3.2.1.1-1: In case of ground DNAI reported from 5GC

1.	the P-CSCF A receives a Late Notification from the SMF which contains a DNAI associated with a PSA UPF on ground network.

2-3. the P-CSCF A sends a re-INVITE Request Indication to IMS-AS A to trigger it to sends a re-INVITE message on behalf of UE A.

4.	After receiving the re-INVITE message with SDP offer, the UE sends a 200 OK message to the P-CSCF A.

5.	As the reported DNAI is associated with UPF on the ground network, P-CSCF A selects an IMS-AGW A' on ground and allocated transport resources with step 5a and 5b and perform normal SIP message routing procedures.

6-7: P-CSCF A sends 200 OK with SDP offer to the IMS-AS A and the IMS-AS A sends a re-INVITE message to P-CSCF B via IMS-B.

8.	P-CSCF B re-selects an IMS-AGW B' on ground and request to allocation transport resource.

9.	P-CSCF B sends a re-INVITE message to UE B.

10.	After receiving 200 OK message from UE, P-CSCF B request to allocation transport resource.

11.	P-CSCF B removes UL CL/BP and L-PSA UPF from satellite by invoking Npcf_PolicyAuthorization_Update service after receiving 200 OK from UE B.

12.	P-CSCF B sends the 200 OK message to IMS-AS A via IMS-AS B.

13.	The IMS-AS A sends an ACK to P-CSCF B via IMS-AS B, and the P-CSCF B forward the ACK to UE B.

14.	The IMS-AS A sends an ACK with SDP answer to P-CSCF A, the P-CSCF A request IMS-AGW A' to allocation transport resource.

15.	The P-CSCF A send an ACK with SDP answer to UE A.

#### AX.3.2.1.2 In case of DNAI and target satellite identifier reported from 5GC

Figure AX.3.2.1.1-2: In case of target satellite identifer and DNAI reported from 5GC

1.	the P-CSCF A receives an Early Notification from SMF which contains a target satellite identifier and a DNAI associated with a UPF onboard the target satellite.

2-3. the P-CSCF A sends a re-INVITE Request Indication to IMS-AS A to trigger it to sends a re-INVITE message to UE A.

4.	After receiving the re-INVITE message with SDP offer, the UE sends a 200 OK message to the P-CSCF A.

5.	Although the reported DNAI is associated with a UPF onboard the target satellite, P-CSCF A selects both IMS-AGW A on satellite and IMS-AGW A' on ground and allocated transport resources with step 5a and 5b, as it is the other side to determine whether UE-satellite-UE communication can continue.

6-7. P-CSCF A sends 200 OK with SDP offer and target satellite identifier to the IMS-AS A and the IMS-AS A sends a re-INVITE message with SDP offer and target satellite identifier to P-CSCF B via IMS-B. The SDP offer contains two transport address of the selected IMS-AGW A on satellite and IMS-AGW A' on ground.

8.	P-CSCF B decides whether the UE-Satellite-UE communication can continue, if it cannot continue, the P-CSCF B re-selects an IMS-AGW B' on ground and request to allocate transport resource. If it can contiunue, the P-CSCF B update the IMS-AGW B on satellite using the address information of the IMS-AWG A on satellite in SDP offer from P-CSCF A.

9.	P-CSCF B sends a re-INVITE message to UE B.

10.	After receiving 200 OK message from UE, P-CSCF B request to allocation transport resource.

11.	If P-CSCF B re-select an IMS-AGW B' on ground in step 8, P-CSCF B removes UL CL/BP and L-PSA UPF from satellite by invoking Npcf_PolicyAuthorization_Update service after receiving 200 OK from UE B.

12.	P-CSCF B sends the 200 OK message with SDP Answer and an indication of UE-satellite-UE communication to IMS-AS A via IMS-AS B.

13.	The IMS-AS A sends an ACK to P-CSCF B via IMS-AS B, and the P-CSCF B forward the ACK to UE B.

14.	The IMS-AS A sends an ACK with SDP answer to P-CSCF A.

15.	Based on whether there is an indication of UE-satellite-UE communication in step 14, the P-CSCF A determines IMS-AGW relocation to target satellite or ground.

15a-15c. If no indication of UE-satellite-UE communication received, P-CSCF A requests IMS-AGW A' on ground to allocate transport resource. If indication of UE-satellite-UE communication received, P-CSCF A selects an IMS-AGW A on target satellite and requests to allocate transport resources. P-CSCF A also releases ground IMS-AGW selected in step 15c if UE-Satellite-UE communication can continue.

16.	Based on the decision in step 15, P-CSCF A removes the UL CL/BP and L-PSA UPF from satellite or relocate the UL CL/BP and L-PSA UPF to the target satellite based on step 2 of Figure 4.3.6.3-1 in TS 23.502 [3]. If to relocate the IMS-AGW to the target satellite, the IP address of IMS-AGW B on the target satellite and DNAI associated with IMS-AGW B on satellite should be carried in the request sent to SMF.

17.	P-CSCF A forwards the ACK message with SDP offer to UE.

### AX.3.2.2 P-CSCF or S-CSCF send a re-INVTE

Figure AX.3.2-1:	Procedure for serving satellite ID change

1a.	If 5GC detects the serving satellite ID of UE A change, PCF notifies P-CSCF A that the serving satellite ID of UE A change has occurred and indicates the new serving satellite ID of UE A. If 5GC detects the UE A is not access via regenative based satellite access but using transparent satellite access, PCF notifies that to P-CSCF.

1b.	If 5GC decides to change the UP path status e.g., due to the serving satellite ID of UE A change, Early/Late Notification including a candidate DNAI will be sent to P-CSCF A since P-CSCF A has subscribed to user plane management event at call set up step 12 or step 18 during Application Function influence on traffic routing procedures described in clause 4.3.6 of TS 23.502 [94].

Case 1:

2.	P-CSCF A determines that the UE-satellite-UE communication for UE A and UE B can continue based on the new serving satellite ID of UE A and stored serving satellite ID of UE B..

3.	P-CSCF A and SMF negotiate to determine the target DNAI.

If receiving the Early/Late Notification with a candidate DNAI associated with UPF onboard satellite from SMF, P-CSCF A shall map the canidate DNAI to the AGW and determine whether the candidate DNAI associated with the AGW can be used. If the candidate DNAI can be used, the positive response will be sent to the PCF as defined in TS 23.502 [94] to establish a path on the target satellite for optimized media routing. If the candidate DNAI cannot be used, the negative response is sent to SMF. SMF may re-selet a new DNAI and trigger the negotiate procedure again or terminate the relocation procedure.

If the Early/Late Notification is not received from SMF, P-CSCF A determines to relocate the AGW to a new satellite, then P-CSCF A sends a request with the candidate DNAI associated with the target new AGW. If P-CSCF A received a positive response, then P-CSCF A is aware that the candidate DNAI can be used as the target DANI. If P-CSCF A received a negative response, then P-CSCF A will re-selet a new AGW and perform this step again.

4.	The P-CSCF A determines to send SIP re-INVITE.

Editor's note:	Whether P-CSCF triggers S-CSCF to send SIP re-INVITE to UE and peer side using the steps described in clause AX.3.2.1 step 2-3 or P-CSCF sends SIP re-INVITE to UE and peer side needs FFS.

5.	The SIP re-INVITE with modified SDP offer and the new serving satellite ID of UE A is sent to the peer side.

6.	The modified SDP offer is sent to UE A using the SIP re-INVITE procedure described in step 4.

Case 2:

2.	P-CSCF A determines that the UE-satellite-UE communication for UE A and UE B can continue based on the new serving satellite ID of UE A and stored serving satellite ID of UE B. In this case, the Early/Late Notification is not received from SMF and P-CSCF A determines AGW does not need to be relocated e.g., not every satellite has a AGW on-board.

3a.	P-CSCF A determines to send SIP re-INVITE.

Editor's note:	Whether P-CSCF triggers S-CSCF to send SIP re-INVITE to peer side using the steps described in clause AX.3.2.1 step 2-3 or P-CSCF sends SIP re-INVITE to peer side needs FFS.

3b.	P-CSCF A determines to send SIP message.

NOTE:	Using SIP message to send the new serving satellite ID of UE A to the peer side can be faster and have a smaller message size.

4a.	The SIP re-INVITE with new serving satellite ID of UE A and SDP offer is sent to the peer side.

4b.	The SIP message with new serving satellite ID of UE A is sent to the peer side.

Case 3:

2.	P-CSCF A determines that the UE-satellite-UE communication for UE A and UE B cannot continue based on the new serving satellite ID of UE A and stored serving satellite ID of UE B. In this case, the access type of UE A does not change.

3.	P-CSCF A and SMF negotiate to determine the target DNAI.

If receiving the Early/Late Notification with a candidate DNAI associated with UPF on the ground from SMF, P-CSCF A shall map the canidate DNAI to the AGW and determine whether the candidate DNAI associated with the AGW can be used. If the candidate DNAI can be used, the positive response will be sent as defined in TS 23.502 [94] to fall back to the ground. If the candidate DNAI cannot be used, the negative response is sent to SMF. SMF may re-selet a new DNAI and trigger the negotiate procedure again. 

If the Early/Late Notification is not received from SMF, P-CSCF A determines to relocate the AGW on the ground, then P-CSCF A sends a request with the candidate DNAI associated with the target new AGW. If P-CSCF A received a positive response, then P-CSCF A is aware that the candidate DNAI can be used as the target DANI. If P-CSCF A received a negative response, then P-CSCF A will re-selet a new AGW and perform this step again.

4.	P-CSCF A determines to send SIP re-INVITE.

Editor's note:	Whether P-CSCF triggers S-CSCF to send SIP re-INVITE to UE and peer side using the steps described in clause AX.3.2.1 step 2-3 or P-CSCF sends SIP re-INVITE to UE and peer side needs FFS.

5.	The SIP re-INVITE with modified SDP offer and the new serving satellite ID of UE A is sent to the peer side.

6.	The modified SDP offer is sent to UE A using the SIP re-INVITE procedure described in step 4.

Case 4:

2.	If receicing the notification that UE A is not using regenerative-based satellite access but transparent satellite access from 5GC, P-CSCF A determines that the UE-satellite-UE communication for UE A and UE B cannot continue.

3.	P-CSCF A and SMF negotiate to determine the target DNAI.

If receiving the Early/Late Notification with a candidate DNAI associated with UPF on the ground from SMF, P-CSCF A shall map the canidate DNAI to the AGW and determine whether the candidate DNAI associated with the AGW can be used. If the candidate DNAI can be used, the positive response will be sent as defined in TS 23.502 [94] to fall back to the ground. If the candidate DNAI cannot be used, the negative response is sent to SMF. SMF may re-selet a new DNAI and trigger the negotiate procedure again. 

If the Early/Late Notification is not received from SMF, P-CSCF A determines to relocate the AGW on the ground, then P-CSCF A sends a request with the candidate DNAI associated with the target new AGW. If P-CSCF A received a positive response, then P-CSCF A is aware that the candidate DNAI can be used as the target DANI. If P-CSCF A received a negative response, then P-CSCF A will re-selet a new AGW and perform this step again.

4.	P-CSCF A determines to send SIP re-INVITE.

Editor's note:	Whether P-CSCF triggers S-CSCF to send SIP re-INVITE to UE and peer side using the steps described in clause AX.3.2.1 step 2-3 or P-CSCF sends SIP re-INVITE to UE and peer side needs FFS.

5.	The SIP re-INVITE with modified SDP offer is sent to the peer side. Upon receiving the SIP re-INVITE without a satellite ID, P-CSCF#2 is aware that the UE-satellite-UE communication for UE A and UE B cannot continue because the access type of UE A is not regenerative-based satellite access.

6.	The modified SDP offer is sent to UE A using the SIP re-INVITE procedure described in step 4.

# * * * * End of changes * * * *