3GPP TSG-WG SA2 Meeting #165	S2-2410412

Hyderabad, IN, 14th Oct – 18th Oct, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Update conclusion on Identifier and Identification Management

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: The conclusion on Identifier and Identification Management is updated to further clarify the components of the Ambient IoT Device ID.

1. Introduction/Discussion

As discussed in S2-2410411, several issues related Ambient IoT Identification are analysed. Based on the analysis, following principles are proposed:

-	the Ambient IoT Device Identifier can further include a component of Home Network Identifier to either indicating an operator holding the subscription-like information or indicating no operator holding the subscription-like information.

-	The temporary ID is not supported.

-	The permanent equipment ID in not needed.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier  and Identification Management:

-	An Ambient IoT Device Identifier can be assigned by an operator or by a third party.

-	The Ambient IoT Device Identifier includes following components:

-	Home Network Identifier: either identify the operator holding the subscription-like information or indicate that no operator is holding the subscription-like information (e.g. by setting to a specific value).

-	Instance Identifier: identify a specific Ambient IoT Device.

NOTE 1:	The Instance Identifier can be GS1 EPC [10] or based on implementation.

-	The Home Network Identifier is used to locate the network storing subscription-like information. If the Home Network Identifier indicates that no operator holding the subscription-like information, local configuration can be checked to locate the subscription-like information.

# * * * * End of changes * * * *