3GPP TSG-WG SA2 Meeting #165	S2-2410422

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5775 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy control based on the energy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | EnergySys (TR23.700-66) concluded in KI#2 for subscription and policy control to support energy efficiency and energy saving (EE & ES) as service criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add new functional description of Subscription and Policy control to support EE & ES. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support Subscription and Policy control for EE & ES. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.y (new), 6.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.x	Enablers for Energy Related Information Collection, and Energy Saving

### 5.x.y	Energy consumption related subscription information

UE energy consumption related subscription information is applicable to non-operator services, i.e., non-GBR traffic of specific QoS Flow(s) or specific service data flow(s) of the PDU Session. This subscription information is stored as part of the subscription data in the UDM/UDR and is identified for the subscriber requiring permanent control. The subscription information includes at least the following data to assist the network to perform energy consumption information collection as well as energy saving strategies related to the UE:

-	Energy consumption monitoring requirements for the UE's permanent energy consumption information collection (e.g. related to maximum energy credit limit as defined in TS 22.261 [2]).

-	Energy consumption enforcement requirements for the UE's maximum energy consumption (e.g. related to a specific period of time or space as defined in TS 22.261 [2]).

-	Energy saving related information of the UE indicating the willingness of the user to participate in energy saving strategies pursued by the operator in the network.

Editor’s Note: The details of Energy consumption related subscription information is FFS.

### 5.x.z	Policy control for energy saving

The policy control for energy saving can be applicable to non-operator services (see clause 5.x.y), special application traffic or all PDU Session traffic. The PCF receives UE subscription data and potentially requests or notifications from other NFs related to the energy consumption related information and makes policy decisions (reusing the existing parameters) based on operator policy. The policy control for energy saving shall be defined according to the following principles:

-	During the PDU Session establishment procedure, the PCF retrieves the UE’s energy consumption related subscription information, based on the UE subscription data stored in the UDR.

-	For a UE subject to energy consumption monitoring requirements (see Clause 5.x.y), the PCF activates the required collection of energy consumption information at the SMF as described in clause 6.1.3.x of TS 23.503 [45].

-	For a UE subject to energy consumption enforcement requirements (see Clause 5.x.y), the PCF makes the policy decisions under consideration of the energy consumption information that is collected for the UE.

-	If the Energy saving related information (see Clause 5.x.y) is available for the UE, the PCF can take this UE’s PDU Sessions into account for energy saving policy decisions.

Editor’s Note: The details of triggering condition for energy saving policy control and how the policy decision is made is FFS.

# * * * * Second change * * * *

### 6.2.7	UDM

The Unified Data Management (UDM) includes support for the following functionality:

-	Generation of 3GPP AKA Authentication Credentials.

-	User Identification Handling (e.g. storage and management of SUPI for each subscriber in the 5G system).

-	Support of de-concealment of privacy-protected subscription identifier (SUCI).

-	Access authorization based on subscription data (e.g. roaming restrictions).

-	UE's Serving NF Registration Management (e.g. storing serving AMF for UE, storing serving SMF for UE's PDU Session).

-	Support to service/session continuity e.g. by keeping SMF/DNN assignment of ongoing sessions.

-	MT-SMS delivery support.

-	Lawful Intercept Functionality (especially in outbound roaming case where UDM is the only point of contact for LI).

-	Subscription management.

-	SMS management.

-	5G-VN group management handling.

-	Support of external parameter provisioning (Expected UE Behaviour parameters or Network Configuration parameters).

-	Support for the Disaster Roaming as described in clause 5.40.

-	Support for the control of time synchronization service based on subscription data as described in clause 5.27.1.11.

-	Support of UE energy consumption related subscription information as described in Clause 5.x.y.

To provide this functionality, the UDM uses subscription data (including authentication data and energy consumption related subscription information) that may be stored in UDR, in which case a UDM implements the application logic and does not require an internal user data storage and then several different UDMs may serve the same user in different transactions.

NOTE 1:	The interaction between UDM and HSS, when they are deployed as separate network functions, is defined in TS 23.632 [102] and TS 29.563 [103] or it is implementation specific.

NOTE 2:	The UDM is located in the HPLMN of the subscribers it serves, and access the information of the UDR located in the same PLMN.

# * * * * End of changes * * * *