3GPP TSG-WG SA2 Meeting #165	S2-2410484

Hyderabad, IN, 14th Oct – 18th Oct, 2024	(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5793 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Scenarios for media flow description information in Packet Filters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to support the traffic detection and QoS Flow mapping for mixed traffic multiplexed within one transport connection, it is concluded in FS_XRM_Ph2 to introduce the additional packet filter information besides the information available for the legacy Packet Filter set. This paper intends to clarify different feasible scenarios and the available media flow description information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds informative annex to describe the feasible scenarios for media flow description information in Packet Filters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification regarding the feasible scenarios for the media flow description information in Packet Filters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# Annex X:	Feasible Scenarios for media flow description information in Packet Filters

As described in clause 5.7.6.2 and clause 5.37.X, media flow description information in Packet Filters may be used for multiplexed media flow detection and QoS flow mapping together with the legacy IP Packet Filter information. There could be different scenarios on how to apply the media flow description information.

-	Media Flows using different protocols are multiplexed within a sinlge UDP/IP traffic flow. In this case, the first byte of the UDP payload may be used to differentiate and map the media flows into corresponding QoS Flows. In this case, the AF may provide the Protocol Type in the media flow description information and the QoS requirements for the media flow(s) as well as the legacy IP Packet Filters.

Figure 1 The first byte of UDP payload is used to identify different protocols

- RTP Streams are multiplexed in a single UDP/IP traffic flow.

Figure 2	RTP Header Format [185]

-	The RTP SSRC can be used to differentiate and map the RTP streams into corresponding QoS Flows. In this case, the AF may provide the Protocol Type which indicates the usage of RTP/SRTP and the RTP SSRC in the media flow description information and the QoS requirements for the media flow(s) as well as the legacy IP Packet Filters. The media flows with different SSRC values may share the same QoS requirements.

-	Alternatively, the RTP Payload Type which indicates the payload type of the RTP packets can also be used to differentiate and map the RTP streams into corresponding QoS Flows when the application decides to request different QoS requirements for RTP traffic with different payload types. For example, the application may decide to apply different QoS handling for video and audio traffic. In this case, the AF may provide the Protocol Type which indicates the usage of RTP/SRTP and the RTP Payload Type in the media flow description information and the QoS requirements for the media flow(s) as well as the legacy IP Packet Filters.

-	RTP and RTCP traffic are multiplexed in a single UDP/IP traffic flow. In this case, the second byte of the UDP payload is shared by the RTP M&RTP Payload Type field for RTP traffic and the RTCP Packet Type field for RTCP traffic as shown below. This can be used to differentiate and map the RTP and RTCP traffic into corresponding QoS Flows. The AF may provide the Protocol Type which indicates the usage of RTP&RTCP and the second byte of the UDP payload (i.e. the RTP M&RTP Payload Type field or the RTCP Packet Type field) in the media flow description information and the QoS requirements for the media flow(s) as well as the legacy IP Packet Filters.

Figure 3 RTP and RTCP Header Format [185]

# * * * * End of changes * * * *