3GPP TSG-WG2 Meeting #165	S2-2410508

# 14 October-18 October 2024, Hyderabad, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1394 | rev | 2 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correlating the application DC with the bootstrap DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA4 has sent an LS response in S2-24xxxx (S4-241671). Based on the response, while it is recommended that the UE should be able to choose the applications only from the local DCSF, this is up to a local policy in DCSF and can be allowed as well. …letting a local UE choose an application from a remote UE’s list should be avoided, or at least made with great care to limit the risk of choice mismatch or collision. Instead, it should be a preferable principle to let a local UE choose from its local list and the remote UE can be informed about the chosen application together with a choice to accept or reject that. If the policy in DCSF allows to download applications from the remote UE, and the remote UE has a bootstrap data channel established with both the local and remote the DCSF, the remote UE should know which bootstrap data channel to use to download the application. However, in clause AC.7.1 when UE#2 receives an SDP offer for application data channel, the SDP offer does not contain any reference to the corresponding bootstrap DC that can be used to download the indicated application. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause AC.5, clarify that the binding information that is included in the SDP offer and received by the SDP answerer, need to contain information to correlate the DC application indicated in the SDP offer with the bootstrap DC. The encoding of the binding information will be defined by SA WG4. In clause AC.7.2.1, the UE#2 uses the binding information received in the SDP offer to determine the corresponding bootstrap data channel, and download the application, if not downloaded before. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | P2P application data channel cannot be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

# AC.5	Binding of DC Application with the related DC

Information for the binding of DC Application with the related DC is required to support multiple, simultaneous DC applications in a UE.

The binding information is maintained by the HPLMN or retrieved from DC application provider when a DC Application is uploaded to DCSF. DCSF may be configured with a DC application profile associated with the binding information, which indicates the DC control policy (e.g. whether the Application DC establishment follows the P2P, P2A/A2P or P2A2P procedure). The UE receives the binding information of a DC application via the Bootstrap DC, e.g. when the DC application is downloaded to the UE.

When the UE is establishing an IMS Application DC as defined in clause AC.7.2, the binding information is provided by the offerer's UE in the SDP offer as described in TS 26.114 [76]. The IMS AS provides the binding information to the DCSF. The DCSF may use the binding information for controlling the Application DC setup.

When the UE is receiving a request to establish an IMS Application DC as defined in clause AC.7.2, the binding information is provided in the SDP offer to the SDP answerer's UE. The UE can use the binding information to determine the corresponding bootstrap DC and download the application, if not downloaded already.

*************************** Next Change ***************************

## AC.7.2	Application Data Channel Setup Signalling Procedure

### AC.7.2.1	Person to Person (P2P) Application Data Channel Setup

Figure AC.7.2.1-1 depicts a signalling flow diagram for establishing an application data channel in a person-to-person use case. In this scenario, the MF is not used to anchor the Application data channel.

In the call flow the UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS data channel session.

Figure AC.7.2.1-1: Person-to-Person Application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0.	IMS session and bootstrap data channel have been established as described in Figure AC.7.1-1. Each UE (UE#1 and UE#2) may have bootstrap data channels established with both originating MF and terminating MF. Selected data channel application has been downloaded to UE#1 and possibly UE#2.

1.	UE#1 initiates a Person-to-Person Application Data Channel for the selected application. The UE#1 sends the SIP reINVITE request with an updated SDP to IMS AS, through originating network P-CSCF and S-CSCF. The updated SDP contains the bootstrap data channel information, as well as the requested application data channel and the associated DC application binding information, according to clause AC.5 and TS 26.114 [76].

NOTE 1:	The SIP re-INVITE can be initiated either from UE#1 or UE#2.

2.	The IMS AS validates user subscription data to determine whether the media change request event should be notified to the DCSF.

3.	The IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (MediaChangeRequest Event, Calling ID, Called ID, Session ID, Event Direction, Media Info List) of the media change request event.

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information) in the notification and/or DCSF service specific policy.

5.	The DCSF determines that the added application data channel media descriptor of the SDP offer takes UE#2 as target endpoint and does not require anchoring on the local MF If MF needs to anchor application data channel, DCSF would have used the Nimsas_MediaControl_MediaInstruction service operation to instruct IMS AS to allocate data channel media resources of the MF.

Depending on a local policy in DCSF, the DCSF may decline the application download from the remote UE#2, unless the application has been selected (i.e. downloaded) by the UE#1 first. 

If the DCSF receives a notification for a media change request event for an application request that is initiated from the remote UE, if the local policy does not allow the remote UE to select an application, the DCSF may use the Nimsas_MediaControl_MediaInstruction service operation to instruct IMS AS to reject the offered media.

6.	DCSF responds to the notification received in step 3.

7-8.	IMS AS sends the re-INVITE to the originating S-CSCF and then to the terminating network side and UE#2.

9-11.	UE#2 and terminating network returns a 200 OK response with SDP answer for application data channel to originating network. Based on the received DC application binding information in the SDP offer of the re-INVITE, UE#2 can correlate the requested application data channel and the associated DC application with the corresponding bootstrap data channel. The UE#2 may need to download the corresponding DC Application signalled in the SDP offer via the corresponding bootstrap data channel (i.e. either from local or remote DCSF), if not done already and associate it with the requested application DC.

NOTE 2:	The UE at the terminating side is capable to determine if to use the DC application based on the received DC application binding information.

12.	IMS AS notifies the DCSF of the successful data channel modification.

13.	DCSF responds to the notification received in step 12.

14-15.	The IMS AS sends 200 OK response to the originating S-CSCF and P-CSCF.

16.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

17.	P-CSCF returns the 200 OK response to UE#1.

18.	UE#1 sends ACK to the terminating network.

19.	The application data channel between UE#1 and UE#2 is established. In this example, it is not anchored in MF.

*************************** End of Changes ****************************