3GPP TSG SA WG2 Meeting #165	S2-2410518

# 14 – 18 October, 2024, Hyderabad, India			                    (revision of S2-2408060)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2132 | rev | 2 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enforcement of max no of simultaneous device identifier behind an UE or 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Huawei, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the TR 23.700-32 under KI#4, it was concluded that enforcement of max no of simultaneous device identifer behind an UE or 5G-RG whose traffic need to be differentiated will be carroied out in normative phase. This CR introduces the normative description on how enforcement can be achieved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Normative description on enforcement of max no of simultaneous device identifier behind an UE or 5G-RG which traffic need to be differentoated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 specififcation will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 4.10.x	Enforcement of maximum number of devices with identifier simultaneously connected via a UE/5G-RG

When non-3GPP devices are connected via a 5G-RG and these devices need traffic differentiation then 5G-RG provides identifiers for these devices to the SMF. Based on the provided device identifiers and corresponding QoS policies, traffic differentiation may be provided. 5GC may support enforcing a maximum number of simultaneously active device identifiers connected via a 5G-RG. This number is configured in the UDM on a per 5G-RG subscription basis.

For such enforcement:

-	If the 5G-RG supports the enforcement of maximum number of non-3GPP device identifiers feature, it includes the indication of support for the feature in the UE 5GMM Network Capability.

-	The AMF, based on received information from the UDM and the indication of the support of the feature from the 5G-RG, provides the maximum number of devices with identifier that can be simultaneously connected to the 5G-RG during registration and UE Configuration Update procedure.

-	The 5G-RG keeps track of the number of non-3GPP devices with associated identifier that are connected to the network and rejects all new requests from non-3GPP devices with associated identifier once the received number from the AMF has been reached.

-	The AMF counts the non-3GPP devices with associated identifier connected via the same 5G-RG when receive PDU Session modification and rejects any new connection request from that 5G-RG to connect a non-3GPP device with associated identifier, if the number received from the UDM has been reached.

NOTE:	The appropriate cause code while rejecting the request will be decided by CT1.

**** End of Change ****