3GPP TSG SA WG2 Meeting # 164	S2-2410521

# Hyderabad, India,  14 – 18 October, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | <> | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Data Collection at LMF to train AI/ML based positioning model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The existing procedure only target UEs for data collection for AI/ML positioning model training. PRUs with reliable ground truth information should be source of measurement input too. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include PRUs as source of measurement inputs for model training at LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A reliable source of data collection (PRU) for model training is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.18.1	Data collection at LMF to train the AI/ML based positioning model to perform positioning based on UE/PRU measurements

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Editor's note:	The procedure to collect the input data for AI/ML based positioning from NG-RAN is FFS.

Figure 5.18.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements

- The LMF starts data collection for the purpose to train an AI/ML Model for UE Positioning. If there are PRUs available in the area of interest that support measurement input for AI/ML model training, the LMF may select a list of PRUs to obtain the measurement input. Additionally, the LMF may select some UEs in the area of interest that support measurement input for AI/ML model training, The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

Editor's note:	How LMF discovers the PRUs in the area of interst is FFS.

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest").

3.	The LMF checks the user consent for data collection for model training.

Editor's note:	Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS.

Editor's note:	Further details on user consent for data collection for model training will be aligned with SA WG3.

4.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE to train the AI/ML based positioning model.

* * * End of Changes * * *