3GPP TSG-SA2 Meeting #165	S2- 2410574

# Hyderabad, India, 14 - 18 October, 2024								(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0153 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transmission QoS Sync with TxInfo in 38.355 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per TS 38.355, TxInfo contains sl-PRS-Priority, sl-PRS-DelayBudget, sl-PRS-Bandwidth and sl-PRS-Periodicity. But in clause 5.7.2 of TS 23.586, when referring positioning QoS attributes from TS 38.355, only Priority level and Delay budget are included. It creates the inconsistencies between SA2 and RAN specifications. TS 23.586 should add bandwidth and periodicity into QoS information so that, besides priority and delay, application layer could also decide full Tx attributes defined by RAN and transfer to ranging layer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding sl-PRS-Bandwidth and sl-PRS-Periodicity into positioning QoS in clause 5.7.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA2 and RAN specifications are not consistent. Ranging application can’t utilise the full RAN Tx attributes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

### 5.7.2	Handling of Ranging/SL Positioning QoS

Ranging/SL Positioning QoS requirement may be provided in the Ranging/SL Positioning service request generated at the application layer, and is provided from the application layer to the Ranging/SL Positioning layer. Ranging/SL Positioning QoS requirement may be included in the Ranging/SL Positioning Service request from SL Positioning Client UE.

The Ranging/SL Positioning layer maps the Ranging/SL Positioning QoS requirement to the Ranging/SL Positioning QoS parameters and provides the Ranging/SL Positioning QoS parameters to the AS layer. If there is no received Ranging/SL Positioning QoS requirement from the application layer, the Ranging/SL Positioning layer determines the Ranging/SL Positioning QoS parameters based on the Ranging/SL positioning Policy/parameters as configured in the clause of 5.1.

Ranging/SL Positioning QoS information contains attributes defined in clause 4.1b of TS 23.273 [8] with the following additions:

-	The accuracy attribute also includes

-	the relative horizontal accuracy, and the relative vertical accuracy for relative positioning;

-	the distance accuracy and direction accuracy for Ranging.

-	Range, which indicates the applicability of the QoS attributes in the Ranging/SL Positioning operation over PC5.

-	Priority level.

-	Delay Budget.

-	Bandwidth

-	Periodicity

NOTE:	The usage of Priority level, Delay Budget, Bandwith and Periodicity are described in TS 38.355 [12].

Ranging/SL Positioning QoS information is used for determination of Ranging/SL Positioning method. The Ranging/SL Positioning methods are defined in TS 38.355 [12].

* * * End of Change * * *