3GPP TSG SA WG2 Meeting #165	S2-2410576

# Hyderabad, India, 14 – 18 October, 2024                                          (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5812 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of energy efficiency and energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR proposes text to incoporate enhancements to support energy efficiency and energy saving in the 5G network. The EECF (Energy Efficiency and Control Function) is proposed to support collection and calucaluation of energy consumption in the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to define a new NF to support collection and calculation of energy consumption information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the energy efficiency and energy saving is supported is not unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

## 5.X	Support of Energy Efficiency and Energy Saving

### 5.X.1	General

For the improved energy efficiency and energy saving, 5G system supports energy saving policies, NF discovery and (re)selection and UP path adjustment, based on the UE subscription, energy consumption in the network, operator’s policy, etc.

The Energy Efficiency Control Function (EECF) is defined for collection and exposure of the energy related information such as energy consumption information and renewable energy information in the 5G network.

The EECF supports collection of the energy related information in different granularities from the following sources:

a) From OAM:

-	NF/Node-level energy consumption information at the gNB(s) and UPF(s) serving the UE;

-	overall data volume transmitted by the gNB;

- The NF/Node-level information of a) and b) could be used for all the UEs serving by the NF/Node.

b)	From UPF

- overall data volume transmitted by the UPF;

- data volume for the QoS flow or the SDF of a UE;

- When the gNB and/or the (I-)UPF(s) which serving the UE are changed, the serving gNB ID and UPF ID will be sent to the EECF through AMF/SMF.

The EECF determines the overall energy consumption based on energy consumption per certain granularities at the serving network entity (e.g. NG-RAN, UPF) of a UE.

The Energy Consumption of a UE in the serving UPF(s) over a time period T is calculated as

EUE,UPF= UPFi∈UPFUETEUE, UPFi

where UPFUET denotes all (I-)UPFs used to server the UE during the time period T, and EUE, UPFi is the energy consumption for the UE at the i-th UPF. And, EUE, UPFi is calculated as

EUE,UPFi=EUPFiDVUE,UPFiDVUPFi

where EUPFi is the total energy consumption at the UPF over a time period T, DVUE,UPFi is the data volume transmitted for the UE at the i-th UPF, and DVUPFi is the total data volume transmitted for all UEs at the i-th UPF.

The Energy Consumption of UE in gNB over a time period T is calculated as

EUE,gNBs=gNBi∈gNBUETEUE, gNBi

where gNBUET denotes all gNBs used to server the UE during the time period T, and EUE, gNBi is the energy consumption for the UE at the i-th gNB. And, EUE, gNBi is calculated as

EUE,gNBi=EgNBiDVUE,gNBiDVgNBi

where EgNBi is the total energy consumption at the gNB over a time period T, DVUE,gNBi is the data volume transmitted for the UE at the i-th gNB, and DVgNBi is the total data volume transmitted for all UEs at the i-th gNB.

5.x.2	Exposure of network energy related information

The EECF may support the exposure of energy consumption information per different granularity subject to operator’s policy, upon request from the authorized consumers, e.g. 5GC NFs, AF.

-	If the authorized consumer is AF, the supported granularities of the exposed energy consumption information are

: per UE, per UE per application, per PDU session;

-	If the authorized consumer is 5GC NFs, the supported granularities of the exposed energy consumption information are

: per application, per UE, per-UE-per-QoS flow, per PDU session;

The exposed energy consumption information may include Energy Consumption information and/or Renewable Energy information that is collected from the serving NFs (i.e. NG-RAN and UPF) in the granularities requested by the authorized consumers. The consumer NFs, including 5GC NFs and/or AF, may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

NOTE 1: In this release, only the energy related information of user plane communication is supported, and that of control plane signalling is not supported.

NOTE 2:	The exposure and collection of renewable energy information are FFS and needs coordination with SA WG5.

NOTE 3: The measurement time period T needs to be aligned over the UPF/NG-RAN nodes for which information is collected, i.e. the start/stop of the time periods is time-synchronized.

# * * * * End of changes * * * *