3GPP TSG-SA WG2 Meeting #165		S2-2410587

14 - 18 October, 2024, Hyderabad, India

Title:	[DRAFT] Reply to Reply LS on CEN's requirements for eCall over IMS

Response to:	S2-2409582 / SP-241421  and  S2-2409585 / C1-245057

Release:	Rel-19

Work Item:	eCallCEN, TEI19

Source:	SA2

To:			SA, CT1

Cc:	CT, SA1, SA3, RAN5

Contact person:	Haris Zisimopoulos

harisz AT qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	S2-2410588, S2-2410589, S2-2410590

# 1	Overall description

SA2 thanks SA for the liaison in SP-241421 and CT1 for the liaison in C1-245057 concerning 3GPP alignment with 9 different cases of requirements from CEN for eCall over IMS.

In the liaison in C1-245057, CT1 provides findings on the 9 CEN cases and some questions to SA related to these 9 cases. In the liaison reply from SA in SP-241421, SA provides answers to some of these questions and requests SA2 and SA1 to provide answers to questions not fully answered by SA.

SA2 wishes to first clarify that SA2 does not have a different view to SA on CT1 questions fully answered by SA and thus agrees with the answers.

For CT1 questions not answered by SA or not fully answered by SA and where SA has requested SA2 answers, SA2 provides the following answers to SA and to CT1.

Finding iii): On CEN Case #1: test call is a normal IMS call based on CEN requirements. CT1 considered that using dedicated URNs can work. However, as the test call is executed as a normal IMS call, then in a home routed scenario, the home P-CSCF has to be profiled to recognise that the normal call is actually to be treated as a test emergency call else, for example, the MSD transfer would not be possible or allowed. Question 4 SA and SA WGs to decide if this shortfall is acceptable?

SA2 Answer: SA2 agrees with the CT1 statement that “in a home routed scenario, the home P-CSCF has to be profiled to recognise that the normal call is actually to be treated as a test emergency call”. However, SA2 considers that a HPLMN for a UE could be expected to provide such support based on regulatory requirements or commercial requirements (e.g. from automakers). If, instead, a test eCall over IMS was treated almost the same as a real eCall over IMS and supported entirely by the serving PLMN IMS, then there would be new impacts to a P-CSCF and E-CSCF since a test eCall over IMS is not an emergency services call. In addition, if a HPLMN were to use IMS home routing when a UE was roaming, a VPLMN solution would no longer work and support would then still be needed from the HPLMN. Since a PSAP callback for a real eCall over IMS will necessarily always go through a HPLMN and will then need some support (or at least tolerance) from the HPLMN IMS for updated MSD transfer, it seems simpler to treat a test eCall over IMS the same as a normal call and provide support entirely within the HPLMN by always using IMS home routing for any eCall capable UE. This will also avoid any dependence on IMS support in a VPLMN which may not be available if the VPLMN does not support a real eCall over IMS. SA2 attaches the related approved CR to TS 23.167 (see S2-2410590), to support a test eCall over IMS with the possibility of support entirely within the HPLMN IMS. CT1 is advised that the CR avoids a need for different support depending on serving PLMN support of eCall over IMS and includes support of a test callback from a test point and transfer of updated MSD which is part of the CEN requirement in EN 17184:2024 clause 7.3.2 and is addressed further in the next answer.

Finding v) On PSAP callback – part of CEN Case#1, #2, #3, CT1 finds that a) for test eCall being conducted as a normal IMS call, there is no need for the UE to support PSAP callback. b) there need to be a wait time that the UE waits for a PSAP callback and that this time can be variable and be provisioned by the PLMN/Operator. The attached CRs reflects this. Question 6 Is this way-forward agreeable to SA?

SA2 Answer: SA2 disagrees with CT1 Finding v) a). The original LS from CEN to SA in SP-230522 references CEN/TS 17184:2022 § 7.3.2 for CEN requirements for a test eCall which includes the following:

7.3.2 Activation of a test eCall

The in-vehicle system shall provide a way to activate a test eCall, for example by simulating the eCall triggering event, in order to test the IVS.

Such activation can be made via specific equipment and procedure(s) which may, but does not need to, be accessible to the end user, but for example only by manufacturer authorized facilities or vehicle repair shops (e.g. via a specific request on the CAN interface for line fitted equipment).

Any interoperability test between a given vehicle type and/or communication network and/or PSAP shall be achieved without interference to an operational emergency system, unless by prior arrangement.

Almost all requirements of manual and automatic eCalls, which are not specific to an emergency call (e.g. support of MSD transfer, voice communication, callback, MSD updates, eCall-only mode and HLAP timers) also apply for test eCalls. The major differences are listed below:

1. Test eCalls shall use a regular IMS call (not IMS emergency call) to a non-emergency or any other service URN and shall not mimic an emergency services call from the (signalling) perspective of a PLMN.

2. Test eCalls are not allowed in limited service conditions.

3. Test eCalls shall contain an MSD, where the test bit is set according to EN 15722 (except for tests specifically agreed with a PSAP).

NOTE 1 An IVS could attempt to activate a test eCall whether or not the PLMN to which it registers supports eCall using IMS (shown via the ECL indicator).

NOTE 2 A PLMN that does not recognize the number or service URN used for a test eCall might remove the MSD from the SIP INVITE / SIP INFO message (for instance to prevent a device that is not an IVS from using the MSD in a SIP INVITE as a free data service) and as such block provision and/or update of the MSD.

It is clear from the above parts that CEN intends that a test eCall can have a callback and that the callback can provide updated MSD just as for a real eCall over IMS. SA2 considers that supporting this will enable testing of more eCall over IMS features and is thus worth supporting.

SA2 agrees with CT1 Finding v) b). In the case of a callback for a test eCall, SA2 considers that the wait time should be very short (e.g. 10 or 15 seconds) both to ensure that the overall test eCall procedure does not take too long, considering that a user may be waiting for a positive or negative test result, and that a short wait time will reduce the possibility of some other unauthorized party from calling the UE and obtaining the MSD. If it can be assured that test points are only provided by PSAPs, then there could be an additional verification that a calling party for a test eCall callback is associated with a PSAP.

Finding vii) For all 9 CEN use cases, CT1 considers that the TS (potentially) impacted will be: Required cases as per CEN requirements/LS: For Case-1, alignment will impact CT1 TS 24.229. See also case-5. For Case-2, alignment will impact SA1 TS 22.101, SA2 TS 23.167, possibly TS 23.292 and CT1 TS 24.229. See also case-5. For Case-3, alignment with CEN impacts SA2 TS 23.167, and CT1 TS 24.229, For Case-4, alignment with CEN impacts CT1 TS 24.229. For Case-5, TBD as consideration of above findings need to be taken first. If way forward in finding vi) is agreeable, CT1 TS 24.229 is impacted. Optional cases as per CEN requirements/LS: For Case-6, Information provided by CEN seems contradicting, but if alignment is needed, there will be impact in TS 23.167, and TS 24.229. For Case-7, alignment with CEN, impacts SA2 TS 23.167 and CT1 TS 24.229. For Case-8, alignment with CEN impacts SA1 TS 22.101, SA2 TS 23.167 and CT1 TS 24.229. For Case-9, alignment with CEN impacts SA1 TS 22.101, SA2 TS 23.167 and CT1 TS 24.229.

SA2 Answer: SA2 agrees that alignment with CEN for Cases 1 to 4 can be supported at a stage 2 level in TS 23.167 and has approved the attached CRs to TS 23.167 with changes to support cases 1 to 4 that mostly align with the changes by CT1 to TS 24.229. Exceptions to this alignment are: (i) support by SA2 of callback and MSD transfer and simplification for a test eCall over IMS (see first SA2 answer above); (ii) support by SA2 for callback from a PSAP in the CS domain to a UE in the PS domain; and (iii) support by SA2 for the CEN requirements in CEN EN 17184:2024 clause 7.13.1.2 and CEN EN 17905:2024 clause 7.5.4 that deprioritize an eCall over the PS domain with no support of eCall over IMS when there is another PLMN with a PS RAT that supports eCall over IMS or with a CS RAT that was not yet attempted. SA2 has not made any changes to support case 5 as no SA2 impacts were identified. SA2 has not made any changes to support the optional cases 6 to 9 which SA2 does not believe are needed.

# 2	Actions

To SA

ACTION: 	SA2 asks SA to take the above answers into account.

To CT1

ACTION: 	SA2 asks CT1 to take the above answers and attached CRs into account in defining any further impacts to support stage 3 alignment with CEN.

# 3	Dates of next TSG SA WG2 meetings

SA2#166			18-22 November 2024		Orlando, USA

SA2 #166-Adhoc-e		20-24 January 2025		E-Meeting