SA WG2 Meeting #165	S2-2410589

# Hyderabad, India, 14 – 18 October 2024	(was S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0376 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of MSD transfer for a PSAP Callback for eCall over IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN, TEI19 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CEN has requested alignment of eCall over IMS with CEN requirements for 5 cases where 3GPP and CEN requirements for eCall over IMS differ as described in their LS to SA#104 in SP-240522. SA has agreed alignment for case 2 in LS SP-240522 according to their LS reply in S2-2409582/SP-241421. Case 2 concerns support of MSD transfer for PSAP callback for an eCall over IMS. CT has approved a CR to TS 24.229 in CP-242255 to provide the stage 3 support for this and SA has asked SA2 to look into alignment of stage 2 support for this. The stage 3 change in CP-242255 supports updated MSD transfer for callback from an NG capable PSAP but not from a non NG capable PSAP. However, TS 23.167 clause 7.7.2 supports an eCall over IMS to such a PSAP. Therefore it seems appropriate to provide stage 2 support for updated MSD transfer for a callback from both types of PSAP. CT1 can later update their stage 3 to support the missing case of callback from a non NG capable PSAP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for updated MSD transfer for a PSAP callback for a previous eCall over IMS where the PSAP may be either NG capable and in the PS domain or non NG capable and in the CS domain. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When a PSAP callback occurs for a previous eCall over IMS, transfer of updated MSD to the PSAP will not be possible according to stage 2 specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.a (new), 7.7.b (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 24.229 CR 6667 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.7.a	Callback from an NG capable PSAP with transfer of updated MSD

Figure 7.7.a-1 illustrates a high level call flow for callback to a UE from an NG capable PSAP with transfer of updated MSD after the UE has established an eCall over IMS with the PSAP using the procedure in clause 7.7.1.

Figure 7.7.a-1: Callback from an NG capable PSAP with transfer of updated MSD

1.	The UE and PSAP perform steps 1 to 5 of the procedure shown in Figure 7.7.1-1 after which, at some later point in time, the call is released. Alternatively, the UE and PSAP perform steps 1, 2 and 3a of the procedure shown in Figure 7.7.1-1 with the PSAP returning an indication to the UE that the PSAP cannot accept the call at this time.

2.	The PSAP sends a SIP INVITE to the IMS core in the HPLMN. The SIP INVITE contains a public user identity of the UE as obtained by the PSAP at step 1 and an indication of support by the PSAP for transfer of updated MSD.

NOTE 1:	The UE is assumed to be still attached or registered to the PS domain with a normal IMS registration when step 2 occurs.

3.	The IMS core forwards the SIP INVITE to the UE, after the UE has been paged if IDLE, and includes an indication that the sender is a PSAP.

NOTE 2:	The IMS Core in step 3 would be entirely within the HPLMN for non-roaming or with roaming and IMS home routing but would be in both the HPLMN and VPLMN with roaming and IMS local breakout.

4.	The UE sends a 200 OK to the IMS Core. If the INVITE includes the PSAP indication and an indication of support for updated MSD at step 3 and if the INVITE at step 3 was received within a configurable time period of either the release or non-acceptance of the eCall over IMS at step 1, then the UE includes an indication of support for transfer of updated MSD.

NOTE 3:	The configurable period is associated with transfer of updated MSD for a PSAP callback and is defined in TS 24.229 [19].

5.	The IMS core forwards the SIP 200 OK to the PSAP.

6.	The audio channel is established to support bidirectional voice communication

7.	The PSAP and UE may perform steps 1 to 4 of Figure 7.7.3-1 for transfer of updated MSD. The UE only supports step 7 if the conditions for  including an indication of support for transfer of updated MSD at step 4 were met.

8.	If the transfer of updated MSD at step 7 fails, the PSAP may request and the UE may return updated MSD transfer via the eCall Inband Modem, as defined in TS 26.267 [42]. The UE conditions for supporting step 8 are the same as the conditions for supporting step 7.

NOTE 4:	MSD transfer could fail if not supported by the IMS Core in the HPLMN or, when there is roaming and IMS local breakout, by the IMS Core in the VPLMN or by some intermediate fixed network.

# * * * * Next change * * * *

### 7.7.b	Callback from a PSAP in the CS domain with transfer of updated MSD

Figure 7.7.b-1 illustrates a high level call flow for callback to a UE from a PSAP in the CS domain with transfer of updated MSD.

This scenario may occur following eCall session establishment and release with a PSAP not supporting NG-eCall as in clause 7.7.2 or may occur following eCall session establishment and release when the UE is in the CS domain and where the UE later attaches to or registers with a network in the PS domain.

Figure 7.7.b-1: Callback from a PSAP in the CS domain with transfer of updated MSD

1a.	The UE performs steps 1 to 5 of the procedure shown in Figure 7.7.2-1 followed by a later call release. Alternatively, the UE and PSAP perform steps 1, 2, 3a and 3b of the procedure shown in Figure 7.7.2-1 with the PSAP returning an indication to the UE that the call cannot be accepted.

1b.	As an alternative, the UE, when in the CS domain, establishes or attempts to establish an eCall with a PSAP using procedures in TS 24.008 [13]. The eCall is either later released or not accepted by the PSAP and UE mobility occurs to the PS domain.

2.	The PSAP sends or instigates an IAM to an MGCF/MGW in the HPLMN for interfacing to the CS domain. The IAM contains a public telephone identity of the UE as obtained by the PSAP at step 1a or step 1b.

NOTE 1:	The UE is assumed to be attached or registered to the PS domain with a normal IMS registration when step 2 occurs. The case where the UE is combined attached to both E-UTRA connected to EPC and 2G/3G, the HPLMN continues the callback in the CS domain and the UE performs CSFB after CS paging is excluded here.

3.	The MGCF sends a SIP INVITE to the IMS core in the HPLMN.

4.	The IMS core forwards the SIP INVITE to the UE, after the UE has been paged if IDLE, and may include an indication that the sender is a PSAP.

NOTE 2:	The IMS Core in step 4 would be entirely within the HPLMN for non-roaming or with roaming and IMS home routing but would be in both the HPLMN and VPLMN with roaming and IMS local breakout.

5.	Voice call establishment is completed with a voice path only.

6.	The PSAP sends a request for an updated MSD via the eCall Inband Modem, as defined in TS 26.267 [42].

7.	The UE returns updated MSD via the eCall Inband Modem, as defined in TS 26.267 [42] if the following conditions are met:

-	The INVITE at step 4 was received within a configurable time period of the release or non-acceptance of the eCall over IMS at step 1; and

-	as an implementation option, the INVITE at step 4 includes the PSAP indication.

NOTE 3:	The configurable time period is associated with transfer of updated MSD for a PSAP callback and is defined in TS 24.229 [19].

NOTE 4:	Receipt of the PSAP indication is an optional condition because it may be difficult for an IMS core in an HPLMN to verify PSAP origination from the CS domain, especially in another country. If this condition is not enforced, the use of the eCall Inband Modem for steps 6 and 7 can still act as an indication of callback from a PSAP.

# * * * * End of changes * * * *