SA WG2 Meeting #165	S2-2410590

# Hyderabad, India, 14 – 18 October 2024	(was S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0377 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of a Test eCall over IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN, TEI19 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CEN has requested alignment of eCall over IMS with CEN requirements for 5 cases where 3GPP and CEN requirements for eCall over IMS differ as described in their LS to SA#104 in SP-240522. SA has agreed alignment for case 1 in LS SP-240522 according to their LS reply in S2-2409582/SP-241421. Case 1 concerns support of a Test eCall over IMS. CT has approved a CR to TS 24.229 in CP-242256 to provide the stage 3 support for this and SA has asked SA2 to look into alignment of stage 2 support for this. The stage 3 change in CP-242256 supports a Test eCall over IMS establishment with transfer of initial and updated MSD but omits support of a Test eCall callback from a test point with transfer of updated MSD which is part of the CEN requirement in CEN EN 17184:2024 clause 7.3.2. The stage 3 change in CP-242256 also conditions sending of the initial MSD in an INVITE on support of eCall over IMS by the serving PLMN. However, when an HPLMN supports a test eCall over IMS and uses IMS home routing for a roaming UE that is eCall capable, there is no dependence on support of a test eCall over IMS at the IMS level in a VPLMN and hence there is no need to condition inclusion of initial MSD on eCall over IMS support by a VPLMN. Therefore it seems appropriate to provide stage 2 support for both of these differences to CT1. CT1 can later update their stage 3 to support these changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for establishment of a test eCall over IMS from a UE to a test point with transfer of initial MSD and transfer of updated MSD. Add support for a callback from a test point to a UE with transfer of updated MSD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for a test eCall over IMS will be missing at a stage 2 level which may lead to errors and/or differences in different implementations which may prevent or impair a test eCall over IMS in some cases. . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 24.229 CR 6669 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex X (normative):
Procedures for a Test eCall over IMS

# X.1	General

The purpose of a test eCall over IMS is to provide a high level of assurance that an eCall capable UE and particularly a UE configured for eCall Only Mode (which may be used very rarely) can reliably support a real eCall over IMS when an actual emergency occurs.

A test eCall over IMS includes all features of a real eCall over IMS except for features that are specific to an emergency services call. A test eCall over IMS thus excludes for example the use of an emergency indication at the RRC level, an emergency PDU session or emergency PDN connection, IMS emergency registration and an emergency service indication or eCall type of emergency service indication in a SIP INVITE. However, it includes other aspects of an eCall over IMS as described in clause 7.7, including MSD transfer, MSD update and call back with MSD update.

NOTE:	A test eCall over IMS is a normal IMS session and not an emergency IMS service and thus is established either using the home IMS alone when not roaming or when roaming with IMS home routing or using both the home and visited IMS when roaming with IMS local breakout. Support for a test eCall over IMS is thus always needed from the home IMS. This support is assumed due to either regulatory or commercial requirements. However, support by a visited IMS is only needed when roaming with IMS local breakout. Because visited IMS support may not always be provided (e.g. in a VPLMN that does not support a real eCall over IMS),  HPLMN operators can choose to enforce use of IMS home routing for any UE that is eCall capable and would require support for a test eCall over IMS.

# X.2	Test eCall over IMS Session Establishment to a Test Point

Figure X.2-1 illustrates a high level call flow for a Test eCall over IMS session establishment to a test point which may either be NG capable and accessible in the PS domain or non NG capable and accessible only in the CS domain. The UE is assumed to be either already attached to or registered on the PS domain if the UE is not in eCall only mode or camped on the PS domain if the UE is in eCall only mode.

NOTE:	A non NG capable test point that is accessible only in the CS domain may be a legacy test point used to test eCall in the CS domain. It is beneficial to make use of such test points for scenarios where an NG capable test point is not available.

Figure X.2-1: Test eCall over IMS Session Establishment to a Test Point

1.	The user requests a test eCall. If the UE is not in limited service state and not already attached to or registered on the PS domain, the UE attaches to or registers on the PS domain. The UE performs a normal IMS registration if not already IMS registered. If the UE is in limited service state, the user request is denied and remaining steps are skipped.

2.	The UE sends a SIP INVITE to the IMS core. The SIP INVITE includes a non-emergency service identity that is defined by a national regulator, PSAP or eCall test provider for testing purposes. The UE also includes initial MSD and an indication of support for transfer of updated MSD. The initial MSD indicates a test eCall.

3a.	If the public user identity for the test point indicates the test point is accessible in the PS domain, the IMS core routes the SIP INVITE towards the test point.

3b.	If the public user identity for the test point indicates the test point is accessible in the  CS domain, the SIP INVITE is sent to an MGCF/MGW for interfacing to the CS domain.

3c.	If step 3b occurs, the MGCF sends an IAM towards the test point.

4a. If step 3a occurs, the test point verifies the correctness of the initial MSD and returns a SIP 200 OK to the IMS core. The SIP 200 OK explicitly includes a positive or negative acknowledgement for the initial MSD and an indication of support for transfer of updated MSD.

4b.	If step 3c occurs, the test point returns an ANM to the IMS core.

4c.	If step 4b occurs, the MGCF returns a SIP 200 OK to the IMS core.

5.	The IMS core returns the 200 OK received in step 4a or step 4c to the UE.

6.	The test call establishment is completed with a voice path only.

NOTE:	If the test call establishment fails at any of steps 1 to 6, the UE may fallback to a different PS RAT or to the CS domain if available, the same as for any other normal call.

7.	If the UE did not receive a positive or negative acknowledgement for the initial MSD in the 200 OK at step 5, the UE shall attempt to transfer the MSD to the test point via the eCall Inband Modem, as defined in TS 26.267 [42].

8.	The test point may optionally request an updated MSD from the UE. If step 4a occurred, the test point and UE use the procedure defined in clause 7.7.3 to request and transfer the updated MSD. If step 4c occurred, the test point and UE use the eCall Inband Modem procedure, as defined in TS 26.267 [42].

# X.3	Callback from a Test Point with transfer of updated MSD

Figure X.3-1 illustrates a high level call flow for callback to a UE from test point with transfer of updated MSD.

This scenario may occur immediately after a test eCall over IMS has been established to a test point as defined in Annex X.2 and has been released.  Successful completion of the procedure in Figure X.3-1 would provide some assurance that a UE would support a PSAP callback with transfer of updated MSD for a real eCall over IMS.

Figure X.3-1: Callback from a Test Point with transfer of updated MSD

1.	The UE performs steps 1 to 8 of the procedure shown in Figure X.2-1 followed by a later call release. The UE remains in the PS domain with a normal IMS registration.

2a.	If steps 3a and 4a had occurred in Figure X.2-1 as part of step 1, the test point sends a SIP INVITE to the IMS Core and includes a public user identity of the UE obtained in step 1. The INVITE also includes an indication that the test point supports transfer of an updated MSD.

2b.	If steps 3c and 4b had occurred in Figure X.2-1 as part of step 1, the test point sends or instigates an IAM to an MGCF/MGW in the HPLMN for interfacing to the CS domain. The IAM contains a public telephone identity of the UE as obtained by the test point at step 1.

NOTE 1:	The case where the UE is combined attached to both E-UTRA connected to EPC and 2G/3G, the HPLMN continues the callback from a PSAP in the CS domain using the CS domain and the UE performs CSFB after CS paging is excluded here.

2c.	If step 2b occurs, the MGCF sends a SIP INVITE to the IMS core in the HPLMN.

4.	The IMS core forwards the SIP INVITE from step 2a or step 2c to the UE, after the UE has been paged if IDLE.

5.	Voice call establishment is completed with a voice path only.

6a.	If step 2a occurred, the test point may instigate the procedure in Figure 7.7.3-1 to request and obtain an updated MSD. The UE shall reject the request for MSD if not received within a configurable time period of release of the test eCall in step 1.

6b.	If step 2b occurred, the test point may request and obtain an updated MSD via the eCall Inband Modem, as defined in TS 26.267 [42]. The UE shall reject the request for MSD if not received within a configurable time period of release of the test eCall in step 1 or if the SIP INVITE in step 4 did not identify the calling party using the same non-emergency service identity that was used by the UE to establish the test eCall over IMS at step 2 in Figure X.2-1.

NOTE 2:	The configurable time period in steps 6a and 6b is a short period to enable call back to occur as part of the same test eCall from a user perspective and to reduce a risk of transferring updated MSD to an unauthorized calling party.