Do I 3GPP TSG SA WG2 Meeting #165	S2-2410613

# October 14-18, 2024, Hyderabad, India										revision S2-2409982)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1409 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: IMS Subscribe/Notify Framework Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, T-Mobile USA, NTT DOCOMO, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Implement the conclusion in the TR on KI#1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. A new NEF service, based on SUBSCRIBE/NOTIFY request, is defined for the IMS events and event categories. Event categories includes a collection of individual events. 2. Subscriber specific events and non-subscriber specific events are supported. 3. A subscriber specific request includes the targeted subscribers. The targeted subscriber(s) can be either one distinct subscriber, or multiple subscribers given as a list. The events in the request are common for all targeted subscribers. This request is sent to the HSS instance(s) serving the given subscribers. 4. HSS supports the handling of subscriber specific requests. To that effect, the subscribe requests that are subscriber specific are sent from the NEF to HSS. HSS handles registered and unregistered UEs and ensures that a subscription is persistent when UE registers, deregisters and re-registers in IMS. Hence, HSS stores the subscription request and sends the subscription request to the IMS AS whenever the IMS AS instance supporting the requested event is assigned to the UE. The information about the subscription events stored in HSS is only relevant and consumed by HSS. NEF and AF use a new service to manage subscription requests (e.g. subscribe/unsubscribe) via HSS for subscriber specific events. 5. The IMS AS instance assigned to a UE during IMS registration is registered in HSS. Existing SBI Nhss_ImsUECM service used for S-CSCF is enhanced to allow the registration of the IMS AS instance in HSS over N71/Sh. 6. Subscription requests that are non-subscriber specific are sent directly to all IMS AS instances configured in the PLMN. 7. Notifications related to a subscription is sent directly to the NF (e.g. NEF or AF) that triggered the subscription request Enhance Nhss to support IMS AS registration, deregistration. Support a new service Nhss_ImsEE to enable subscription to subscriber related events. Support a new service Nimsas_ImsEE to enable subscription to subscriber related events, as well as non-subscriber related events. Support a new service Nnef_ImsEE to enable subscription to subscriber related events, as well as non-subscriber related events. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for a main key issue in NG_RTC PH2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX (new), AA.2.1.1, AA.2.1.2.1, AA.2.1.2.2, AA.2.1.X (new), AA.2.4.1AA.2.4.2, AA.2.4.2.2, AA.2.4.2.3 AA.2.X (new), AC.2.2.4, AC.2.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# Annex XX (normative): Subscribe/Notify Framework for Event Monitoring in IMS

## XX.1	General

This annex describes the Subscribe/Notify framework to support event monitoring in the IMS architecture. The framework enables third party AF and network functions to subscribe to IMS DC events.

IMS event exposure for event monitoring allows a consumer (e.g., a DC AS) to subscribe to and being notified about subscriber specific and non-subscriber specific IMS events and to invoke IMS services exposed by the IMS network via the NEF.

A subscriber specific event monitoring request includes the targeted subscribers. The targeted subscribers can be either one distinct subscriber or multiple subscribers indicated by their IMS public user identities. The events in the subscriber specific event monitoring request are common to all targeted subscribers. The NEF sends one event subscription request per targeted subscriber to the HSS serving the given subscriber. The NEF may determine the serving HSS based on configuration or based on HSS discovery as depicted in clause AA.3.3.

A non-subscriber specific event monitoring request does not contain any target subscriber. Non-subscriber specific event monitoring requests are sent by the NEF directly to all IMS AS instances which support monitoring of the requested IMS event. The NEF may be configured with available IMS AS instances, or the NEF can discover IMS AS instances via NRF.

Once an IMS subscriber successfully registers in IMS, and one of the IMS AS is assigned to the subscriber, the IMS AS which supports monitoring of IMS events registers its address in the HSS.

For subscriber specific event monitoring requests received from the AF (e.g., DC AS), the NEF uses Nhss_ImsEE service to manage the subscription requests (e.g., subscribe/unsubscribe) via HSS. HSS uses Nimsas_ImsEE service to manage the subscriber specific subscription requests with the IMS AS assigned to the subscriber.

## XX.2	Architecture and functions

### XX.2.1	Architecture

Figure XX.2.1-1 depicts the Subscribe/Notify framework for event monitoring in IMS. The framework supports trusted and untrusted AFs.

Figure XX.2.1-1: Subscribe/Notify Framework for event monitoring in IMS.

NOTE 1: 	Trusted AFs can use N33 or, based on operator policy, can use Nhss and Nimsas services.

NOTE 2:	 An untrusted AF can only use N33.

### XX.2.2	Reference points

The following service-based reference points support the IMS subscribe/notify framework:

-	N33:	Reference point between AF and NEF.

-	Nxx:	Reference point between NEF and HSS.

-	Nyy:	Reference point between IMS AS and NEF.

-	N71/Sh:	Reference point between IMS AS and HSS.

### XX.2.3	Functional Entities

#### XX.2.3.1	NEF

The NEF, in addition to its role as defined TS 23.502 [94], is the entry point for subscriptions to notifications for monitoring of IMS related events, via N33 reference point using the Nnef_ImsEE_Subscribe service operation.

An incoming subscription request to the NEF for a subscriber specific event may include a single IMS subscriber or a list of IMS subscribers. The NEF initiates an individual subscription request to the HSS instance corresponding to each IMS subscriber included in the incoming subscribe request from the AF.

NEF initiates a subscription request for subscriber specific events towards HSS via Nxx reference point using the Nhss_ImsEE_Subscribe service operation.

NEF initiates a subscription request for non-subscriber specific events towards applicable IMS AS instances via Nyy reference point using the Nimsas_ImsEE service.

NEF receives notifications corresponding to subscribed IMS events from the applicable IMS AS instances via Nyy reference point using the Nimsas_ImsEE_Notify service operation. NEF sends the received notifications to the requesting AFs via N33 reference point using the Nnef_ImsEE_Notify service operation.

#### XX.2.3.2	HSS

HSS handles subscriptions to IMS DC events related to registered and unregistered IMS subscribers and ensures that a subscription is persistent when UE registers, deregisters and re-registers in IMS.

HSS receives subscription requests for subscriber specific events from NEF via Nxx reference point using the Nhss_ImsEE_Subscribe service operation.

HSS stores the incoming subscription request and sends the subscription request to the IMS AS instance supporting the requested event and assigned to the IMS subscriber via N71 reference point, using the Nimsas_ImsEE_Subscribe service operation, or via Sh reference point.

The IMS AS assigned to an IMS subscriber is registered in HSS via N71 reference point using the Nhss_ImsUECM_Registration service operation, or via Sh reference point. If the IMS subscriber in the incoming subscription request is not registered, HSS ensures that when the IMS subscriber registers, any pending subscription request is sent to the applicable IMS AS instance assigned to the IMS subscriber. IMS AS rejects subscriptions to IMS DC events for a subscriber that is not subscribed to IMS DC service.

HSS ensures that corresponding notifications are directly sent to the NEF as subscribing entity, and not via HSS.

#### XX.2.2.3	IMS AS

Once an IMS subscriber successfully registers in IMS, any DC supporting IMS AS allocated to the subscriber that supports monitoring of IMS events shall register in HSS its IMS AS instance via N71 reference point using the Nhss_ImsUECM_Registration service operation, or via Sh reference point.

The IMS AS receives subscription requests from HSS related to supported subscriber specific IMS DC events via the N71 reference point using the Nimsas_ImsEE_Subscribe service operation, or via the Sh reference point. IMS AS also receives subscription requests from NEF related to non-subscriber specific events via the Nyy reference point using the Nimsas_ImsEE service operation.

IMS AS sends notification events directly to the NEF as subscribing entity included in the subscription request via Nyy reference point using the Nimsas_ImsEE_Notify service operation.

### XX.2.4	Determination of IMS Events supported by an IMS AS

The NF profile registered in NRF of an IMS AS that supports monitoring of IMS events may include support for the Nimsas_ImsEE service and related service operations. Additionally, the NF profile of the IMS AS may include the list of IMS events the IMS AS instance supports. NF consumers of the Nimsas_ImsEE service (i.e., HSS and NEF) can make use of this information in the NF profile of the IMS AS to determine the IMS AS instances that support monitoring of a given IMS event.

Editors Note: The support for subscription to a collection of events in a single subscription is FFS.

### XX.2.5 Exposure Events

The following events are defined:

- Notify when a bootstrap data channel is established or terminated.

- Notify when an application data channel is established or terminated.

Editors Note: The definition of these events, including required event filters to support them is FFS

This includes e.g. notification when an IMS DC session to or from a certain number and/or including a specific DC application ID is established or terminated.

The subscription request for the notification of an IMS event type may include event filter information, e.g. condition for notifying the event. The event filter depends on the event type.

The actual events of IMS event types and corresponding event filters are only meaningful for the AF and the IMS AS. The involved NF consumer of the IMS AS (e.g., HSS/NEF) determines which IMS AS supports a given IMS event type by matching the IMS event type requested by the AF with the list of supported IMS event types included in the NF profile of IMS AS registered in the NRF or by implementation means.

NOTE: 	The actual value of IMS event types and corresponding event filters are transparent for the NRF, HSS and NEF. This allows that additional standardized as well as non-standardized event types can be supported without the need to upgrade the NRF, HSS or NEF.

## XX.3	Subscribe/Notify Procedures for Event Monitoring in IMS

### XX.3.1	IMS AS instance Registration in HSS

Figure XX.3.1-1depicts a signalling flow diagram for an IMS AS registering in HSS the IMS AS instance assigned to a registering UE.

Figure XX.3.1-1: IMS AS Instance Registration in HSS

1.	UE performs initial IMS Registration (see clause 5.2).

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS AS assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

### XX.3.2 Subscribe/Notify Procedure for subscriber specific IMS events

Figure XX.3.2-1depicts a signalling flow diagram for establishing a subscription for notification of a subscriber specific IMS event.

Figure XX.3.2-1: SUBSCRIBE/NOTIFY Procedure for Subscriber specific IMS events.

The steps in the call flow are as follows:

1.	UE performs initial IMS Registration as per clause XX.3.1.

2.	AF subscribes to NEF initiating the Nnef_imsEE_Subscribe Request for a subscriber specific IMS event. The AF may include one or more IMS subscriber IDs in the subscription request.

3.	NEF initiates for each IMS subscriber in the incoming subscription request a separate subscription request towards HSS for the requested event via the Nhss_ImsEE_Subscribe Request service operation. The NEF creates a Notification Target address and Notification Correlation ID and includes it to the Nhss_ImsEE_Subscribe Request.

4.	NEF returns to AF the Nnef_ImsEE_Subscribe Response.

5.	HSS locates the IMS AS instance serving the UE based on the IMS AS registration in HSS executed in step 1. The HSS determines if the IMS AS instance for the IMS subscriber (IMPU) supports the requested IMS event  based on the NF profile of the registered IMS AS instances stored in NRF.

NOTE:	If the UE is not IMS registered as in step 1, then the HSS will store the event subscription request per IMPU.

6.	HSS subscribes to the IMS AS instance serving the UE and supporting the requested IMS event, using Nimsas_ImsEE_Subscribe Request for the requested IMS event including the IMPU and the NEF address as notification endpoint. Alternatively, HSS can use Sh interface to send the requested IMS event to the IMS AS instance assigned to the UE.

7.	IMS AS returns to HSS with the Nimsas_ImsEE_Subscribe Response or returns to HSS via Sh interface.

8.	HSS returns to NEF the Nhss_ImsEE_Subscribe Response.

9.	At some point, the requested event for the UE is detected by the IMS AS.

10. The IMS AS sends an Nimsas_ImsEE_Notify Request to NEF.

11.	NEF sends to IMS AS Nimsas_ImsEE Notify Response to the IMS AS.

12.	The NEF maps the Notification Correlation ID to the subscription in the NEF. NEF sends Nnef_ImsEE_Notify Request to AF using the Notification Target Address and Notification Correlation ID received in Step 2.

13.	The AF sends an Nnef_ImsEE_Notify Response to the NEF.

### XX.3.3 Subscribe/Notify Procedure for Non-subscriber specific IMS events

Figure XX.3.3.1-1 depicts a signalling flow diagram for establishing a subscription for notification of a non-subscriber specific IMS event.

Figure XX.3.3-1: SUBSCRIBE/NOTIFY Procedure for non-subscriber specific IMS events

The steps in the call flow are as follows:

1.	AF subscribes to NEF initiating the Nnef_imsEE_Subscribe Request for non-subscriber specific IMS event.

2.	NEF returns to AF the Nnef_ImsEE_Subscribe Response.

3.	NEF locates the IMS AS instances that support the requested IMS event via NRF or by local configuration.

4.	For each IMS AS instance that supports the requested IMS event, NEF subscribes to the IMS AS using Nimsas_ImsEE_Subscribe Request for the requested IMS event.

5.	IMS AS returns to NEF the Nnef_ImsEE_Subscribe Response.

6.	At some point, the requested event for the UE is detected by the IMS AS.

7.	The IMS AS sends an Nimsas_ImsEE_Notify Request to NEF.

8.	NEF sends to IMS AS Nimsas_ImsEE Notify Response.

9.	NEF sends Nnef_ImsEE_Notify Request to AF.

10.	 The AF sends an Nnef_ImsEE_Notify Response to the NEF.

**** Next Change ****

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| ImsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| Ims UE Context | Registration | Request/Response | S-CSCF, IMS AS, |
| Management | DeregistrationNotification | Subscribe/Notify | S-CSCF |
| (_ImsUECM) | Deregistration | Request/Response | S-CSCF, IMS AS |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure (_ImsEE) | Subscribe | Subscribe/Notify | NEF, Trusted AF |
|  | Unsubscribe |  | NEF, Trusted AF |

**** Next Change ****

#### AA.2.1.2.1	Nhss_ImsUECM_Registration service operation

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer, Registration Type (e.g. Initial Registration, Unregistered).

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

**** Next Change ****

#### AA.2.1.2.2	Nhss_ImsUECM_Deregistration service operation

Service operation name: Nhss_ImsUECM_Deregistration

Description: This service operation deregisters the S-CSCF allocated to a public identity if the consumer is S-CSCF, and deregisters the IMS AS instance assigned to the public identity if the consumer is IMS AS.

Inputs, Required: S-CSCF name if the S-CSCSF is the consumer, IMS AS instance ID if the IMS AS is the consumer, Deregistration Type.

Inputs, Optional: User Identity (Private Identity and/or Public Identity), P-CSCF Restoration indication, Session Priority.

Outputs, Required: Result indication.

Outputs, Optional: None.

**** Next Change ****

### AA.2.1.X	Nhss_ImsEventExposure (ImsEE) service

#### AA.2.1.X.1	Nhss_ImsEE_Subscribe service operation

Service operation name: Nhss_ImsEE_Subscribe

Description: The NF consumer subscribes for a specific subscriber related IMS event.

Inputs, Required: IMS Public Subscriber identity, Event-ID, Notification Target address.

Event-ID represents one or multiple events as defined in clauses XX.2.4, and XX.2.5.

Inputs, Optional: Notification Correlation ID, Event Filters.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

#### AA.2.1.X.2	Nhss_ImsEE_Unsubscribe service operation

Service operation name: Nhss_ImsEE_Unsubscribe

Description: The NF consumer unsubscribes for notifications to requested IMS event.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

**** Next Change ****

## AA.2.4	IMS AS Services

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |
| Nimsas_ImsSessionManagement | Create | Request/Response | NEF, Trusted AF |
|  | Update | Request/Response | NEF, Trusted AF |
|  | Delete | Request/Response | NEF, Trusted AF |
| Nimsas_ImsEventExposure (ImsEE) | Subscribe | Subscribe/Notify | NEF, HSS, Trusted AF |
|  | Unsubscribe |  | NEF, HSS, Trusted AF |
|  | Notify |  | NEF, Trusted AF |

NOTE Y:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

NOTE  X: 	There shall be no overlap between events supported in Nimsas_ImsEventExposure service vs events implicitly supported in Nimsas_SessionEventControl service.

### AA.2.5.2	Nimsas_ImsEventExposure (ImsEE) service

#### AA.2.5.2.1	Nimsas_ImsEE_Subscribe service operation

Service operation name: Nimsas_ImsEE_Subscribe

Description: The NF consumer subscribes for a specific subscriber related IMS Event.

Inputs, Required: Event-ID, Notification Target address.

Event-ID represent one or multiple events as defined in clauses XX.2.4, and XX.2.5.

Inputs, Optional: IMS Public Subscriber identity if the subscribe request is for a subscriber  specific event, Notification Correlation ID, Event Filters.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

#### AA.2.5.2.2	Nimsas_ImsEE_Unsubscribe service operation

Service operation name: Nhss_ImsEE_Unsubscribe

Description: The NF consumer unsubscribes for updates to Requested event.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

#### AA.2.5.2.3	Nimsas_ImsEE_Notify service operation

Service operation name: Nimsas_ImsEE_Notify

Description: This service operation is used by the IMS AS to report the information associated with a subscribed Event.

Inputs, Required: Event ID, Event Reporting information.

The Event ID parameter defines the event for which the notification applies.

Inputs, Optional: Notification Correlation Information.

Outputs, Required: None.

**** Next Change ****

## AA.2.X	NEF Services

### AA.2.X.1	General

NEF services are defined in TS 23.502 (94).

**** Next Change ****

### AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS interacts with the NEF for event subscription and notification;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 or wit MRF via Mr'/Cr for data channel media resource management;

-	The IMS AS interacts with HSS for retrieving and storing DC enhanced IMS AS service specific data via N71/Sh.

-	The IMS AS interacts with the NEF and HSS to register its NF instance for event subscription;

**** Next Change ****

### AC.2.2.6	HSS

In order to support data channel service, the HSS is additionally enhanced with the following functionalities:

-	The HSS stores IMS Data Channel subscription data as transparent data.

-	The HSS interacts with DCSF during service data retrieval by DCSF via N72/Sc.

-	The HSS interacts with IMS AS during service data retrieval by IMS AS via N71/Sh.

-	The HSS interacts with IMS AS to subscribe event notifications on behalf of other NF, e.g. NEF or DC application server.

**** End of Changes ****