3GPP TSG-SA2 Meeting #165	S2-2409828r01

# Hyderabad, India, Oct 14 – 18, 2024                                    Revision of S2-2409416

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1134 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General training procedure for Vertical Federated Learning between NWDAF(s) and AF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, CATT, ZTE, OPPO, China Telecom, ETRI, Huawei, Nokia, Futurewei, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions for KI#2: 5GC Support for Vertical Federated Learning in clause 8.2 of TR23.700-84, this CR aims to specify the general training procedure for vertical federated learning between AF(s) and NWDAF(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2G.2.X is added to specify the general training procedure for vertical federated learning between AF and NWDAFs. Compared to the revision S2-2409416 in last meeting SA2#164, in TS23.288, VFL Registration and Discovery procedure in clause 6.2H.2.1 is already defined, hence, the related EN can be removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training procedure not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 6.2H 	Vertical Federated Learning

#### 6.2H.1 	General

#### 6.2H.2 	Procedures

#### 6.2H.2.1 	Registration and Discovery Procedure for Vertical Federated Learning

#### 6.2H.2.2 	Preparation Procedure for Vertical Federated Learning

#### 6.2H.2.3 	Training Procedure for Vertical Federated Learning

##### 6.2H.2.3.1 	Training Procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

##### 6.2H.2.3.2 	Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

#### 6.2H.2.4 	 Inference Procedure for Vertical Federated Learning

##### 6.2H.2.4.1 	Inference Procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

##### 6.2H.2.4.2 	Inference Procedure for Vertical Federated Learning NWDAF is acting as VFL server

* * * Next Change * * *

### 6.2H.2.3 Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3-1: Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The VFL training procedure illustrated below is conducted after VFL preparation procedure.

Editor's Note: Further extensions are needed to show when any of the VFL participants are untrusted AF(s). In this case the procedure below will contain a NEF, and how the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor’s Note: A possible need for a process of sample exchange between VFL server and VFL client is FFS. The process may be determined after agreement on sample and/or feature alignment.

Editor’s Note: Whether and how to exchange feature between VFL server and VFL client is for FFS. The process may be determined after agreement on sample and/or feature alignment.

Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training round or there is a session concept.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor´s Note: It is FFS whether and how VFL server collects trained ML model from VFL clients after the VFL training process.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H2.2.

NOTE: VFL Server determines to perform VFL training based on the operator’s policy.

2.	To start VFL training, the VFL server allocates VFL correlation ID, The VFL server sends the VFL correlation ID request to the selected VFL clients. When VFL server is NWDAF, it sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF clients(s) and Naf_MLModelTraining_Subscribe or Naf_MLModelTrainingInfo_Request to the selected AF clients(s), which participate in the Federated learning to perform the local model training. The request includes VFL correlation ID, sample IDs.

Editor´s Note: Whether more parameters are needed in the Nnwdaf_MLModelTraining or Naf_MLModelTraining service is FFS.

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data , and reports the Client intermediate training result of the local ML model to the VFL server. When the clients reports the Client intermediate training result, it also includes the corresponding VFL correlation ID.

5.	The VFL server may collect the local data and generate a local intermediate training result of its own local ML model (if available)The NWDAF acting as VFL Server computes the backward local ML model training information (e.g. gradient information or loss information) based on local intermediate training result (if available) and the Client intermediate training result received in step 4 and the label. The backward local ML model training information is used for updating the local ML model and the local ML model of VFL clients. Different backward local ML model training information may be computed for different VFL clients, respectively, taking into account the supported feature(s) per VFL client.

Editor’s Note: whether and how VFL server and VFL client share features is FFS.

The VFL server may also compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged.

Editor’s Note: whether and how to deal with sample changes and VFL clients changes is FFS.

If the VFL procedure can be terminated, the VFL Server sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request message that includes termination request to the VFL clients. The VFL Server and each VFL Client, stores the VFL correlation ID and the corresponding latest information related to their locally trained Models. Step 7 and 8 is not performed.

NOTE: The VFL correlation ID is later used for inference.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS.

Editor’s Note: The flow should be updated to show the message exchange for VFL training termination.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include global ML model metric (e.g. ML model accuracy).

Editor’s Note: The content of the VFL status report is FFS.

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

If the VFL server received a request from consumer or decide to stop the Vertical Federated Training process, steps 7 and 8 are skipped.

7.  If VFL procedure continues, the NWDAF acting as a VFL Server sends to NWDAF client the Nnwdaf_MLModelTraining_Subscribe message or Nnwdaf_MLModelTrainingInfo_Request message and to AF client the Naf_MLModelTraining_Notify message or the Naf_MLModelTrainingInfo_Request response, which includes the backward local ML model training information, for next round of VFL training.

8.	Each VFL client computes gradient of its local model and updates its local ML model based on backward local ML model training information distributed by the VFL server at step 7. The NWDAF acting as VFL Server and each NWDAF or AF acting as VFL Client, stores the latest information about their locally trained Models.

Editor’s Note: It is FFS whether model storage after each training round is required.

Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS.

The steps 3-8 should be repeated until the training termination decision is made in step 6.

# >>>>END OF CHANGES<<<<