3GPP TSG-WG SA2 Meeting #165	S2-2410697

# Hyderabad, India, October 14 - 18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1498 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DCSF instructing the IMS AS to terminate the session at the IMS AS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In "Originating Standalone Bootstrap DC Setup" in clause AC.10.2.1, the DCSF needs to instruct the IMS AS to terminate the IMS session establishment at the IMS AS, i.e. IMS AS acts as SIP UAS and generates the final response to the INVITE. It is not specified how the DCSF instructs the IMS AS to perform this, instead of R18 behaviour where the DCSF instructs the IMS AS to forward the received INVITE towards the terminating network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add an optional parameter "Session instruction" to the Nimsas_MediaControl_MediaInstruction service operation. The default action in IMS AS is "continue", this is used when the parameter is omitted in the request. The value "terminate" instructs the IMS AS to act as UAS for the IMS session and to generate the final response to the INVITE request. The DCSF sets the value to "terminate" in step 6 in figure AC.10.2.1-1. In step 11, the statement that the IMS AS acts based on the received Request URI is incorrect and is removed. Other changes: - "using PSI" is removed from the title, this gives an impression that there is another procedure where PSI is not used. - Subscription for standalone data channels needs to be verified by IMS AS in step 2. Rev-1: Remove the first change regarding the introduction of a new attribute in the service definition. Add PSI back. Make it consistent in the procedure that the Request URI is a PSI. In step 6, remove the statement that the MediaInstruction indicates the IMS AS to terminate the session; the termination is done based on the PSI in step 11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCSF cannot instruct the IMS AS to terminate the SIP INVITE at the IMS AS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.3.2, AC.10.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

### AC.10.2.1	Originating Standalone Bootstrap DC Setup using PSI

The standalone IMS data channel session setup with only bootstrap DC is triggered by a UE or the user of a UE for downloading application list and/or applications. After standalone bootstrap DC is established, the UE downloads the application list via the bootstrap DC, and may display the list of application information to the user for selection, downloading, and/or initiating standalone application DC. The UE terminates the standalone IMS data channel session when the session timer expired.

Figure AC.10.2.1-1depicts a signalling flow diagram for establishing a standalone bootstrap data channel using PSI.

AC.10.2.1-1: Originating Standalone Bootstrap DC Setup using PSI

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The Request URI of the SIP INVITE message is a specific target URI, i.e. a specific PSI indicating standalone bootstrap DC. The initial SDP contains offers only for the bootstrap data channel establishment request with bootstrap DC stream ID(s).

NOTE 1:	The PSI is pre-configured in the UE#1. How and when the PSI is pre-configured is left to implementation.

NOTE 2:	UE#1 can perform step 1 at any time before initiating a standalone application data channel, such as, immediately after completing the SIP registration procedure.

2-4.	Steps 2-4 of clause AC.7.1 apply with the following difference:

The IMS AS rejects a session initiated by a UE not subscribed for a standalone IMS data channel session.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered a local bootstrap media, DCSF, based on the value of the Called ID (i.e. the specific PSI) and the content of the initial SDP, determines that UE#1 requests to establish a standalone bootstrap DC and reserves only originating side MDC1 media information, which are used to receive UE request for application downloading from MF or MRF.

6.	DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) operation based on its policies instructing the IMS AS how to set up bootstrap data channel with MF for originating side. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

7.	Step 7 of clause AC.7.1 applies.

8.	IMS AS invokes Nmf_MRM_Create (List of Media Termination Descriptors) service operation to instruct MF to allocate required data channel media resources for originating side only. IMS AS request creation of only one Media Termination which representing the local bootstrap media to be terminated.

9-10.	Steps 9-10 of clause AC.7.1 apply.

11.	IMS AS terminates the IMS session based on the specific Request URI (i.e. PSI) and sends 18X response with the SDP answer through S-CSCF and P-CSCF to UE#1.

12-13.	The session with bootstrap data channel is established.

14.	The bootstrap data channels have been established between originating MF or MRF and UE#1. The UE#1 sends application request messages to MF or MRF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF or MRF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 based on their data channel capabilities and their choices through MF or MRF.

15-16.	If the established bootstrap DC is not used for a period of time (expiry of the session timer), UE#1 sends SIP BYE to release the bootstrap DC.

*************************** End of Changes ****************************