3GPP TSG-WG SA2 Meeting #165	S2-2410745

Hyderabad, IN, 14th Oct – 18th Oct, 2024	(revision of S2-24010410)

Source:	Huawei, HiSilicon, Samsung, Futurewei, OPPO

Title:	Conclusion on key Issue 1 for Topology 1 Architecture

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusion principles are proposed on key issue 1 for topology 1 architecture.

# 1. Introduction/Discussion

This is a revision of S2-24010410 which takes all the inputs related to KI1 (copied into Annex A for reference), sorts them into various categories and from then then presents the conclusion for Topology 1 aspects of KI1. Topology 2 and common aspects will be handled by revisions of other documents and together they form the conclusion for KI1.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13, v1.0.0.

Foe guidance, the expected clause structure for clause 8.1 is:

8.1.1 General

8.1.2 Architecture to Support Topology 1

8.1.3 Architecture to Support Topology 2

# * * * * First change (All new) * * * *

### 8.1.2	Architecture to Support Topology 1

Editor’s Note: The conclusion details in this clause are interim and will be discussed at SA2#166.

The principles and aspects in this clause are agreed to support Topology 1:

-	The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1:

-	Communicate with BS Reader to trigger e.g., inventory request and command request).

NOTE x:	It is assumed that the BS Reader for Topology 1 can be deployed without having to deploy other gNB functionality for NR-Uu or the BS Reader can be deployed with other gNB functionality for NR-Uu implementation matter.

-	BS Reader selection is performed by the AIoT NF selecting one or multiple BS Reader(s) and forwards the AIoT service request to the BS Reader(s).

-	Ambient IoT services can be deployed independently from existing deployments.

-	The configuration of a BS Reader is assumed to be handle by OAM as part of the BS Reader integration in the network.

When a BS Reader and the AIoTF communicate directly:

-	The AIOTF communicates with a BS Reader via a direct interface Nx.

-	Figure 8.1.2-x below shows the aspects related to Topology 1 (direct path) reference architecture with other NFs removed.

Figure 8.1.2-1: Non-Roaming 5G System Architecture (Direct Path)

-	Figure 8.1.2-x below shows the aspects related to Topology 1 (direct path) in reference point representation with other NFs removed.

Figure 8.1.2-2: Non-Roaming 5G System Architecture in reference point representation (Direct Path)

-	The protocol used over Nx will support procedures and information to be exchanged as specified by RAN2, RAN3 and SA2.

NOTE X:	The protocol stack used between the AIoTF and the BS Reader will be concluded by RAN3.

-	Figure 8.1.2-x below shows the aspects related to Topology 1 (direct path) protocol stack between the BS Reader and AIoTF.

Figure 8.1.2-3:	Example Protocol Stack between AIoTF and AIoT Device for Topology 1 (Direct Path)

Editor’s Note: The details of the protocol stack are FFS.

When a BS Reader and the AIoTF communicate indirectly via an AMF:

-	The AIoTF connects with a BS Reader via an AMF. The N2 interface between the BS Reader and AMF supports Ambient IoT services including delivery of inventory/command messages.

NOTE X: If network isolation is required an AMF instance is deployed for supporting the AIOTF communication with the BS Reader.

- 	NGAP between the BS Reader and the AMF is	 enhanced to support Ambient IoT Services. The details of the enhancements will be concluded by RAN3.

-	AMFs may update the NRF with information that allows the AIOTF to determine which AMFs to use for which BS Readers.

Editor’s Note: Details of the NRF updates are FFS and how the AIOTF uses the NRF is FFS.

-	The AMF shall be enhanced to support Services which are used by an AIoTF for Ambient IoT Operations. New SBI based services on the AMF (to be used by the AIoTF) are introduced in 5GC.

-	The AMF passes information to and from AIoT Devices (e.g. identity etc) and BS Readers (e.g. operation requests and responses) to the AIOTF.

Editor’s Note: The AMF enhancements are FFS.

Editor’s Note: Details of the Service (e.g. whether it is a new service, whether the existing Namf service is enhanced is FFS).

-	Figure 8.1.2-x below shows the aspects related to Topology 1 (indirect path via AMF) reference architecture with other NFs removed.

Figure 8.1.2-4: Non-Roaming 5G System Architecture (indirect Path via AMF)

-	Figure 8.1.2-x below shows the aspects related to Topology 1 (indirect  Path via AMF) in reference point representation with other NFs removed.

Figure 8.1.2-5: Non-Roaming 5G System Architecture in reference point representation (indirect Path via AMF)

-	Figure 8.1.2-x below shows the aspects related to Topology 1 (indirect path via AMF) protocol stack between the BS Reader and AIoTF.

Figure 8.1.2-6: Example Protocol Between AIoTF and AIoT Device for Topology 1 (indirect Path via AMF)

Editor’s Note: The details of the protocol stack are FFS.

# * * * * End of changes * * * *